Tender for Design, Manufacture, Supply, Installation, Validation and Commissioning of

VENTILATOR AUTOCLAVE

Size: 1850 mm DIA X 4200 mm LONG

IFB No: HLL/AFT/PUR/AUTOCLAVE/2013-14	                           DATE: 01.11.2013

##### HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIYAM  P.O.

THIRUVANANTHAPURAM – 695 017

Phone: +91 471 244 5930, Fax : +91 471 244 5935

Email: materialsaft@lifecarehll.com

Web: www.lifecarehll.com

##### HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIYAM  P.O.

THIRUVANANTHAPURAM – 605 017

IFB NO

DATE OF COMMENCEMENT 
OF SALE OF BIDDING DOCUMENT

LAST DATE FOR SALE OF BIDDING DOCUMENT

DATE OF PRE BID MEETING

LAST DATE AND TIME FOR

RECEIPT OF BIDS

TIME AND DATE OF OPENING OF BIDS

PLACE OF OPENING OF BIDS

ADDRESS FOR COMMUNICATION

:	HLL/AFT/PUR/AUTOCLAVE/2013-14

Dt. 01.11.2013

:	01.11.2013

:	21.11.2013, 11.30 HRS

:	11.11.2013

:	21.11.2013, 14.30 HRS

:	21.11.2013, 15.00 HRS

:	HLL LIFECARE LIMITED

AKKULAM  FACTORY,

SREEKARIYAM  P.O.

THIRUVANANTHAPURAM – 17.

:	HLL LIFECARE LIMITED

AKKULAM  FACTORY,

SREEKARIYAM  P.O.

THIRUVANANTHAPURAM – 17

##### HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIYAM  P.O.

THIRUVANANTHAPURAM – 605 017

CONTENTS

| SECTION NO. | DESCRIPTION | PAGE NO. |
| --- | --- | --- |
| SECTION I | INVITATION FOR BIDDERS (IFB) | 5-7 |
| SECTION II | INSTRUCTION TO BIDDERS (ITB) | 9-20 |
| SECTION III | GENERAL CONDITIONS OF CONTRACT (GCC) | 22-29 |
| SECTION IV | SPECIAL CONDITIONS OF CONTRACT (SCC) | 31-36 |
| SECTION V | TECHNICAL SPECIFICATIONS | 38-42 |
| SECTION VI | QUALIFICATION CRITERIA | 43 |
| SECTION VII | PROFORMA FOR EQUIPMENT AND QUALITY & CONTROL | 44 |
| SECTION VIII | PERFORMANCE STATEMENT | 45-53 |
| SECTION IX | BID FORM | 54 |
| SECTION X | PRICE SCHEDULE | 55-56 |
| SECTION XI | VENTILATOR AUTOCLAVE AMC PRICE SCHEDULE | 57 |
| SECTION XII | CHECK LIST | 59-60 |
| SECTION XIII | CONTRACT FORM | 61-63 |

SECTION I

INVITATION FOR BIDS (IFB)

##### HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY,

SREEKARIYAM  P.O.

THIRUVANANTHAPURAM – 695 017

INVITATION FOR BIDS (IFB)

IFB No.HLL/AFT/PUR/AUTOCLAVE/2013-14

Sealed and superscribed tenders under two bid system are invited for the Design, Manufacture, Supply, Installation, Validation and Commissioning of VENTILATOR AUTOCLAVE for blood bags. Size: 1850 mm DIA X 4200 mm LONG at HLL Lifecare Ltd, Akkulam Plant.

| SL.No. | Name of the Item | Last Date& Time of Submission of Bids | Date& Time of Opening of Technical bids |
| --- | --- | --- | --- |
| 1. | Design, Manufacture, Supply, Installation, validation and Commissioning of VENTILATOR AUTOCLAVE Size: 1850 mm DIA X 4200 mm LONG | 21st Nov, 2013 at 14.30 hrs, | 21st Nov, 2013 at 15.00 hrs |

1. Interested eligible Bidders may obtain further information from the office of the Joint General Manager (Materials) , HLL Lifecare Ltd, Akkulam Factory, Thiruvananthapuram -695 017. (Phone No: 0471-2442641, email: materialsaft@lifecarehll.com )

2. A complete set of bidding documents may be purchased by any interested eligible bidder from the above office from 10.30 Hrs to 15.00 Hrs on any working day on submission of a written application to the above office and remitting a non- refundable fee of Rs. 2000 /- (including taxes) in the form of DD drawn in favour of HLL Lifecare Limited, payable at Thiruvananthapuram. Incase of overseas bidders non-refundable fee of US $ 30 (including taxes) in the form of DD drawn in favour of HLL Lifecare Limited, payable at Thiruvananthapuram

3.The Tender Document can also be downloaded from our Website www.lifecarehll.com and cost of the Tender Document as mentioned above should be furnished in the form of Demand Draft in favor of M/s HLL Lifecare Limited payable at Thiruvananthapuram, along with Technical Bid. In case of non-submission of DD along with bid, the bid shall be deemed rejected.

4. Interested eligible bidders may submit their bidding documents at the office of the Joint General Manager (Materials), HLL Lifecare Ltd, Akkulam Factory, Sreekariyam.P.O,Thiruvananthapuram –695 017 on or before the specified date and time.

5. A pre-bid meeting will be conducted on 11.11.2013.

6. All bids must be accompanied by the items/documents specified herein and must be delivered to the Joint General Manager (Materials) on or before 21.11.2013, 14.30 Hrs.  The bids (Technical Bids) will be opened on 21.11.2013 at 15.00 Hrs, in the presence of the bidders or their duly authorized personnel who wish to attend the bid opening on the specified date and time at HLL's office mentioned above. In the event of the date being declared is a closed holiday for HLL, the due date for submission of bids and opening of bids will be the following working day at the same time.

7. Completion Period: 4 months from date of issue of Purchase order/L.O.I or handing over of the site, whichever is earlier.

8.Other terms:

  - AMC charges for a period of 5 years excluding warranty period to be included in bids. These shall not be considered during price bid comparison and the payments to the supplier shall be made on six monthly basis after satisfactory completion of said service, duly certified by HLL Lifecare Limited.

  - Price list for all critical and essential spares to be specified in bid. These shall NOT be considered during price bid comparison.

  - G.A. drawings, design specifications and price of SS 304 trolleys, SS 304 carriages and Aluminium mesh trays to be specified in bids. The prices of these items shall NOT be considered for price bid comparison. HLL reserves the right to include/not include these items in the schedule of items while issuing purchase order to qualified supplier.

  - Leading & Unloading at site excluded from Bidders scope.

### SECTION II

INSTRUCTION TO BIDDERS

INSTRUCTION TO BIDDERS

A.INTRODUCTION

1. Eligible Bidders

The Eligibility criteria for the bidders is as per Section VI Qualification criteria

2. Cost of Bidding

2.1 The Bidder shall bear all costs associated with the preparation and submission of its bid and HLL Lifecare Limited, Akkulam Factory, Thiruvananthapuram hereinafter referred to as “the Purchaser”, will in no case be responsible or liable for these  costs,  regardless  of  the  conduct  or  outcome  of  the  bidding process.

B. THE BIDDING DOCUMENTS

3. Contents of Bidding Documents

3.1 The goods required, bidding procedures and contract terms are prescribed in the Bidding documents. In addition to the Invitation for Bids, the Bidding documents include:

a.   Instruction to Bidders (ITB);

b.   General Conditions of Contract (GCC);

c.   Special Conditions of Contract (SCC);

d.   Technical Specifications

e.   Qualification criteria;

f.    Bid Form

g.   Price Schedule;

h.   Check List

3.2   The Bidder is expected to examine all instructions, forms, terms and specifications in the Bidding Documents. Failure to furnish all information required  by  the  Bidding  Documents  or  submission  of  a  bid  not substantially responsive to the Bidding Documents in every respect will be at the Bidder’s risk and may result in rejection of its bid.

4. Clarification of Bidding Documents

4.1 A  prospective  Bidder  requiring  any  clarification  of  the  Bidding Documents may notify the Purchaser in writing at  the Purchaser’s mailing address indicated in the Invitation for Bids. The Purchaser will respond  to  any  request  for  clarification  of  the  Bidding Documents which it receives not later than 5 days prior to the deadline for submission of bids prescribed by the Purchaser. Purchaser’s response (including an explanation of the query but without identifying the source of inquiry) will be communicated through our web site / email to all prospective Bidders who have received the bidding documents.

5. Amendment of Bidding Documents

5.1 At any time prior to the deadline for submission of bids, the Purchaser may, for any reason, whether at its own initiative or in response to a clarification requested by a prospective bidder, modify the Bidding Documents by an amendment.

5.2 Any subsequent amendments in the bid shall be notified only on HLL website.

5.3 In order to allow prospective bidders reasonable time in which to take the amendment into account in preparing their bid, the Purchaser may, at its discretion, extend the deadline for the submission of bids

C. PRE-BID CONFERENCE

6. Pre- Bid conference

6.1 A pre bid meeting will be held on 11.11.2013(tentative). Confirmed date, time & venue will be published in HLL website as an addendum

D. PREPARATION OF BIDS

7. Language of Bid

7.1 	The Bid prepared by the bidder and all correspondence and documents relating to the bid exchanged by the Bidder and the Purchaser, shall be 
written in the English language. Supporting documents and printed 
literature furnished by the Bidder may be written in another language 
provided, they are accompanied by an accurate translation of the 
relevant passages in the English language in which case, for purposes 
of interpretation of the Bid, the English translation shall govern

8. Documents Comprising the Bid

Following documents and forms are to be included in the technical bid and commercial bid respectively.

8.1 TECHNICAL

The technical bid shall consist of the following documents.

- EMD / Bid Security amount in the form of DD, drawn in favor of HLL Lifecare Limited, payable a Thiruvananthapuram. The EMD / Bid Security shall be refunded to the non-responsive bidders within 60 days from the date of opening of Bid.

- The tender document cost in the form of DD, if the document is downloaded from HLL website.

- The technical specifications enclosed along with the bid document shall be confirmed by signature of the bidder/authorized signatory of the bidding firm, in all pages and authorized by official seal. The information shall be filled in the technical specifications wherever necessary as per the instructions given.

- Duly attested copies of factory license/ Industrial license, sales tax registration, and documents to prove the legal status, place of registration and principal place of business of the undertaking.

- Duly attested copies of quality certificates for the products, quality system certifications and quality accreditation certificate as specified in technical specification.

- Copy of Balance sheet for the past three financial years, duly certified by a chartered accountant

- Pro-forma for equipment and quality control, duly filled as per section VII

- Performance statement in the pro-forma under section VIII

- Documentary evidence established in accordance with ITB Clause 10 that the Bidder is eligible to bid and is qualified to perform the contract if the bid is accepted;

- Documentary evidence established in accordance with ITB Clause 11 that the equipment and ancillary services to be supplied by the Bidder shall conform to the Bidding Documents

- Duly filled Check list as per section XII

- All technical aspects including GA drawings, spare lists.etc that are mentioned in IFB clause 8.

8.2 PRICE BID

a.)  Duly filled Bid Form and price schedule as per sections IX, X and XI respectively.

b.) All commercial aspects related to items that are mentioned in IFB

9. Bid Prices

9.1 The bidder shall give the total composite price inclusive of all Levies & Taxes i.e. Sales Tax & Excise, packing, forwarding, freight and insurance etc. but excluding Octroi/Entry Tax which will be paid extra as per actuals, wherever applicable.  The basic unit price and all other components of the price need to be individually indicated against the goods it proposes to supply under the contract as per the price schedule given in Section X.  Prices of incidental services should also be quoted. The offer shall be firm in Indian Rupees. No Foreign exchange will be made available by the purchaser.

9.2   Prices indicated in the Price Schedule shall be entered in the following manner :

- The Basic Unit price (Ex-Factory Price) of the goods,  Excise duty,  Sales Tax,  Freight,  Forwarding,  Packing, Insurance and any other Levies/Charges already paid or payable by the supplier, shall be quoted separately item wise.

(ii)	The supplier shall quote as per price schedule given in section X for all the items given in schedule of requirement.

  - The price quoted by the bidder shall remain fixed during the entire period of contract and shall not be subject to variation on any account. A bid submitted with an adjustable price quotation will be treated as non - responsive and rejected.

9.4  The prices quoted by the bidder shall be in sufficient detail to enable the Purchaser to arrive at the price of equipment/system offered.

  - The price approved by HLL for procurement will be inclusive of Levies and Taxes, packing, forwarding, freight and insurance as mentioned in Para 9.1 above.  Breakup in various heads like excise duty, sales tax, insurance, freight and other taxes paid/payable as per clause 9.2 (i) is for the information of the purchaser and any change in these shall have no effect on price during the scheduled delivery period.

10. Bid Currencies

10.1 Indian Bidders should quote in INR and overseas bidders should quote in USD.

11. Documents establishing bidder’s eligibility and qualifications

11.1   Pursuant to ITB Clause 9.1, the bidder shall furnish, as part of its bid, documents establishing the bidder’s eligibility to bid and its qualifications, to perform the Contract if its bid is accepted.

11.2 The documentary evidence of the Bidder’s qualifications to  perform the contract if the bid is accepted,  shall  establish  to  the  Purchaser’s satisfactions.

(a) that the bidder has the financial, technical and production capability necessary to perform the Contract and meets the criteria outlined in the qualification requirements specified in Section VI. To this end, all bids submitted shall include the following information:

(i) The legal status, place of registration and principle place of business of the company or firm or partnership, etc;

(ii) Details of experience and past performance of the bidder on items offered and on those of similar nature within the past five years and details of current contracts in hand and other commitments (as per proforma given in Section VIII (a), (b) & (c) )

12. Documents establishing equipment and its parts and accessories conformity to bidding documents

12.1 Pursuant to ITB Clause 8, the Bidder shall furnish, as part of its bid, documents establishing the conformity to the bidding documents of all goods and services which the bidder proposes to supply under the contract.

12.2 The documentary evidence of conformity of the equipment and its parts and accessories and services to the bidding documents may be in the form of literature, drawings and data and shall consist of:

(a) a detailed description of the essential technical and performance characteristics of the VENTILATOR AUTOCLAVE;

(b) a list giving full particulars, including available sources of all spare parts, special tools, etc., necessary for the proper and continued functioning of the VENTILATOR AUTOCLAVE for a period of six years, following commencement of the VENTILATOR AUTOCLAVE installed by the Purchaser; and

(c)  an  item-by-item  commentary  on  the  Purchaser’s  Technical  Specifications demonstrating substantial responsiveness of the equipment and services to those specifications or a statement of deviations and exceptions to the provisions of the Technical Specifications.

12.3  For the purpose of the commentary to be furnished pursuant to ITB Clause12.2 (c)  above,  the  Bidder  shall  note  that  standards  for workmanship, material and equipments and references to brand names or  catalogue  numbers,  if  any,  designated  by  the  Purchaser  in  its Technical Specifications are intended to be descriptive only and not restrictive.  The  Bidder  may  substitute  alternative  standards,  brand names  and/or  catalogue numbers  in  its  bid,  provided  that  it demonstrates to the Purchaser’s satisfaction that the substitutes are substantially equivalent or superior to those designated in the Technical Specifications.

13. Period of Validity of Bids

13.1 Technical Bid & Price Bid shall be valid for minimum one year

13.2 	In exceptional circumstances, the Purchaser may solicit the bidders consent to an extension of the period of validity. The request and the responses thereto shall be made in writing.

14. Format and Signing of Bid

14.1 The bidder shall prepare two copies of the bid clearly marking each “Original Bid” and “Copy Bid” as appropriate. In the event of any discrepancy between them, the original shall govern.

14.2    The original and all copies of the bid shall be typed or written in indelible ink and shall be signed by the Bidder or a person or persons duly authorized to bind the Bidder to the Contract. The letter of authorization shall be indicated by written power-of-attorney accompanying the bid. All pages of the bid, except for unamended printed literature, shall be initialed by the person or persons signing the bid.

14.3    Any interlineations, erasures or overwriting shall be valid only if they are  initialed by the person or persons signing the bid.

15. Sealing and Marking of bids

Separate bids shall be submitted for price bid and technical bids.

15.1     The bidders shall seal Technical bid     and Price bid in separate inner 	envelopes, duly  marking  the  envelopes  as “TECHNICAL  BID”  and ‘PRICE BID. The Bidders shall then place both the inner envelopes in an outer envelope. The name of the product for which the bid is made must be written on both the inner envelops and the outer envelop.

15.2     The inner and outer envelopes shall be addressed to the Purchaser at  the following address:

Joint General Manager (Materials),

HLL Lifecare Ltd, Akkulam Factory, Sreekariyam.PO,

Thiruvananthapuram –17

(a) The outer envelope shall bear the Invitation for bids (IFB) number, 
and a statement: “DO NOT OPEN BEFORE” 15.00 Hrs. ON 21.11.2013”

(b)  The outer and inner envelope shall also indicate the name and address of the bidder to enable the bid to be returned unopened in case it is declared “late”.

(c)   If the inner and outer envelopes are not sealed and marked as required, the Purchaser will  assume  no  responsibility  for  the  bid’s misplacement or premature opening.

(d) Bids must be received by the Purchaser at the address specified not later than the date and time specified in the Invitation For Bid.   In the event of the specified date for the submission of bids, being declared a holiday for the Purchaser, the bids will be received up to the appointed time on the next working day.

(e) The Purchaser may, at its discretion, extend this deadline for the  submission of bids by amending the bidding documents in which case all 
rights and obligations of the Purchaser and bidders previously subject to 
the deadline will thereafter be subject to the deadline as extended.

16. LATE BIDS

16.1     Any bid received by the Purchaser after the deadline for submission of  bids  prescribed  by  the  Purchaser,  will  be  rejected  and  returned  unopened to the bidder.

17.  MODIFICATION AND WITHDRAWAL OF BIDS

17.1   The bidder may modify or withdraw its bid after the bid submission,  provided that written notice of the modification or withdrawal is received  by the Purchaser prior to the deadline prescribed for submission of bids.

17.2    The bidder’s modification or withdrawal notice shall be prepared, sealed, marked and dispatched in accordance with the provisions of ITB Clause 15.   A withdrawal notice may also be sent by fax or email but followed by a signed confirmation copy, post-marked not later than the deadline for submission of bids.

17.3     No bid may be modified subsequent to the deadline for submission of  bids.

17.4   No bid may be withdrawn in the interval between the deadline for submission of  bids and the expiration of  the period of bid validity  specified by the bidder in the Bid Document.

E. BID OPENING AND EVALUATION

18. Opening of Bids by Purchaser

18.1  The  Purchaser  will  open  all  bids,  in  the  presence  of  bidder's  representatives who choose to attend, at 15.00 Hrs on 21.11.2013 at the  following location:

HLL Lifecare Limited,

Akkulam Factory, Sreekariyam PO																				Akkulam Factory, Sreekaryam  P.O. ,

Thiruvananthapuram -695 017.

18.2 The bidder’s representatives who are present shall sign a register evidencing their attendance. In the event of the specified date of bid opening being declared a holiday for the Purchaser, the bids shall be opened at the appointed time and location on the next working day.

18.3 The bidder’s names, modifications, bid withdrawals and the presence or   absence of the requisite documents and such other details as the  Purchaser, at its discretion, may consider appropriate will be announced  at the opening. No bid shall be rejected at bid opening, except for late  bids, which shall be returned unopened to the bidder pursuant to ITB   Clause 16.

18.4   The Purchaser will prepare minutes of the bid opening

18.5 The “Price Bid” (Cover B) will be opened after evaluation of “Technical  bids” (Cover A) and the date and time of opening of price bid will be intimated to bidders whose bids are responsive and selected by the Purchaser.

18.5   The purchaser will scrutinize the technical bid for compliance to the specifications and documentation requirement as per the bid document.  Purchaser will depute its competent officers to the premises of the bidder  qualified on the basis of technical scrutiny, for on-site evaluation of the claims made in the technical bid, if deemed appropriate on purchaser’s sole  discretion.  The  bidders  will  be  short-listed  on  the  basis  of responsiveness of technical bid as well as report of on-site technical  evaluation. The price bid of the bidders who are disqualified at the  technical scrutiny and on-site evaluation will be returned un-opened. The  on-site evaluation may include the inspection of the specimen model of  the equipment. The short listed bidders will be informed about the time, date and venue of the price bid opening. The successful bidder shall be  identified on the basis of lowest evaluated substantially responsive bid.

19.  CLARIFICATION OF BIDS

19.1     During evaluation of bids, the Purchaser may, at its discretion, ask the bidder for a clarification of its bid.   The request for clarification and the response shall be in writing and no change in prices or substance of the bid shall be sought, offered or permitted.

20    PRELIMINARY EXAMINATION

20.1   The Purchaser will examine the bids to determine whether they are complete, whether any computational errors have been made, whether the documents have been properly signed, and whether the bids are generally in order.

20.2     Arithmetical errors will be rectified on the following basis.   If there is a discrepancy between the unit price and the total price that is obtained by multiplying the unit price and quantity, the unit price shall prevail and the total price shall be corrected. If there is a discrepancy between words and figures, the amount in words will prevail.   If the bidder does not accept the correction of the errors, its bid will be rejected.

20.3   The Purchaser may waive any minor informality or non-conformity or irregularity in a bid, which does not constitute a material deviation, provided such a waiver does not prejudice or affect the relative ranking of   any bidder.

20.4   Prior to the detailed evaluation pursuant to Clause ITB 21, the purchaser will determine the substantial responsiveness of each bid to the bidding documents. For purposes of these clauses a substantially responsive bid is one which conforms to all the terms and conditions of the bidding documents without material deviations. Deviations from or objections or reservations to critical provisions such as those concerning Warranty, Force Majeure, Applicable law  and Taxes and Duties   will be deemed to be material deviation. The purchaser’s determination of a bid’s responsiveness is to be based on the contents of the bid itself without recourse to extrinsic evidence.

20.5    If a bid is not substantially responsive, it will be rejected by the Purchaser and  may  not  subsequently  be  made  responsive  by  the  bidder  by correction of the non-conformity.

21. EVALUATION AND COMPARISON OF BIDS

21.1   The Purchaser will evaluate and compare bids previously determined to be substantially responsive, pursuant to ITB Clause 20.

21.2   The purchaser’s evaluation of a bid will take into account, in addition to the bid price (ex-factory/ex-warehouse/off-the-shelf price of the VENTILATOR AUTOCLAVE offered from within India, such price to include all costs as well as duties and taxes paid or payable on components and raw material incorporated or to be incorporated in the equipment and its parts and accessories, and excise duty on the equipment, if payable) and price of incidental services, the following factors, in the manner and to the extent indicated in ITB Clause 20.3 and in the technical specifications:

Cost of inland transportation, insurance and other costs incidental to the delivery of VENTILATOR AUTOCLAVE to HLL Lifecare Limited, Akkulam, Thiruvananthapuram.

21.3 	Pursuant to ITB Clause 21.2 the following evaluation methods will be applied:

(a) Inland transportation, from factory, insurance and incidentals.

(i)  Inland transportation, insurance and other incidentals, for Design, Manufacture, Supply, Installation, Validation and Commissioning of VENTILATOR AUTOCLAVE  and its parts and accessories to the required locations mentioned in ITB Section 21.2 The above costs will also be considered in the bid price.

(b)  Spare parts and after sales service facilities in India:

The cost of the Purchaser of establishing the minimum service facilities and parts inventories, as outlined elsewhere in the bid invitation, if quoted separately, shall  be added to the bid price.

(c)Annual  Maintenance Contract (VENTILATOR AUTOCLAVE AMC)

The Purchaser desires to have separately Annual maintenance charges for a period of 5 years after the expiry of Warranty period, clearly indicating year wise comprehensive maintenance charges, as per Section XI.

22. CONTACTING THE PURCHASER

22.1 No bidder shall contact the Purchaser on any matter relating to its bid, from the time of the bid opening to the time the contract is awarded. If the bidder wishes to bring additional information to the notice of the purchaser it should do so in writing.

22.2 Any effort by a bidder to influence the purchaser in its decisions on bid evaluation, bid comparison, or selection may result in the rejection of the bidders bid.

F. AWARD OF CONTRACT

23. POST QUALIFICATION

23.1 The determination will take into account the bidders financial, technical and production capabilities. It will be based upon an examination of the documentary evidence of the bidder’s qualifications submitted by the bidder, pursuant to ITB Clause 13 as well as such other information as the Purchaser deems necessary and appropriate.

23.2  	An affirmative determination will be a prerequisite for award of the contract to the bidder. A negative determination will result in rejection of the bidders bid, in which event the Purchaser will proceed to the next lowest evaluated bid to make a similar determination of that bidder’s capabilities to perform satisfactorily.

24. AWARD CRITERIA

24.1 	Subject to ITB Clause 28, the Purchaser will award the contract to the successful bidder whose bid has been determined to be substantially responsive and has been determined as the lowest evaluated bid,    provided further that the bidder is determined to be qualified to perform the contract satisfactorily.

25. PURCHASER’S RIGHT TO VARY QUANTITIES AT TIME OF AWARD

25.1 The Purchaser reserves the right at the time of award of contract to increase or decrease the quantity of equipment and its parts and accessories, wherever applicable, and services originally specified in the bid document without any change in unit price or other terms and conditions

26. PURCHASER’S RIGHT TO ACCEPT ANY BID AND TO REJECT ANY OR ALL BIDS

26.1 The Purchaser reserves the right to accept or reject any bid, and to annul the bidding process and reject all bids at any time prior to award of Contract, without thereby incurring any liability to the affected bidder or bidders.

27. NOTIFICATION OF AWARD

27.1 Prior to the expiration of the period of bid validity, the Purchaser will notify the successful bidder in writing by registered letter or by fax or email to be confirmed, that its bid had been accepted.

27.2 The notification of award will constitute the formation of the contract

28. SIGNING OF CONTRACT AGREEMENT

28.1 The successful bidder has to sign a supply agreement is Kerala stamp paper worth Rs 100, the draft of which will be sent to the lowest responsive bidder, with a letter informing acceptance  of  bid.  Within 20 days of receipt of the information regarding acceptance of the bid, the successful bidder shall sign the supply agreement and return it to the Purchaser.

29. Delays in Design, Manufacture, Supply, Installation, Validation and Commissioning of VENTILATOR AUTOCLAVE

29.1 Delivery, Installation, Validation and Commissioning of the equipment and its parts and accessories shall be made by the Supplier within the stipulated delivery date as specified in the purchase order / work order / contract, from the date of placing the date of Order. If at any time during performance of the Contract, the Supplier should encounter conditions impeding timely Delivery, Installation, Validation and Commissioning of the VENTILATOR AUTOCLAVE, the Supplier shall promptly notify the Purchaser in writing of the fact of the delay, its likely duration and its cause(s). As soon as practicable after receipt of the Supplier's notice, the Purchaser shall evaluate the situation and may at its discretion extend the Supplier's time for performance, with or without penalty.

29.2 A delay by the Supplier in the performance of its Delivery, Installation, Validation and Commissioning obligations shall render the Supplier liable to the imposition of penalty pursuant to agreement, unless an extension of time is agreed upon pursuant to agreement without the application of liquidated damages.

29.3 If the Supplier fails to deliver any or all of the equipment or its parts or perform the Services within the time period(s) specified in the Purchase Order, the purchaser shall, without prejudice to its other remedies under the Contract, deduct from the Contract Price, as penalty, a sum equivalent to the percentage specified in the purchase order.

30. PAYMENT TERMS

30.1 Payment terms is as follows

A. Supply

- 60% of the order value against supply of all items, FAT and actual inspection at HLL site.

- 30% of order value against successful SAT(Site Acceptance Test)

- 10% will be retained as Retention Money which shall be paid after one year from the date of successful SAT OR shall be paid against a performance bank guarantee for an amount equivalent to 10% of order value from a scheduled bank with validity of one year from date of successful SAT.

Normally the Purchaser will not entertain any advance payment. However, the bidder can indicate advance payment in the payment terms, if required. Moreover such advance payment, will be released against submission of a Bank guarantee, on a specific format given by the supplier, from a nationalized/scheduled bank for like amount valid up to the supply of equipment at purchaser’s site.

B. Installation

- 100% of work order against successful SAT

30.2 Test Reports (wherever applicable) are to be forwarded along with the original invoices.

30.3  Payment for Annual Maintenance Contract Charges:

The payment of VENTILATOR AUTOCLAVEAMC will be made on six monthly basis after satisfactory completion of said , duly certified by HLL Lifecare Limited.

30.4  Payment shall be made in Indian rupee. Foreign bidder have to submit their bid in USD.

30.5 All other Statutory levies, such as Taxes, duties, KVAT, VAT, WCT, IT, ST & PF, ESI of the work men etc (are inclusive) will be deducted from bill, as applicable.

31. Completion period.

The successful bidder has to design , supply, manufacture, validate & commission the Equipment within 4 months from the date of issue of work order/Letter of Indent or handing over of the site whichever is earlier.

###### SECTION III

###### GENERAL CONDITIONS OF CONTRACT

TABLE OF CLAUSES

| Sl No | Description | Page No. |
| --- | --- | --- |
| 1. | DEFINITIONS | 23 |
| 2. | APPLICATION | 23 |
| 3. | STANDARDS | 23 |
| 4. | USE OF CONTRACT DOCUMENTS AND INFORMATION | 24 |
| 5. | PATENT RIGHTS | 24 |
| 6. | INSPECTION AND TESTS | 24 |
| 7. | PACKING | 25 |
| 8. | DELIVERY AND DOCUMENTS | 25 |
| 9. | INSURANCE | 25 |
| 10. | INCIDENTAL SERVICES | 25 |
| 11. | SPARE PARTS | 26 |
| 12. | WARRANTY | 26 |
| 13. | PRICES | 26 |
| 14. | CHANGE ORDERS | 27 |
| 15. | CONTRACT AMENDMENTS | 27 |
| 16. | ASSIGNMENT | 27 |
| 17. | SUBCONTRACTS | 27 |
| 18. | LIQUIDATED DAMAGES | 27 |
| 19. | TERMINATION BY DEFAULT | 27-28 |
| 20. | FORCE MAJEURE | 28 |
| 21. | TERMINATION FOR INSOLVENCY | 28 |
| 22. | RESOLUTION OF DISPUTES | 28 |
| 23. | GOVERNING LANGUAGE | 29 |
| 24. | APPLICABLE LAW | 29 |
| 25. | NOTICES | 29 |
| 26. | TAXES AND DUTIES | 29 |

GENERAL CONDITIONS OF CONTRACT

1. DEFINITIONS

1.1 In this contract the following terms shall be interpreted as indicated:

(a) “The Contract” means the agreement entered into between the Purchaser and the Supplier as recorded in the Contract Form signed by the parties, including all the attachments and appendices thereto and all documents incorporated by reference therein;

(b) “The Contract Price” means the price payable to the Supplier under the Contract for the full and proper performance of its contractual obligations;

(c) “The Goods” means all the products, equipment, machinery, and/or other materials which the Supplier is required to supply to the Purchaser under the Contract;

(d) “Services” means services ancillary to the supply of the Goods, such as transportation and insurance, and other incidental services, such as installation, commissioning, provision of technical assistance, training and other obligations of the Supplier covered under the contract;

(e) “GCC” means the General Conditions of Contract contained in this 
section.

(f) “SCC” means the Special Conditions of Contract.

(g) “The Purchaser” means the Organization purchasing the Goods, as named in SCC;

(h) “The Supplier” means the individual or firm supplying the Goods under this Contract;

(i) “Day” means calendar day.

(j) “Delivery period” means the period applicable upto completion of supply; installation and testing by the supplier  at  the required location mentioned in  purchase order  and accepted by the Purchaser.

2. APPLICATION

2.1 These General Conditions shall apply to the extent that they are not superseded by provisions in other parts of the Contract.

3. STANDARDS

3.1 The Goods supplied under this Contract shall conform to the standards mentioned in the Technical Specifications, and, when no applicable standard is mentioned, to the authoritative standard appropriate to the Goods’ country of origin and such standards shall be the latest issued by the concerned institution.

4. USE OF CONTRACT DOCUMENTS AND INFORMATION

4.1 The Supplier shall not, without the Purchaser’s prior written consent, disclose the Contract, or any provision thereof, or any specification, plan, drawing, pattern, sample or information furnished by or on behalf of the Purchaser in connection therewith, to any person other than a person employed by the Supplier in performance of the Contract. Disclosure to any such employed person shall be made in confidence and shall extend only so far as may be necessary for purposes of such performance.

4.2 The Supplier shall not, without the Purchaser’s prior written consent, make use of any document or information enumerated in GCC Clause 4.1 except for purposes of performing the Contract.

4.3 Any document, other than the Contract itself, enumerated in GCC clause 4.1 shall remain the property of the Purchaser and shall be returned (in all copies) to the Purchaser on completion of the supplier’s performance under the Contract if so required by the Purchaser.

5. PATENT RIGHTS

5.1 The Supplier shall indemnify the Purchaser against all third-party claims of infringement of patent, trademark or industrial design rights arising from use of the Goods or any part thereof in India.

6. INSPECTION AND TESTS

6.1 The Purchaser or its representative shall have the right to inspect and/or  to test the Goods to confirm their conformity to the Contract at no extra cost of the Purchaser. The Special conditions of Contract and/or the Technical Specifications shall specify what inspections and tests the Purchaser requires and where they are to be conducted. The Purchaser shall notify the Supplier in writing of the identity of any representatives retained for these purposes.

6.2 The inspections and test may be conducted on the premises of the Supplier or its subcontractor(s), at point of delivery and/or at the Goods final destination. Where conducted on the premises of the Supplier or its subcontractor(s),  all  reasonable  facilities  and  assistance  including access to drawings and production data - shall be furnished to the inspectors at no charge to the Purchaser.

  - Should  any  inspected  or  tested  Goods  fail  to  conform  to  the specifications, the Purchaser may reject them and the Supplier shall either replace the rejected Goods or make all alternations necessary to meet specification requirements free of cost to the Purchaser.

  - The Purchasers right to inspect, test and, where necessary, reject the Goods’ arrival in at site shall in no way be limited or waived by reason of the Goods having previously been inspected, tested and passed by the Purchaser or its representative prior to the Goods dispatched.

7. PACKING

7.1 The Supplier shall provide such packing of the Goods as is required to  prevent  their  damage  or  deterioration  during  transit  to  their  final  destination as indicated in the Contract. The packing shall be sufficient  to  withstand,  without  limitation,  rough  handling  during  transit  and exposure to extreme temperatures, salt and precipitation during transit  and  open  storage.  Packing  case  size  and  weights  shall  take  into consideration, where appropriate, the remoteness of the Goods final  destination and the absence of heavy handling facilities at all points in transit.

7.2 The  packing,  marking  and  documentation  within  and  outside  the packages shall comply strictly with such special requirements as shall be provided in the agreement / purchase order including additional requirements, if any,  specified in SCC and in any subsequent instructions ordered by the Purchaser.

8 DELIVERY AND DOCUMENTS

8.1 Delivery, Installation, Testing and Commissioning of the Ventilator Autoclave shall be made by the Supplier in accordance with  the terms specified in the purchase order by the Purchaser. The details of dispatching and/or other documents to be furnished by the supplier are specified in the purchase order, if any.

9. INSURANCE

9.1 The Goods Supplied under the Contract shall be fully insured in Indian Rupees (as specified in work order / purchase order / contract) against  the  loss  or  damage  incidental  to  manufacture  or acquisition, transportation, storage and delivery in the manner specified in the Special Conditions of Contract.

10.  INCIDENTAL SERVICES

10.1 The supplier may be required to provide any or all of the following services, including additional services, if any, specified in SCC:

(a) Performance or supervision of the on-site assembly and/or start-up of the supplied goods

(b) Furnishing of tools required for assembly and/or maintenance of the supplied goods;

(c) Furnishing of detailed operations and maintenance manual

(d) performance or supervision or maintenance and/or repair of the supplied Goods, for a period of time agreed by the parties, provided that this service shall not relieve the Supplier of any warranty obligations under this Contract; and

(e) Training of the Purchaser’s Personnel, at the Supplier’s plant and/or on-site, in assembly, start-up, operation, maintenance and/or repair of the supplied Goods.

11 SPARE PARTS

1.1   As specified in the Special Conditions of Contract, the Supplier may be  required to provide any or all of the following materials and notifications and information pertaining to spare parts manufactured or distributed by the Supplier:

(a) Such spare parts as the Purchaser may select to purchase from the Supplier, providing that this selection shall not relieve the Supplier of any warranty obligations under the Contract; and

(b)In the event of termination of production of the spare parts:

(i) Advance notification to the Purchaser of the pending termination, in  sufficient  time  to permit  the  Purchaser  to  procure  needed requirements; and

(ii) Following such termination, furnishing at no cost to the Purchaser, the blueprints, drawings and specifications of the spare parts, if and when requested.

12. WARRANTY

12.1  The Supplier warrants that the Goods supplied under this Contract are of the most recent or current models and incorporate all 	recent improvements in design and materials unless provided otherwise in the Contract. The supplier further warrants that the Goods supplied under this Contract shall have no defect arising from design, materials or workmanship (except when the design and/or material is required by the 	Purchaser’s specifications) or from any act or omission of the Supplier, that may develop under normal use of the supplied Goods in conditions 	obtaining in the country of final destination

12.2   This warranty shall remain valid for minimum one year, as applicable, after the Goods or any portion thereof  as  the  case may  be,  have  been  delivered  to  and accepted/ installed & commissioned at the final destination indicated in the Contract.

12.3   The Purchaser shall promptly notify the supplier in writing of any claims arising under this warranty.

12.4   Upon  receipt  of  such  notice,  the  Supplier  shall,  within  the  period specified in SCC, and with all reasonable speed, repair or replace the defective Goods or parts thereof, without cost to the Purchaser

12.5   If the Supplier, having been notified, fails to remedy the defect(s) within the period specified in SCC, within a reasonable period, the Purchaser may proceed to take such remedial action as may be necessary, at the Supplier’s risk and expense and without prejudice to any other rights which the Purchaser may have against the Supplier under the contract.

13. PRICES

13.1  Prices  charged  by  the  Supplier  for  Goods  delivered  and  Services performed under the Contract shall not vary from the prices quoted by  	the Supplier in its bid, with the exception of any prices adjustments 	authorized in the special Conditions of Contract or in the Purchaser’s request for bid validity extensions, as the case may be.

14. CHANGE ORDERS

14.1 The Purchaser may at any time by written order given to the Supplier,  	make changes within the general scope of the Contract in any one or  	more of the following:

(a) drawings, designs or specifications, where Goods to be furnished  under  the Contract are to be specifically manufactured for the Purchaser;

(b) the method of shipping or packing

(c) the services to be provided by the Supplier.

14.2   If any such changes cause an increase or decrease in the cost of, or the time required for, the Supplier’s performance of any provisions under the Contract, an equitable adjustment shall be made in the Contract Price or delivery  schedule,  or  both,  and  the  Contract  shall  accordingly  be amended. Any claims by the Supplier for adjustment under this clause must be asserted within thirty (30) days from the date of the Supplier’s receipt of the Purchaser’s change order.

15. CONTRACT AMENDMENTS

15.1   Subject to GCC Clause 17, no variation in or modification of the terms of the Contract shall be made except by written amendment signed by the parties.

16. ASSIGNMENT

16.1   The Supplier shall not assign, in whole or in part, its obligations to perform under the contract, except with the Purchaser’s prior written 	consent.

17. SUBCONTRACTS

17.1 The supplier shall notify the Purchaser in writing of all subcontracts awarded under the contract if not already specified in his bid. Such notification, in his original bid or later, shall not relieve the Supplier from any liability or obligation under the contract.

18. LIQUIDATED DAMAGES

18.1  If the Supplier fails to deliver any or all of the Goods or to perform the Services within the period(s) specified in the Contract, the Purchaser shall, without prejudice to its other remedies under the Contract, deduct from the Contract Price, as liquidated damages, a sum equivalent to the percentage specified in SCC, up to a maximum deduction of the percentage specified in the SCC if any. Once the maximum is reached, the Purchaser may consider termination of the Contract.

19. TERMINATION BY DEFAULT

19.1  The Purchaser may, without prejudice to any other remedy for breach of  	contract, by written notice of default sent to the supplier, terminate the  Contract in whole or part;

(a)if the Supplier fails to deliver any or all of the goods within the time period(s) specified in the Contract,  or within any extension thereof granted by the Purchaser, or

(b) if the Supplier fails to perform any other obligation(s) under the contract.

19.2  In the event the Purchaser terminates the Contract in whole or in part,  the Purchaser may procure, upon such terms and in such manner as it  deems appropriate, Goods or Services similar to those undelivered, and  the Supplier shall be liable to the Purchaser for any excess costs for  such  similar  Goods.  However,  the  Supplier  shall  continue  the  performance of the Contract to the extent not terminated.

20.  FORCE MAJEURE

20.1    For purposes of this Clause “Force Majeure” means an event beyond the  control  of  the  Supplier  and  not  involving  the  Supplier’s  fault  or 	negligence and not foreseeable. Such events may include, but are not 	limited to, acts of the Purchaser either in its sovereign or contractual capacity,  wars  or  revolutions,  fires,  floods,  epidemics,  quarantine  restrictions and freight embargoes.

20.2 If a Force Majeure situation arises, the Supplier shall promptly notify the  Purchaser in writing of such conditions and the cause thereof. Unless 	otherwise  directed  by  the  Purchaser  in  writing,  the  Supplier  shall  	continue to perform its obligations under the Contract as far as is reasonably practical, and shall seek all reasonable alternative means for performance not prevented by the force majeure event.

21. TERMINATION FOR INSOLVENCY

21.1   The Purchaser may at any time terminate the Contract by giving written  	notice to the Supplier, if the Supplier becomes bankrupt or otherwise 	insolvent. In this event, termination will be without compensation to the  	Supplier, provided that such termination will not prejudice or affect any right of action or remedy which has accrued or will accrue thereafter to 	the Purchaser.

22. RESOLUTION OF DISPUTES

22.1   The Purchaser and the supplier shall make every effort to resolve  amicably by direct informal negotiation any disagreement or dispute  	arising between them under or in connection with the Contract.

If, after thirty (30)  days from the commencement  of  such informal  negotiations, the Purchaser and the Supplier have been unable to resolve amicably a Contract dispute, either party may require that the dispute be referred for resolution to the formal mechanisms specified in the Special Conditions of Contract. These mechanisms may include, but or not limited to, conciliation mediated by a third Party, adjudication in an agreed national forum, and national arbitration.

22 .3  The jurisdiction of any disputes, suits and proceeding arising out of the tender shall be only in the court of Thiruvananthapuram.

23. GOVERNING LANGUAGE

23.1   The contract shall be written in English language. English language version of the Contract shall govern its interpretation. All correspondence and documents pertaining to the Contract which are exchanged by the parties shall be written in the same language.

24. APPLICABLE LAW

24.1   The Contract shall be interpreted in accordance with the laws of the Union of India.

25. NOTICES

25.1   Any notice given by one party to the other pursuant to this Contract shall be sent to other party in writing or by fax or email and confirmed in writing to the other Party’s address specified in Special Conditions of Contract

25.2   A notice shall be effective when delivered or on the notice’s effective date, whichever is later.

26. TAXES AND DUTIES

26.1  Suppliers shall be entirely responsible for all taxes, duties, license fees, octroi  etc.,  incurred  until  Delivery, Installation, Testing and Commissioning  of  the  contracted  equipment / VENTILATOR AUTOCLAVE to  the 	Purchaser. However, Sales tax in respect of the transaction between the 	Purchaser and the Supplier shall be payable extra, if so stipulated in the 	Notification of Award.

SECTION IV

SPECIAL CONDITIONS OF CONTRACT

TABLE OF CLAUSES

| Sl No | Description | Page No. |
| --- | --- | --- |
| 1. | DEFINITIONS | 32 |
| 2. | INSPECTION AND TESTS | 32 |
| 3. | INSURANCE | 32 |
| 4. | INCIDENTAL SERVICES | 32 |
| 5. | SPARE PARTS | 32 |
| 6. | WARRANTY | 33 |
| 7. | EARNEST MONEY DEPOSIT | 33 |
| 8. | PRICES | 33 |
| 9. | SUBCONTRACTS | 33 |
| 10. | LIQUIDATED DAMAGES | 33 |
| 11. | RESOLUTION OF DISPUTES | 34 |
| 12. | INSTALLATION, VALIDATION& COMMISSIONING | 34 |
| 13. | OPERATIONAL ACCEPTANCE | 34 |
| 14. | TRAINING | 34 |
| 15. | ANNUAL MAINTENANCE CONTRACT | 35 |
| 16. | ENCLOSURES TO BID | 35-36 |
| 17. | SITE ACCEPTANCE TEST | 36 |

SPECIAL CONDITIONS OF CONTRACT

The following Special Conditions of Contract shall supplement the General Conditions of Contract. Whenever there is a conflict, the provisions herein shall prevail over those in the General Conditions of contract. The Corresponding clause number of the General Conditions is indicated in parentheses.

1. DEFINITIONS (GCC Clause 1)

(a)The Purchaser is HLL Lifecare Limited, Akkulam Factory, Thiruvananthapuram.

2. INSPECTION AND TESTS (GCC Clause 6)

2.1 The following inspection procedures  and tests are required by the Purchaser;  the  supplier shall get  inspected  in manufacturer’s  works  and  submit  a  test  certificate  and  also guarantee/warranty certificate that the equipment conforms to laid down specifications.

2.2 The Purchaser or its representative shall inspect and/or test any or all the equipment to confirm their conformity to the Contract specifications, prior to dispatch from the manufacturer’s premises. Such inspection and clearance will not prejudice the right of the consignee to inspect and test the equipment on receipt at destination.

2.3 If the equipment fails to meet the laid down specifications the supplier shall take immediate steps to remedy the deficiency or replace the defective equipment/it parts to the satisfaction of the Purchaser.

3. INSURANCE (GCC Clause 9)

3.1 For delivery of Ventilator Autoclave at site, the insurance shall be obtained by the Supplier in an amount equal to 110% of the value of the equipment from “Warehouse to Warehouse” (Final destinations) on “All Risks” basis including War Risks and Strike if applicable.

4. INCIDENTAL SERVICE (GCC Clause 10)

4.1 The following services covered under GCC Clause 10 shall be furnished and the cost shall be included in the contract price:

(a)Transportation, safe storage and handling of consignment      off site.

(b)On site assembly if any of the supplied equipment, installation, testing and commissioning of the equipment.

(c)Furnishing of detailed operations and maintenance manual for each appropriate unit of supplied equipment;

5 .SPARE PARTS (GCC Clause 11)

Add as Clause 11. to the GCC the following:

5.1 Supplier shall carry sufficient inventories to assure ex-stock supply of consumable  spares  for  the  Goods.  Other  main  spare  parts  and components shall be supplied as promptly as possible but in any case within one week of placement of order.

6. WARRANTY (GCC Clause 12)

Substitute GCC Clause 12.2 by the following:-

6.1 This warranty shall remain valid for minimum ONE year after Installation, Validation and Commissioning and handing over of the VENTILATOR AUTOCLAVE size 1850 DIA X 4200 LONG (mm) or any portion thereof as the case may be, have been accepted by HLL Lifecare Limited as indicated in the contract.

6.2 Any major repair pointed out by the Purchaser within the warranty period shall be rectified by the Supplier from the date of intimation within a period  of 3  calendar  days  and  commission  the  equipment  to  the satisfaction of the Purchaser. Failing which the Purchaser has the right to  levy  penalty  on  the  Supplier  per  day  or  part  thereof  for  the equipment until the equipment are repaired and commissioned to the satisfaction of the Purchaser.

6.3 The Supplier shall, in addition, comply with the performance and/ or consumption guarantees specified under the contract. If for reasons attributable to the Supplier, these guarantees are not attained in whole or in part, the Supplier shall at its discretion either:

(a) make such changes, modifications, and/or additions to the equipment or any part thereof as may be necessary in order to attain the contractual guarantees specified in the Contract at its own cost and expense and to carry out further performance tests in accordance with SCC 2:

7. Earnest Money deposit (EMD)

Technical Bid should accompany a DD drawn in favour of M/s.HLL LIFECARE LIMITED, and payable at Thiruvananthapuram for a sum of Rs.1,50,000/- towards Earnest Money Deposit, in case of overseas bidders for a sum of USD 2500.   Failing which the Tender is   likely to be summarily rejected.

8. PRICES (GCC Clause 13)

Substitute Clause 13.1 of the GCC with the following:

8.1 Prices payable to the Supplier as stated in the Contract shall not be subject to adjustment during performance of the Contract.

9. SUB CONTRACT (GCC Clause 17)

Add at the end of sub-clause 17.1 the following:

9.1 Sub-contract shall be only for bought-out items and sub-assemblies.

10. LIQUIDATED DAMAGES (GCC Clause 18)

For delays: Substitute GCC Clause 18.1 by the following:

10.1 If the Supplier fails to deliver any or all of the equipment or perform of services  within  the  time  period(s)  specified  in  the  Contract,  the purchaser  shall  without  prejudice  to  its  other  remedies  under  the Contract, deduct from the Contract Price, as liquidated damages, a sum equivalent to 0.5 percent of the value of order per week of delay subject  to a maximum deduction of 7.5%.  Once  the  maximum  is reached, the Purchaser may consider termination of the Contract.

11. RESOLUTION OF DISPUTES (GCC Clause 22)

Add as GCC Clauses 22.3 and 23.4 the following:

11.1 The dispute resolution mechanism to be applied pursuant to GCC Clause 22 shall be as follows:

(a)In the case of a dispute or difference arising between the Purchaser 
and a Supplier relating to any matter arising out of or connected with this 
agreement, such dispute or difference shall be settled in accordance 
with the Arbitration and Conciliation Act 1996 the Arbitral Tribunal shall 
consist of 3 Arbitrator, one each to be nominated by the Purchaser and 
the supplier. The third Arbitrator shall be chosen by the two Arbitrators 
so appointed by the parties and shall act as Presiding Arbitrator. In case 
of failure of the two Arbitrator appointed by the parties to reach upon a 
consensus with in a period of 30 days from the appointment of the 
Arbitrator  appointed  subsequently,  the  Presiding  Arbitrator  shall  be 
appointed by the President of Institute of Engineers (India).

11.2 The venue of arbitration shall be the place from where the Contract is issued (i.e.) Thiruvananthapuram

12.    INSTALATION, VALIDATION & COMMISSIONING

Design, manufacture, Supply, installation, validation and commissioning of VENTILATOR AUTOCLAVE size 1850 DIA X 4200 LONG (mm) shall be completed by the supplier in accordance with the terms and technical specifications made by the purchaser.

13.   Operational Acceptance:

- Performance Test: The performance test shall be conducted by the supplier during the commissioning of the facilities to ascertain whether the facilities can attain the functional guarantees.

- Operational Acceptance: Operational Acceptance shall occur in respect of the facilities when the performance test has been successfully completed and the functional performances are met.

14.TRAINING

The bidder shall provide training for maintenance staff of the purchaser free of cost where required.

The bidder shall provide all training material and documents.

Conduct of training of the purchaser’s personnel shall be on-site.

15. ANNUAL MAINTENANCE CONTRACT (AMC):

15.1 During AMC period, any major repair pointed out by the Purchaser shall  be rectified by the Supplier from the date of intimation within a period of 2 calendar days and commission the equipment to the satisfaction of the  	Purchaser. Failing which the Purchaser has the right to levy penalty on the Supplier  per  day  or  part  thereof  for  each  equipment  until  the  	equipments are repaired and commissioned to the satisfaction of the Purchaser.

15.2 The Supplier shall indicate clearly the free guarantee maintenance of the  whole system supplied by the Supplier and the same should not be less than 1 year.

15.3 The Supplier shall also indicate separately post guarantee maintenance  cost of the entire system for 5 years subsequent to Warranty  maintenance  	period  and  shall  clearly  indicate  year  wise  maintenance  cost  with  	probable cost  of  spares  required  for  each  year,  in  addition  to 	Annual maintenance charges.

16. ENCLOSURES TO BID:

16.1 Technical bid (Cover A):

Technical Bid shall include duly filled up Tender documents along with

- Tender document fee in the form of DD, drawn in favor of HLL Lifecare Limited, payable at Thiruvananthapuram, in case the document is downloaded from website.

- Bid Security / EMD amount in form of DD, drawn in favor of HLL Lifecare Limited, payable at Thiruvananthapuram.

c) Duly attested copy of License, approved by the concerned Licensing Authority.

d) Documentary evidence of constitution of firm such as Memorandum of Articles,  Partnership Deed, etc., with details of Name, Address, Tel. No.,      Fax No., E-mail Address of firm and the Managing Director / Partner /       Proprietor.

e) Authorization of senior responsible officer of the Company to transact   business.

f) Annual turnover statement for last three years certified by the Auditor.

g) Copies of Balance Sheet and Profit & Loss Account for three years certified by the Auditors.

h) Self certified statement of the Installed manufacturing capacity of the Items quoted.

i) Qualification Criteria – Section VI

j) Performance Statement - Section VIII

k) Technical literature and other documents in support of the goods /  services.

l) Specification compliance statement

m) List of items quoted (without prices)

n) Duly filled check list as per Section – XII

o) Copy of the receipt against the purchase of tender document.

16 .2 	Price Bid (Cover B):

Price bid shall include

a) Duly filled in Price Schedule - Section X and XI

b) Bid Form – Section IX

- The Annual maintenance charges for a period of 5 years subsequent to free guarantee maintenance period of 1 year along with probable cost of spare parts, as per Section XI.

- Price List  of critical essential spares valid for 2 years from the date of SAT

- Price List  of Optional accessories

Equipment wise separate list with details such as

1) Make:

- Model:

- Part No

4) Others

Please note that the Bidder runs the risk of his bid being rejected if the Price Schedule contains any conditional offers.

17. Site Acceptance Test (SAT)

SAT covers the installation, validation and commissioning and taking over of the equipment which includes taking three commercial batches of the product and QA clearing the batch pertaining to sterility as per standards specified in technical specifications.

SECTION V

TECHNICAL SPECIFICATIONS

TECHNICAL SPECIFICATIONS

VENTILATOR AUTOCLAVE -   (Quantity :1 No.)

This specification covers the requirements of design, manufacture, installation, testing and commissioning of VENTILATOR AUTOCLAVE:Size 1850 DIA X 4200 LONG (mm)  is as below:

# CHAMBER SIZE:

# 1850 mm DIA X 4200 mm LONG

# CAPACITY:

# The sterilizer should be capable of sterilizing 3500 single blood bags per charge.

# DESIGN STANDARD:

The sterilizer must be fabricated, tested, installed and commissioned as per the guidelines ISO 17665: 2006.

SAFETY:

The system must include an alarm which shall get activated if the utility air pressure goes below required value.

# CHAMBER:

The sterilization chamber shall be fabricated with SS 316 plates of minimum 6 mm thickness with full argon welding. The chamber shall be externally reinforced with carbon reinforcements welded on SS 304 backing strips. The jacket besides reinforcing the chamber also helps to maintain temperature uniformity in the sterilizing space.

|  | # CHAMBER | # TUBES IN HEAT EXCHANGER |
| --- | --- | --- |
| Working Pressure | 2.5 KG/CM2 | 6 KG/CM2 |
| Hydro test Pressure | 3.75 KG/CM2 | 9 KG/CM2 |
| Vacuum | FULL | N.A. |

# DOOR:

The sterilization chamber shall be provided with two sliding type doors. The door movement shall be horizontal. The door shall be made of SS 316 plate re-enforced with carbon steel support structure. Door opening and closing is to be automatic with aid of an electro-pneumatic circuit.

The following door safety features shall be provided for maximum operator safety:

    - Door inter-lock to prevent simultaneous opening of both doors.

    - Process-lock to prevent opening of the door during the process

INSULATION:

The sterilization chamber is to be insulated with Rock Wool/Mineral Wool, which is held in place by an outer cover of SS 304.

FLOOR MOUNTING:

The sterilizer shall be mounted on a support suitable for installation on the floor and the support shall be manufactured of stainless steel.

APPROVED MATERIAL OF CONSTRUCTION:

| # Sl No. | # Element | # M.O.C |
| --- | --- | --- |
| # 1 | # Chamber | # SS 316 |
| # 2 | # Service Platform & Ladder | # SS 304 |
| # 3 | # Rail | # SS 316 |
| # 4 | # Heat Exchanger fins & tubes | # SS 316 L |
| # 5 | # Air steam circulation system | # SS 316 |

# DESIGN STANDARDS:

# The technical bids shall mandatorily include information on all design standards, relevant to the equipment, that were considered for design.

# INCLUSIONS:

# The technical bids shall mandatorily include the following details:

    - Process details including suitable graphs, charts and other indicators like F0 value,.etc

    - All G.A. drawings including drawings of SS 304 carriages, SS 304 trolleys and  Aluminum mesh trays shall be submitted along with bids. The bidder has to submit prices for these carriages which shall NOT be considered during price bid comparison.

    - Chamber design details

    - Door design details

    - Insulation details

    - Mounting specifications

    - Paneling specifications

    - Recirculation system specifications and drawings

    - Details of Heat exchanger

    - Service platform details

    - Piping package details

    - Valve package details

    - Filters

    - Surface finish

    - Validation ports

    - Process management system including PLC, temperature sensors, Pressure sensors, Electrical switch gear, safeties and alarms, process recording including details of printing equipment and its interface.

    - Details of utility connections along with necessary specifications, sizes and drawings

    - List of makes of all components.

In addition to the above, other technical details that are deemed necessary, by the bidder, shall also be included in the technical bid submitted by the bidder.

# The supplier shall submit Operation manual in triplicate after successful validation and commissioning of the Autoclave.

# EXCLUSIONS:

Unloading of the machine at site.

Site clearance and all necessary civil work.

COMMISSIONING:

At the time of commissioning the following shall be met:

INSTALLATION QUALIFICATION, OPERATIONAL QUALIFICATION, & PERFORMANCE QUALIFICATION INCLUDING VALIDATION

These shall be performed after the completion of installation, by the supplier. The protocol for the same is to be provided to HLL in advance in both hard and soft copy formats.

Microbiological:

The microbiological performance qualification shall demonstrate the     adequacy of the process for the sterilization of product by the inactivation of biological indicators for steam sterilization, complying with ISO 17655:2006 guidelines.

The bio-burden of the product shall be established as described in ISO 11737-1.

The sterility of the product after validation trial should meet the requirements of ISO 11737-2.

The results of testing indicators for steam sterilization (10 indicators for steam sterilization), recovery media and culture conditions used to monitor the sterilization process shall comply with ISO 11138-3.

PROCESS CONTROL AND MONITORING:

Data shall be recorded for each sterilization load to demonstrate that the sterilization process specification has been met. These data include the following:

- Temperature and pressure in the chamber, monitored and recorded from a position which can be related to that at which it is a representative position within the chamber.

- Time of commencement, process and removal of load from sterilization of each sterilization load

- Evidence that the air-steam mixture has been admitted to the sterilizer chamber

LIST OF APPROVED MAKES OF COMPONENT FOR VENTILATOR AUTOCLAVE

| S No | Description | MAKE |
| --- | --- | --- |
| 1 | Programmable Logic controller | Mitsubishi |
| 2 | Operator Interface | Mitsubishi |
| 3 | Strip chart Recorder | Yokogawa |
| 4 | Pressure Transmitter | Siemens |
| 5 | Pressure/Compound Gauges | Forbes Marshall/ Waree |
| 6 | Steam Trap | Spirax |
| 7 | Filter Regulator Lubricator | Festo |
| 8 | Switch Gear & Relays | Siemens |
| 9 | Miniature Circuit Breaker | Siemens |
| 10 | Rotary Switch | Siemens |
| 11 | Pneumatic cylinders & solenoid valves | Festo/Rotex/SME |
| 12 | PU hoses & Push-and-fit | Festo |

Any deviation in makes of the above components shall lead to disqualification of bids. The makes of other components of the equipment shall be clearly specified by the bidder in the technical bid document.

SECTION VI

QUALIFICATION CRITERIA

(Referred to in Clause 11.2 of ITB)

- The bidder should be a manufacturer and should be in business of the similar equipments for the last three years

- The bidder should have designed, manufactured and commissioned at least one sterilizer for large volume parental manufacturing application during the last 3 years

- The average annual turnover of the bidder should be above Rs 20.0 Crore proven by balance sheet of three immediate previous financial years and they should produce proof for the same.

- The bidder should be profit making during the previous 3 years.

Note:-

- The bidder should furnish the information on past supplies in the Pro-forma given under Section - VIII.

- The bidder should furnish Registration Certificate and Certificate of Incorporation.

- Bidders shall invariably furnish documentary evidence (Client’s Certificate) in support of the satisfactory operation of the equipment as specified above.

- The Bidder shall furnish data to support that he has the financial and production capacity to perform the contract and complete the supplies within the stipulated delivery period.

- The bidder should furnish the copy of Audited balance sheet in proof  of financial strength.

- Conditional Bid will be summarily rejected

SECTION VII

PROFORMA FOR EQUIPMENT AND QUALITY CONTROL EMPLOYED

BY THE MANUFACTURER

(Please attach detailed pro-forma incorporating the information given below)

BID NO……………………….

DATE OF OPENING……..

NAME OF THE BIDDER:……………………………………..

(Note: All details should relate to the manufacturer for the items offered for supply).

1.	Name & Full address of the Manufacturer

(a) PAN No                          (b). ECC Code                    (c) IEC if applicable

(d) CST No                          (e) TIN No                           (f)   Any other details

2. (a) Telephone No.: Office/Factory/Works

(b) Fax No.: Office/Factory/Works

(c) e-mail address

- Location of the manufacturing factory

- Details of important Plant & Machinery functioning in each dept..

- Details of the process of manufacturer in the factory

- Details & stocks of raw materials held

- Production capacity of item(s) quoted for, with the existing Plant & Machinery

7.1 Normal                                  	7.2 Maximum

- Details of arrangement for quality control of products such as laboratory, testing equipment etc.

- Details of staff:

9.1 Details of technical supervisory staff in charge of production & quality control

  - Production: Managers/engineers/ officers/supervisors

  - QA / Q.C  :Managers/engineers/ officers/supervisors

  - Skilled labor employed

  - Unskilled labor employed

  - Maximum No. of workers (skilled & unskilled) employed on any day during the 18 months preceding the date of Tender.

- Whether Goods are tested to any standard specification?  If so, copies of specifications & original test certificates should be submitted.

- Whether bidder is willing to keep minimum stock of the item? If yes, how much can be kept?

12.	What is the lead-time for execution of order for mentioned quantity of the item?

SECTION VIII

PROFORMA FOR PERFORMANCE STATEMENT

Proforma for Performance Statement (for a period of last 5 years)

Bid Bid No. HLL/AFT/PUR/AUTOCLAVE/2013-14

Name of the firm:

| Order placed by (Full Address of Purchaser) | Order No. and Date | Description | Quantity of ordered | Value of order | Date of Completion of Supply & Installation | Remarks indicating reasons for late delivery, if any | Has the equipment been satisfactorily functioning? (Attach a certificate from the Purchaser) |
| --- | --- | --- | --- | --- | --- | --- | --- |

Signature and Seal of the Bidder ...................................

.

SECTION VIII (a)

ANNEXURE-I

| DETAILS OF MAJOR INSTALLATIONS OF SIMILAR LARGE VOLUME PARENTRAL STERILIZATION SYSTEMS SUCCESSFULLY COMMISSIONED BY THE TENDERER AS PRIME CONTRACTOR DURING THE LAST 3 YEARS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sl. No. | Name of Client with full address, telephone numbers and nature of work | ## Project details | Value of works completed (in lakhs) | Period of Completion with dates (in months) |

SIGNATURE OF BIDDER

SECTION VIII (a)

ANNEXURE – II

| DETAILS OF ONGOING INSTALLATIONS OF LARGE VOLUME PARENTRAL STERILIZATION SYSTEMS |  |  |  |
| --- | --- | --- | --- |
| Sl. No. | Name of Client with full address, telephone numbers and nature of work | ## Project details | Value of works (in lakhs) |

SECTION VIII (a)

ANNEXURE-III

DETAILS OF ORGANIZATION

| Sl.No | Name & Postal Address | Date of Birth | Qualification | Total experience in years | Name of organization served showing position held and period of service in each position |
| --- | --- | --- | --- | --- | --- |
| 1. 2 | # A.ENGINEERING PERSONNELS |  |  |  |  |
| 3. 4 |  |  |  |  |  |
| 5. 6 |  |  |  |  |  |
| 1. 2 | B.MANAGEMENT (NON TECHNICAL) |  |  |  |  |
| Attach an organization chart and resumes of all key technical persons for this project with their consent letters to work for this project. |  |  |  |  |  |

SIGNATURE OF BIDDER

SECTION VIII (a)

ANNEXURE IV

BIDDER'S EQUIPMENTS

LIST OF EQUIPMENTS

| SL.NO | EQUIPMENTS | NOS.AVAILABLE | NOS.PROPOSED TO BE DEPLOYED IN THIS PROJECT |
| --- | --- | --- | --- |

SIGNATURE OF BIDDER

SECTION VIII (a)

ANNEXURE V

# TECHNICAL SPECIFICATION TO BE ATTACHED

| 1.1 | VENTILATOR AUTOCLAVE |  |
| --- | --- | --- |
| a. | Manufacturer(Make) |  |
| b. | Model |  |
| c. | Rated Capacity (Dimension) |  |
| d. | Type |  |
| e. | Make of all electrical & mechanical items used, to be specified |  |
| 1.2 | MICROPROCESSOR |  |
| a. | Indicate-point wise display system etc. |  |
| b. | Interface with BMS |  |
| c. | Safety devices detail |  |
| d | Make & Model:- |  |
| 1.3 | GENERAL |  |
| a. | Overall Dimension (M): LxBxH |  |
| b. | Service Clearance required (M) LXBxH |  |
| c. | Size of Foundation (M) LxBxH |  |
| d. | Net weight (Kg) |  |
| e. | Shipping Weight (Kg) |  |
| f. | Noise Level (d BA) |  |
| g. | Applicable Voltage and Hz |  |
| h. | Maximum Running Current (A) |  |
| i. | Total Power Consumption (KW) |  |

SECTION VIII (a)

ANNEXURE VI

CERTIFICATE

I / we hereby certify that the information given with this bidding document is correct. If, at any stage, it is found to be incorrect, I / we understand that the contract will be liable to be terminated and action could be taken against me/us by the Company for damages.

SIGNATURE (S) OF BIDDER

(To be submitted in the letter pad of the firm indicating full name and address, telephone & fax numbers etc.)

SECTION VIII (b)

## HLL LIFECARE LIMITED

Akkulam Factory

THIRUVANANTHAPURAM-695017

HLL/AFT/PUR/AUTOCLAVE /2013-14

INDEMINITY CLAUSE

If the supplier fails to execute the order within the time prescribed for the Design, Manufacture, Supply, Installation, Validation and Commissioning of VENTILATOR AUTOCLAVE ordered or violates or infringes the existing rates as agreed to as mentioned in the supply order, the supplier shall and will indemnify the company against all loses or damages whatsoever to be incurred or sustained including the legal cost or expenses incurred by the company by reason of non-delivery of VENTILATOR AUTOCLAVE at agreed quantity and rate with in the time specified in the supply order.  The company will initiate legal action if the supplier fails to execute the supply order as per the schedule in the supply order for the actual loss suffered. No quantity tolerance will be permitted in this regard unless otherwise prior approval is taken by the company before dispatching any excess quantity supplied which shall be returned back on freight to pay basis at the risk of the supplier.   Responsiveness of the Bid shall be at the discretion of HLL.

The supplier shall have no right to change the quantity stipulated in the supply order.

Bid pronounced Non Responsive by HLL shall be summarily rejected.

The decision of HLL will be final and no correspondence of this shall be entertained.

We have read and understood the above conditions and agree to abide by the same.

PLACE:                                NAME AND SIGNATURE OF THE APPLICANT

DATE:                                                          (WITH OFFICE SEAL)

SECTION VIII (c)

## HLL LIFECARE LIMITED

Akkulam Factory

THIRUVANANTHAPURAM-695017

HLL/AFT/PUR/AUTOCLAVE/2013-14

#### DECLARATION

We confirm having read and understood all the specifications, instructions, forms, terms and conditions and other requirements of the above tender (both expressed and implied) in full and that we agree to abide by all without any deviation.

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

PART  B

SECTION IX

PRICE BID FORM

Date :..............................

IFB NO: HLL/AFT/PUR/AUTOCLAVE/2013-14

To:

HLL Lifecare Limited,

Akkulam Factory,

Sreekariam P.O.

Thiruvananthapuram - 17

Dear Sir

Having examined the Bidding Documents including Addenda Nos......................, the receipt of which is hereby duly acknowledged, we, the undersigned, offer to supply and deliver...........................................(Description of Goods and Services) in conformity with the said Bidding Documents for the sum of.............................. (Total Bid amount in Words and Figures) or such other sums as may be ascertained in accordance with the Schedule of Prices attached herewith and made part of this bid.

We agree to abide by price per unit mentioned in price schedule, for the bid validity period specified in the bid document and it shall remain binding upon us and may be accepted by you at any time before the expiration of that period.

Until a formal final contract is prepared and executed between us, this bid, together with your written acceptance of the bid and your notification of award, shall constitute a binding Contract between us. We understand that you are not bound to accept the lowest or any bid you may receive.

We confirm that we comply with the eligibility requirements as per ITB Clause 1 of the bidding documents.

Dated this ....... day of ............................ 20 ......

(signature)

(in the capacity of) __________________

Duly authorized to sign Bid for and on behalf of  _______________

SECTION X

PRICE SCHEDULE I (a)

(For Supply Part)

| S.No. | Item Description | Quantity | Brand/Model Country of Origin | Ex-factory price (Basic Unit Price exclusive of all levies & charges) | Packing and forwarding | Excise Duty |  | Sales Tax |  | Freight & Insurance |  | Others,if any | Unit Price (ColumnNo. E+F+H+J+L+M) | TOTAL AMOUNT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | % | Amt. | % | Amt. | % | Amt. |  |  |  |
| A | B | C | D | E | F | G | H | I | J | K | L | M |  |  |
| 1 | VENTILATOR AUTOCLAVE Size: 1850 DIA X 4200 (LONG)mm. |  |  |  |  |  |  |  |  |  |  |  |  |  |

** Detailed split up rate details may also be incorporated along with your offer.

Seal and Signature of the bidder

SECTION X

PRICE SCHEDULE I (b)

(For Installation testing and commissioning)

| S.No. | Item Description | Quantity | Price inclusive of all levies & charges | service Tax |  | Unit Price | Total Amount |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | % | Amt. |  |  |
| A | B | C | D | E | F | K |  |
| 1 | VENTILATOR AUTOCLAVE Size: 1850 DIA X 4200 (LONG)mm. | 1 Nos |  |  |  |  |  |

** Detailed split up rate details may also be incorporated along with your offer.

All other Statutory levies, such as Taxes, duties, KVAT, VAT, WCT, IT, ST & PF, ESI of your work men etc (are inclusive) will be deducted from bill, as applicable.

Seal and Signature of the bidder

SECTION XI

VENTILATOR AUTOCLAVE AMC Price Schedule (I)

| Year 1 Price |  |  | Year 2 Price |  |  | Year 3 Price |  |  | Year 4 Price |  |  | Year 5 Price |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spares | Others | Total | Spares | Others | Total | Spares | Others | Total | Spares | Others | Total | Spares | Others | Total |
| 1 A | 1 B | 1 C | 2 A | 2 B | 2 C | 3 A | 3 B | 3 C | 4 A | 4 B | 4 C | 5 A | 5 B | 5 C |

Scope of Contract:  No of Visit (during the contract period)    a) Preventive maintenance visit:…………

b) Break down maintenance  visit:……..   “VENTILATOR AUTOCLAVEAMC” Annual  maintenance Contract includes; labors,  spare  and  preventive & break down maintenance.

Note: Bidders should offer AMC for the entire quantity asked by the purchaser in the tender document.

Seal and Signature of the bidder

SECTION XII

CHECK LIST

| Sl. No. | Documents | Page No. | Remarks |
| --- | --- | --- | --- |
| 1 | Tender fee(in case of download)/EMD in the form of DD drawn in favor of HLL Lifecare Limited, Payable at Thiruvananthapuram |  |  |
| 2 | Copy of Receipt of payment against the purchase of tender document/ DD against the purchase of tender document (in case downloaded from website) |  |  |
| 3 | Market standing certificate to prove that the bidder manufactures the quoted items for the past FIVE years |  |  |
| 4 | Duly attested copy of factory license/ Industrial license |  |  |
| 5 | Duly attested copy of sales tax registration certificate |  |  |
| 6 | Duly attested copy of document to prove the legal status, place of registration and principal place of business of the undertaking |  |  |
| 7 | Duly attested copies of quality certificates for the products and quality system certifications |  |  |
| 8 | Copy of Balance sheet for the past three financial years, duly certified by a chartered accountant |  |  |
| 9 | Copy of Profit & Loss Account for three years certified by chartered accountant |  |  |
| 10 | Duly filled proforma showing details of Equipment and Quality Control employed by the manufacturer as per section VII |  |  |
| 11 | Proforma for Performance Statement (for a period of last FIVE years) as per Section VIII |  |  |
| 12 | Self attested statement showing financial capability necessary to perform the Contract |  |  |
| 13 | Self attested statement showing technical and production capability necessary to perform the Contract as per Section VI |  |  |
| 14 | Client's Certificate as per Section VI |  |  |

| 15 | Product brochure literature, write up etc. |  |  |
| --- | --- | --- | --- |
| 16 | Item-by-item commentary (compliance statement) on the Purchaser’s Technical Specifications demonstrating substantial responsiveness of the goods and services to those specifications or a statement of deviations and exceptions to the provisions of the Technical Specifications |  |  |
| 17 | The letter of authorization indicated by written power-of-attorney |  |  |
| 18 | Spare parts and after sales service facilities in India |  |  |
| 19 | Documentary evidence of constitution of firm such as Memorandum of Articles, Partnership Deed, etc., with details of Name, Address, Tel. No., Fax No., E-mail Address of firm and the Managing Director / Partner / Proprietor |  |  |
| 20 | Annual turnover statement for last three years certified by the Auditor |  |  |
| 21 | List of items quoted (without prices) |  |  |

SECTION XIII (TO BE PRINTED ON STAMP PAPER)

CONTRACT FORM FOR DESIGN, MANUFACTURE, SUPPLY, INSTALLATION, VALIDATION & COMMISSIONING OF VENTILATOR AUTOCLAVE

Between

HLL Lifecare Limited

…………….

……………..hereinafter called the ‘Purchaser’

And

…………(Name and Address of Supplier)

…………

………… 					hereinafter called the ‘Supplier’

Contract No. ………. Dated………

This is in continuation to this office’s Notification of Award No………….(Work order No.) dated ……...

- Name & address of the Supplier: ………………

- Purchaser’s TE document No. ……………(IFB No.) dated

- Minutes of negotiation meeting dated…….exchanged between the supplier and the purchaser in connection with this tender.

- In addition to this Contract Form, the following documents etc, which are included in the documents mentioned under paragraphs 2 and 3 above, shall also be deemed to form and be read and construed as integral part of this contract:

  - (i) 	General Conditions of Contract;

  - (ii) 	Special Conditions of Contract;

- (iii) 	List of Requirements;

- (iv)	Technical Specifications;

- (v) 	Quality Control Requirements;

- (vi) 	Price Schedule(s) furnished by the supplier in its Applications;

- (vii) Purchaser’s Notification of Award

Note: The words and expressions used in this contract shall have the same meanings as are respectively assigned to them in the conditions of contract referred to above. Further, the definitions and abbreviations incorporated under clause .. of Section II – ‘General Instructions to Applicant’ of the Purchaser’s TE document shall also apply to this contract.

- Some terms, conditions, stipulations etc. out of the above-referred documents are reproduced below for ready reference:

- Brief particulars of the goods and services which shall be supplied/ provided by the supplier are as under:

| Sl No. | Brief description of goods/services | Quantity to be supplied | Unit Price | Total price | Terms of delivery |
| --- | --- | --- | --- | --- | --- |

- Payment terms: The payment of goods will be made against the bills raised to the purchaser by the supplier as per the payment terms in Letter of Acceptance

- Paying authority : HLL Lifecare Limited

- Received and accepted this contract

(Signature, name and address of the supplier’s executive duly authorised to sign on behalf of the supplier)

For and on behalf of

(Name & Address of Supplier)

Date:

Place: Thiruvananthapuram

Witness 1: 					Witness 2:

Name : ............... 				Name : ...............

Signature:................				Signature:.................

- Received and accepted this contract

(Signature, name and address of the purchaser’s executive duly authorised to sign on behalf of the purchaser)

For and on behalf of

HLL Lifecare Limited

Date :

Place: Thiruvananthapuram

Witness 1: 					Witness 2:

Name : ............... 				Name : ...............

Signature:................				Signature:.................