##### HLL LIFECARE LIMITED

AKKULAM  FACTORY,   SREEKARIAM  P.O.  THIRUVANANTHAPURAM - 17

Sub: Tender for Renovation CIVIL WORKS for ART IVF CENTER

Amendment to TENDER NOTICE

at IMCH, Medical College Hospital, Kozhikode

BID REFERENCE : IFB NO: HLL/AFT/IMCH/ART-IVF/2014-15/III dtd 25/11/2014

.DATE OF    RECEIPT & OPENING OF TENDER

| Sl No | Original Tender Details | # Amended Tender Details |
| --- | --- | --- |
| 1. | a) Date of receipt of Bid is: 15/12/2014, 14.00 HRS b) Date of Opening of Technical Bid is: 15/12/2014, 14.30 HRS | - Date of receipt of Bid is: 18/12/2014, 14.00 HRS b) Date of Opening of Technical Bid is: 18/12/2014, 14.30 HRS |

SECTION IX

## HLL LIFECARE LIMITED

Akkulam Factory, THIRUVANANTHAPURAM-695017

IFB NO: HLL/AFT/IMCH/ART-IVF/2014-15/III

| REVISED PRICE BID (SCHEDULE) CIVIL WORKS FOR ART IVF AT IMCH, MCH KOZHIKODE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (TO BE SUBMITED IN SEPARATE SEALED COVER) |  |  |  |  |  |  |  |  |  |  |  |
|  | Sl.No. |  |  | Description | Unit | Qty | Rate | Amount |  |  |  |
|  | 1 |  |  | Demolishing brick work manually/ by mechanical means including stacking of serviceable material,isolation of area with suitable material and disposal of unserviceable material within 200 metres lead as per direction of Engineer-in-charge. | cum | 2 |  |  |  |  |  |
|  | 2 |  |  | Brick work with common burnt clay F.P.S ( non modular ) bricks of class designation 7.5 in superstructure above plinth level upto 2 M height in all shades and sizes in cement mortar 1:4( 1 cement : 4 coarse sand | cum | 0.84 |  |  |  |  |  |
|  | 3 |  |  | 12 mm cement plaster of mix 1:4 (1 cement: 4 coarse sand) | sqm | 98 |  |  |  |  |  |
|  | 4 |  |  | Providing and fixing 18mm thick gang saw cut mirror polished premoulded and prepolished ) machine cut for kitchen platforms, vanity counters, window sills, facias and similar locations of required size of approved shade, colour and texture laid over 20mm thick base cement mortar 1:4 (1 cemnet :4 course sand) with joints treated with white cement, mixed with matching pigment, epoxy touch ups, including rubbing, curing, moulding and polishing to edges to give high gloss finish etc complete at all levels, Granite of any colour and shade. Area of slab over 0.5 sqm. | sqm | 4 |  |  |  |  |  |
|  | 5 |  |  | Providing edge moulding to 18mm thick marble stone counters, Vanities etc. including machine polishing to edge to give high gloss finish etc. complete as per design approved by Engineer-in-Charge, granite work | metre | 18 |  |  |  |  |  |
|  | 6 |  |  | Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners of required dia and size, including necessary filling up the gaps at junctions, i.e. at top, bottom and sides with required EPDM rubber/ neoprene gasket etc. Aluminium sections shall be smooth, rust free, straight, mitred and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing / paneling, C.P. brass / stainless steel screws, all complete as per architectural drawings and the directions of Engineer-in-charge. (Glazing, paneling and dash fasteners to be paid for separately) : |  |  |  |  |  |  |  |
|  | 6a |  |  | For Fixed Portion , Powder coated aluminium (minimum thickness of powder coating 50 micron) | Kg | 750 |  |  |  |  |  |
|  | 6b |  |  | For shutters of doors, windows & ventilators including providing and fixing hinges/ pivots and making provision for fixing of fittings wherever required including the cost of EPDM rubber / neoprene gasket required (Fittings shall be paid for separately). Powder coated aluminium (minimum thickness of powder coating 50 micron) | Kg | 247 |  |  |  |  |  |
|  | 7 |  |  | Providing and fixing 12mm thick prelaminated particle board flat pressed three layer or graded wood particle board conforming to IS: 12823 Grade l Type ll, in panelling fixed in aluminum doors, windows shutters and partition frames with C.P. brass / stainless steel screws etc. complete as per architectural drawings and directions of engineer-in-charge. | sqm | 84 |  |  |  |  |  |
|  | 8 |  |  | Providing and fixing glazing in aluminium door, window, ventilator shutters and partitions etc. with EPDM rubber / neoprene gasket etc. complete as per the architectural drawings and the directions of engineer-in-charge . (Cost of aluminium snap beading shall be paid in basic item):With float glass panes of 5.5 mm thickness | sqm | 58 |  |  |  |  |  |
|  | 9 |  |  | Providing and fixing aluminium sliding door bolts ISI marked anodised ( anodic coating not less than grade AC 10 as per IS: 1868) transparent or dyed to required colour or shade with nuts and screws etc. complete-300x16mm | each | 12 |  |  |  |  |  |
|  | 10 |  |  | Providing and fixing aluminium handles ISI marked anodised (anodic coating not less than grade AC 10 as per IS : 1868) transparent or dyed to required colour or shade with necessary screws etc. complete :125 mm | each | 12 |  |  |  |  |  |
|  | 11 |  |  | Providing and fixing P.V.C. waste pipe for sink or wash basin including P.V.C. waste fittings complete. |  |  |  |  |  |  |  |
|  | a) |  |  | Flexible pipe, 32 mm dia | 4.00 | each |  |  |  |  |  |
|  | 12 |  |  | Providing and fixing Stainless Steel A ISI 304 (18/8) kitchen sink as per IS 13983 with C.I. brackets and stainless steel plug 40 mm including painting of fittings and brackets, cutting and making good the walls wherever required : |  |  |  |  |  |  |  |
|  | a) |  |  | 600 x 400 x 250 depth | 4.00 | each |  |  |  |  |  |
|  | b) |  |  | 600mm long towel rail with total length of 645mm, width 78mm and effective height of 88mm, weighing not less than 190gms. | 4.00 | each |  |  |  |  |  |
|  | 13 |  |  | Providing and fixing PTMT liquid soap container 109mm wide, 125mm high and 112mm distance from wall of standard shape with bracket of the same materials with snap fittings of approved quality and colour. weighing not less than 105 gms. | 4.00 | each |  |  |  |  |  |
|  | 14 |  |  | Providing and fixing Chlorinated Polyvinyl Chloride (CPVC) pipes, having thermal stability for hot & cold water supply including all CPVC plain & brass threaded fittings i/c fixing the pipe with clamps at 1.00 m spacing. This includes jointing of pipes & fittings with one step CPVC solvent cement and the cost of cutting chases and making good the same including testing of joints complete as per direction of Engineer in Charge. |  |  |  |  |  |  |  |
|  |  |  |  | Concealed work including cutting chases and making good the walls etc |  |  |  |  |  |  |  |
|  | a) |  |  | Water line work -20 mm nominal outer dia .Pipes | 45.00 | metre |  |  |  |  |  |
|  | b) |  |  | Waste water line -40 mm nominal outer dia Pipes | 32.00 | metre |  |  |  |  |  |
|  | 15 |  |  | Removing white or colour wash by scrapping and sand papering and preparing the surface smooth including necessary repairs to scratches etc. complete | sqm | 650 |  |  |  |  |  |
|  | 16 |  |  | Providing and applying plaster of paris putty of 2 mm thickness over plastered surface to prepare the surface even and smooth complete | sqm | 650 |  |  |  |  |  |
|  | 17 |  |  | Wall painting with plastic emulsion paint of approved brand and manufacture to give an even shade : One or more coats on old work. | sqm | 650 |  |  |  |  |  |
|  |  | 18 | Half brick masonry with common burnt clay F.P.S (non modular) bricks of class designation 7.5 in superstructure above plinth level upto floor V level |  |  |  |  |  |  |  |  |
|  |  | a) | Cement mortar 1:4( 1 cement : 4 coarse sand |  | Sqm | 38.05 |  |  |  |  |  |
|  |  | 19 | Providing, hoisting and fixing up to floor five level precast reinforced cement concrete in lintels, beams and bressumers including setting in cement mortar 1:3 (1 cement : 3 coarse sand), cost of required centering and shuttering but excluding the cost o |  | cum | 1.15 |  |  |  |  |  |
|  |  | 20 | Steel reinforcement for R.C.C. work including straightening, cutting, bending, placing in position and binding all complete above plinth level. |  |  |  |  |  |  |  |  |
|  |  |  | Thermo-Mechanically Treated bars. |  | Kg | 200.00 |  |  |  |  |  |
|  |  | 21 | 12mm cement plaster of mix: |  |  |  |  |  |  |  |  |
|  |  |  | (1 cement: 4 fine sand) |  | sqm | 86.09 |  |  |  |  |  |
|  |  | 22 | Dismantling tile work in floors and roofs laid in cement mortar including stacking material within 50 metres lead. For thickness of tiles 10 mm to 25 mm |  | Sqm | 69.49 |  |  |  |  |  |
|  |  | 23 | Dismantling doors, windows and clerestory windows (steel or wood) shutter, including chowkhats, architrave, holdfasts etc. complete and stacking within 50 metres lead :Of area 3 sq. metres and below |  | Sqm | 3.6 |  |  |  |  |  |
|  |  | 24 | Providing and laying in position cement concrete of specified grade excluding he cost of centering and shuttering - All work up to plinth level : 1:1½:3 (1 Cement: 1½ coarse sand: 3 graded stone aggregate 20 mm nominal size) |  | cum | 4.83 |  |  |  |  |  |
|  |  | 25 | Providing and fixing white vitreous china pedestal type water closet (European type) with seat and lid, 10 litre low level white vitreous china flushing cistern & C.P. flush bend with fittings & C.I.brackets, 40 mm flush bend, overflow arrangement with sp |  | each | 4.00 |  |  |  |  |  |
|  |  | 26 | Providing and fixing wash basin with C.I. brackets, 15 mm C.P. Brass pillar taps, 32 mm C.P. brass waste of standard pattern, including painting of fittings and brackets, cutting and making good the walls wherever require: White Vitreous China Wash basin |  | each | 4.00 |  |  |  |  |  |
|  |  | 27 | Providing and fixing gun metal gate valve with C.I. wheel of approved quality -25MM |  | each | 8.00 |  |  |  |  |  |
|  |  | 28 | Providing and fixing C.P. brass long body bib cock of approved quality conforming to IS standards and weighing not less than 690 gms. |  | each | 4.00 |  |  |  |  |  |
|  |  | 29 | Providing and fixing trap of self cleansing design with screwed down or hinged grating with or without vent arm complete, including cost of cutting and making good the walls and floors : |  | each | 8.00 |  |  |  |  |  |
|  |  | 30 | Providing and fixing M.S. stays and clamps for sand cast iron/ centrifugally cast (spun) iron pipes of diameter : |  |  |  |  |  |  |  |  |
|  |  |  | 100 mm |  | each | 10.00 |  |  |  |  |  |
|  |  |  | 50mm |  | each | 10.00 |  |  |  |  |  |
|  |  |  | Providing and fixing bend of required degree with access door, insertion rubber washer 3 mm thick, bolts and nuts complete. |  |  |  |  |  |  |  |  |
|  |  | 31 | 100mm |  | each | 16.00 |  |  |  |  |  |
|  |  | 32 | Providing and fixing soil, waste and vent pipes : 17.35.1.1 Sand cast iron S&S pipe as per IS: 1729 |  | Meter | 15.00 |  |  |  |  |  |
|  |  | 34 | Waste water line -40 mm nominal outer dia Pipes |  | metre | 32 |  |  |  |  |  |
|  |  | 35 | Elbow 90o for 22.22 mm outer dia pipe |  | each | 20.00 |  |  |  |  |  |
|  |  | 36 | Providing and fixing factory made uPVC door frame made of uPVC extruded sections having an overall dimension as below (tolerance ±1mm), with wall thickness 2.0 mm (± 0.2 mm), corners of the door frame to be Jointed with galvanized brackets and stainless s |  | metre | 15.00 |  |  |  |  |  |
|  |  | 37 | 24 mm thick factory made PVC door shutters made of styles and rails of a uPVC hollow section of size 59x24 mm and wall thickness 2 mm (± 0.2 mm) with inbuilt edging on both sides. The styles and rails mitred and joint at the corners by means of M.S. galva |  | sqm | 6.72 |  |  |  |  |  |
|  |  | 38 | Providing and laying Vitrified Floor Tiles in different sizes (thickness to be specifoed by the manufacturer) with water absorption's less than 0.08 % and conforming to IS : 15622 of approved make in all colours and shades, laid on 20 mm thick cement mortar 1: 4(1 cement : 4 course sand)including grouting the joints with white cement and matching pigments etc., complete |  | sqm | 10 |  |  |  |  |  |
|  |  | 39 | Providing and fixing ceramic tile of Johnon /asian or equivalent ,colour and size (300 X 200 mm) as approved by Engineer in charge in dados over 12 mm thick bed of cement mortar (1 cement : 3 coarse sand)and jointing with filler of epoxy grouting as wall |  | sqm | 53.13 |  |  |  |  |  |
|  |  | 40 | Providing and fixing ceramic tile of Johnon /asian or equivalent ,colour and size (300 X 300 mm) as approved by Engineer in charge over 12 mm thick bed of cement mortar (1 cement : 3 coarse sand)and jointing with filler of epoxy grouting as floor tile |  | sqm | 16.36 |  |  |  |  |  |
|  |  | 41 | Dismantling or demolishing works such as floor demolishing ,digging etc for new toilet and tiles ,sanitary fitting dismantling etc for exsisting toilets |  | Ls |  |  |  |  |  |  |
|  |  | 42 | Supplying and providing two in one type tap with tap and health faucet ,good quality material with ISI standered,like jaguar etc |  | Eech | 4.00 |  |  |  |  |  |
|  |  | 43 | Demolishing for making holes for plumbing work,maintains works etc and finishing with proper meterials |  | Ls |  |  |  |  |  |  |

TOTAL Rs:

Signature of Tenderer

All other terms and conditions in the original Tender Notice and tender documents remain same and unchanged.

Joint General Manager (Materials)