TENDER   DOCUMENT FOR CIVIL, ELECTRICAL & INTERIOR WORKS AT LIFECARE CENTRE,

GOVERNMENT MEDICAL COLLEGE, ERNAKULAM

Tender No. HLL/CHO/HCS/PROJ/2014-15/003 dated 12-09-2014

HLL LIFECARE Ltd

CORPORATE AND REGD. OFFICE

HLL BHAVAN, POOJAPPURA

THIRUVANANTHAPURAM

0471-2354949, Ext. 331,325,326,289.

CONTENTS

| SL No. | ITEM | Page No. |
| --- | --- | --- |
| 1 | Notice Inviting Tender | 3 |
| 2 | Terms and Conditions | 4 - 6 |
| 3 | Annexure A - Schedule | 7 - 20 |

NOTICE INVITING TENDER

Sub: Civil, Electrical & Interior work for Lifecare Centre at Governement Medical College, Ernakulam

Tender No. - HLL/CHO/HCS/PROJ/2014-15/003 dated 12-09-2013

HLL Lifecare Ltd, a Government of India Undertaking, invites sealed and super scribed quotation from interested contractors having minimum two years of experience in the relevant field for the work at Regional Institute of Ophthalmology, Trivandrum for our Lifecare Centre.

Publishing date of bid: 12th September 2014.

Last date of submission (at the office of the Group Head (HCS), Corporate and Regd. Office, HLL Bhavan, Poojappura, Thiruvananthapuram): 26th September 2014, 15.00 Hours.

Opening of bid: 26th September 2014, 16.00 Hours at HLL Lifecare Ltd, Poojappura Thiruvananthapuram)

Interested applicants can participate in the bid.

HLL reserves the right to accept or reject any application without assigning any reason or incurring any liability whatsoever. Prospective bidders are advised to regularly scan through HLL web site as corrigendum/amendments etc., if any, will be notified on the HLL web site and separate advertisement will not be made for this regard

For HLL Lifecare Ltd.

Associate Vice President (HCS)i/c

GENERAL TERMS & CONDITIONS

- Eligibility criteria:

  - Bidders should have executed satisfactorily at least one job preferably of similar kind and contract should be valuing not less than ten lakhs during the last one year in satisfactory manner.

  - Bidders should have minimum two years’ experience in the relevant field.

Documental proof for the above eligibility criteria should be submitted along with offer.

- The schedule of quantities (ANNEXURE-A) is appended. Bidders are advised to go through the entire general terms and conditions of this contract carefully.

- Earnest money of Rs. 50,000/-(Rupees Fifty thousand only) in the form of a Demand Draft of a scheduled bank issued in favor of HLL Lifecare LTD, HLL Bhavan, Poojappura, Thiruvananthapuram, which should be placed in a separate sealed cover marked “Earnest Money” shall be submitted along with the tender.EMD of the unsuccessful bidders will be returned with in 15days from the date of issue of work order. The EMD of the successful bidder will be converted into security deposit. Security deposit will be released along with the final payment.

- The filled & duly signed tender (signature on all pages of the tender) should be submitted to Associate Vice President (HCS)i/c, Healthcare Service Division, HLL Lifecare Limited, HLL Bhavan, Poojappura, Thiruvananthapuram, PH- 0471-2354949 Ext. 331, 325, 326, 289 on or before 26-09-2014 by 15.00 Hrs. The application shall be clearly marked “Tender for Civil, Electrical & Interior work at Lifecare Centre at Government Medical College Hospital, Ernakulam”. The tender will be opened at 16.00Hrs on the same day itself in the presence of interested bidders or their authorized representatives who choose to attend at the time of opening of tender.

- The work shall be completed within 30 days from the date of receipt of Work Order or LOI

- The tender for the work shall remain open for acceptance for a period of 90days from the date of opening of tender.

- The contractor shall visit the site before quoting the rates and clarifications if any required can be had from the address mentioned above.

- Rates quoted should be inclusive of all cost of materials, Tools/Equipments labor charges, conveyance to site, handling charges, loading and unloading charges, hiring charges, clearing of debris, statutory payments etc.

- The Management reserves the right to accept or reject the Tender either partially or fully without assigning any reason what so ever.

- The work should be carried out without causing any inconvenience to the public and shall ensure that no damages are caused to the existing site premises.

- During the execution of work the contractor or authorized representative should be present at site.

- All Materials, Equipments/ Tools required for the work should be arranged by the contractor and brought to site for the timely completion of the work.

- The materials used shall be as per specification and of good quality.

- The Contractor has to arrange necessary insurance coverage for the machine, workmen etc. deployed by him. He shall arrange all safety measures to protect his workmen and also the properties of HLL.

- Retention Money and Defect liability period: Retention money will be recovered @ 5% on each bills submitted by the contractor. The retention money will be released after the defect liability period of one year from the date of issue of Work Completion Certificate. The work site safety of all employees, their ESI, PF etc will have to be borne by the contractor.

- Taxes & Duties: The amounts quoted in the tender  shall be inclusive of all taxes and duties

- Penalty for delay: Penalty shall be imposed at 0.5% per week to a maximum of 7.5% of the contract value for any delay committed by the contractor in completing the work as per the order.

- Measurement & Payment terms: The method of measurement of completed work shall be in accordance with the standard measurement. Payment will be made on satisfactorily completion of work as per the order. Interim or running account bills shall be submitted by the contractor for the work executed on the basis of such recorded measurements in the format of HLL. All such interim payments shall be regarded as payment by way of advances against final payment only, and shall not preclude the requiring of bad, unsound and imperfect or unskilled work to be rejected, removed, taken away and reconstructed or re erected. Any certificate given by the officer  relating to the work done or materials delivered forming part of such payment, may be modified, or corrected by any subsequent such certificate(s) or by the final certificate and shall not by itself be conclusive evident that any work or material so which it relates is /are in accordance with the contract and certificate. Any such interim payment,/any part  there of shall not in any respect conclude, determine or affect in any way powers of the engineer in charge under the contract or any of such payment s be treated as final settlement and adjustment of accounts or in any way vary or affect the contract.

- Payment Schedule: The contractor can submit two running account bills during the work period. 80% of the bill value will be paid on each running account bill submitted by the contractor.  The remaining 20% will be paid along with the final bill only after issue of Work Completion certificate by Engineer In Charge/Officer In Charge of HLL Lifecare Ltd  .The running account bills is to be submitted detailing the work description ,quantity and rate as per the Work Order. Payments will be made after effecting the statutory deductions of TDS, Work Contract Tax etc .Retention money @ 5% will be deducted on each bill as per clause 15.

- The Quantity shown in the schedule is an approximate estimated quantity. No rate revision will be entertained if the quantity exceeds due to the site condition while executing the work.

- Final payment shall be paid only after clearing the site as per direction of Engineer-in-charge/ Officer in charge.

- Settlement of Dispute: Arbitration shall not be the means of settlement of dispute or claims or   anything on account of this Contract. If any disputes and difference arising out of the contract are to be settled by a civil court at Thiruvananthapuram.

For HLL Lifecare Ltd.

Associate Vice President (HCS)i/c

ANNEXURE-A

| Item No | Details of items | Quantity | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | Demolishing brick work manually/ by mechanical means including stacking of serviceable material and disposal of unserviceable material within 50 metres lead as per direction of Engineer-in-charge.In cement mortar | 5.60 | m3 |  |  |
| 2 | Dismantling doors, windows and clerestory windows (steel or wood) shutter including chowkhats, architrave, holdfasts etc. complete and stacking within 50 metres lead : Of area 3 sq. metres and below | 7.00 | Each |  |  |
| 3 | Dismantling aerocon panel wall and stacking for disposal inside partition | 27.50 | m2 |  |  |
| 4 | Disposal of building rubbish / malba / similar unserviceable, dismantled or waste materials by mechanical means including loading, transporting, unloading to approved municipal dumping ground or as approved by Engineer-in-charge, beyond 50 m initial lead, for all loads including all lifts involved | 7.00 | m3 |  |  |
| 5 | Earth work in excavation by mechanical means (Hydraulic excavator) / manual means in foundation trenches or drains (not exceeding 1.5 m in width or 10 sqm on plan) including dressing of sides and ramming of bottoms, lift upto 1.5 m, including getting out the excavated soil and disposal of surplus excavated soil as directed, within a lead of 50 m. All kinds of soil. | 4.50 | m3 |  |  |
| 6 | Providing and laying in position cement concrete of specified grade excluding the cost of centering and shuttering - All work up to plinth level : 1:4:8 (1 Cement : 4 coarse sand : 8 graded stone aggregate 40 mm nominal size). | 1.20 | m3 |  |  |
| 7 | Providing and laying in position cement concrete of specified grade excluding the cost of centering and shuttering - All work up to plinth level : 1:2:4 (1 cement : 2 coarse sand : 4 graded stone aggregate 20 mm nominal size) | 2.60 | m3 |  |  |
| 8 | Brick work with common burnt clay F.P.S. (non modular) bricks of class designation 7.5 in superstructure above plinth level up to floor V level in all shapes and sizes in : Cement mortar 1:6 (1 cement : 6 coarse sand) | 2.00 | m3 |  |  |
| 9 | Providing and laying vitrified floor tiles in different sizes (thickness to be specified by the manufacturer) with water absorption's less than 0.08% and conforming to IS : 15622 of approved make in all colours and shades, laid on 20mm thick cement mortar 1:4 (1 cement : 4 coarse sand) including grouting the joints with white cement and matching pigments etc., complete. Size of Tile 600x600 mm | 60.00 | m2 |  |  |
| 10 | 12 mm cement plaster of mix : 1:4 (1 cement: 4 fine sand)for inside walls | 35.00 | m2 |  |  |
| 11 | Providing and fixing 12 mm thick frameless toughened glass door shutter of approved brand and manufacture, including providing and fixing top & bottom pivot & spring type fixing arrangement and making necessary holes etc. for fixing required door fittings, all complete as per direction of Engineer-in-charge (Door handle, lock and stopper etc.to be paid separately). | 2.50 | m2 |  |  |
| 12 | Supplying and fixing of BTS 75 V floor spring for Aluminium door (Dorma make) as per specification and as directed by the Engineer in charge. | 1 | no |  |  |
| 13 | Providing and fixing aluminium tubular handle bar 32 mm outer dia, 3.0 mm thick & 2100 mm long with SS screws etc .complete as per direction of Engineer-in-Charge. Powder coated minimum thickness 50 micron aluminium tubular handle bar. | 2 | Each |  |  |
| 14 | Reinforced cement concrete work in walls (any thickness), including attached pilasters, buttresses, plinth and string courses, fillets, columns, pillars, piers, abutments, posts and struts etc. up to floor five level excluding cost of centering, shuttering, finishing and reinforcement : 1:1.5:3 (1 cement : 1.5 coarse sand : 3 graded stone aggregate 20 mm nominal size) | 0.50 | m3 |  |  |
| 15 | Centering and shuttering including strutting, propping etc. and removal of form for : Suspended floors, roofs, landings, balconies and access platform. | 10.00 | m2 |  |  |
| 16 | Providing and fixing 18mm thick gang saw cut mirror polished premoulded and prepolished) machine cut for kitchen platforms, vanity counters, window sills , facias and similar locations of required size of approved shade, colour and texture laid over 20mm thick base cement mortar 1:4 (1 cement : 4 coarse sand) with joints treated with white cement, mixed with matching pigment, epoxy touch ups, including rubbing, curing, moulding and polishing to edges to give high gloss finish etc. complete at all levels. Area of slab over 0.50 sqm. | 9.00 | m2 |  |  |
| 17 | Providing edge moulding to 18mm thick marble stone counters, Vanities etc. including machine polishing to edge to give high gloss finish etc. complete as per design approved by Engineer-in-Charge. Granite work. | 20.00 | m2 |  |  |
| 18 | Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners of required dia and size, including necessary filling up the gaps at junctions, i.e. at top, bottom and sides with required EPDM rubber/ neoprene gasket etc. Aluminium sections shall be smooth, rust free, straight, mitred and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing / paneling, C.P. brass / stainless steel screws, all complete as per architectural drawings and the directions of Engineer-in-charge. (Glazing, paneling and dash fasteners to be paid for separately) : For fixed portion: Powder coated aluminium (minimum thickness of powder coating 50 micron) | 1030.00 | kg |  |  |
| 19 | Providing and fixing 12mm thick prelaminated particle board flat pressed three layer or graded wood particle board conforming to IS: 12823 Grade l Type ll, in panelling fixed in aluminum doors, windows shutters and partition frames with C.P. brass / stainless steel screws etc. complete as per architectural drawings and directions of engineer-in-charge. Pre-laminated particle board with decorative lamination on both sides: | 75.00 | m2 |  |  |
| 20 | Providing and fixing glazing in aluminium door, window, ventilator shutters and partitions etc. with EPDM rubber / neoprene gasket etc. providing necessary opening for coun ter etc. complete as per the architectural drawings and the directions of engineer-in-charge . (Cost of aluminium snap beading shall be paid in basic item): With float glass panes of 8 mm thickness | 11.00 | m2 |  |  |
| 21 | Providing and fixing anodised aluminium grill (anodised transparent or dyed to required shade according to IS: 1868 with minimum anodic coating of grade AC 15) of approved design/pattern, with approved standard section and fixed to the existing window frame with C.P. brass/ stainless steel screws @ 200mm centre to centre, including cutting the grill to proper opening size for fixing and operation of handles and fixing approved anodised aluminium standard section around the opening, Providing necessary opening of counters etc. all complete as per requirement and direction of Engineer-in-charge. (Only weight of grill to be measured for payment). | 55.00 | m2 |  |  |
| 22 | Supplying and fixing rolling shutters of approved make, made of required size M.S. laths, interlocked together through their entire length and jointed together at the end by end locks, mounted on specially designed pipe shaft with brackets, side guides and arrangements for inside and outside locking with push and pull operation complete, including the cost of providing and fixing necessary 27.5 cm long wire springs manufactured from high tensile steel wire of adequate strength conforming to IS: 4454 – part 1 and M.S. top cover of required thickness for rolling shutters. 80x1.20 mm M.S. laths with 1.20 mm thick top cover | 31.00 | m2 |  |  |
| 23 | Providing and fixing ball bearing for rolling shutters | 4.00 | Each |  |  |
| 24 | Steel work in built up tubular (round, square or rectangular hollow tubes etc.) trusses etc. including cutting, hoisting, fixing in position and applying a priming coat of approved steel primer, including welding and bolted with special shaped washers etc. complete. Hot finished welded type tubes | 1705.00 | kg |  |  |
| 25 | Providing and fixing precoated galvanised iron profile sheets (size, shape and pitch of corrugation as approved by Engineer-in-charge) 0.50 mm + 0.05 %, total coated thickness with zinc coating 120 gsm as per IS: 277 in 240 mpa steel grade, 5-7 microns epoxy primer on both side of the sheet and polyester top coat 15-18 microns. Sheet should have protective guard film of 25 microns minimum to avoid scratches while transportation and should be supplied in single length upto 12 metre or as desired by Engineer-in-charge. |  |  |  |  |
|  | The sheet shall be fixed using self drilling /self tapping screws of size (5.5x 55mm) with EPDM seal, complete upto any pitch in horizontal/ vertical or curved surfaces excluding the cost of purlins, rafters and trusses and including cutting to size and shape wherever required. | 77.00 | m2 |  |  |
| 26 | Providing and fixing precoated galvanised steel sheet roofing accessories 0.50 mm + 0.05 % total coated thickness, Zinc coating 120gsm as per IS: 277 in 240 mpa steel grade, 5-7 microns epoxy primer on both side of the sheet and polyester top coat 15-18 microns using self drilling/ self tapping screws complete : Ridges plain (500 - 600mm). | 6.50 | m |  |  |
| 27 | Providing and applying plaster of paris putty of 2 mm thickness over plastered surface to prepare the surface even and smooth complete | 42.00 | m2 |  |  |
| 28 | Applying one coat of water thinnable cement primer of approved brand and manufacture on wall surface : Water thinnable cement primer | 42.00 | m2 |  |  |
| 29 | Wall painting with acrylic emulsion paint of approved brand and manufacture to give an even shade:Two or more coats on new work- Inside walls -Two or more coats on new work. | 42.00 | m2 |  |  |
| 30 | Finishing walls with Acrylic Smooth exterior paint of required shade : New work (Two or more coat applied @ 1.67 ltr/10 sqm over and including priming coat of exterior primer applied @ 2.20 kg/ 10 sqm).-external walls | 50.00 | m2 |  |  |
| 31 | Painting with synthetic enamel paint of approved brand and manufacture to give an even shade : Two or more coats on new work. For steel works | 120.00 | m2 |  |  |
| 32 | Providing and fixing false ceiling at all height including providing and fixing of frame work made of special sections, power pressed from M.S. sheets and galvanized with zinc coating of 120 gms/sqm (both side inclusive) as per IS : 277 and consisting of angle cleats of size 25 mm wide x 1.6 mm thick with flanges of 27 mm and 37mm, at 1200 mm centre to centre, one flange fixed to the ceiling with dash fastener 12.5 mm dia x 50mm long with 6mm dia bolts other flange of cleat fixed to the angle hangers of 25x10x0.50 mm of required length with nuts & bolts of required size and other end of angle hanger fixed with intermediate G.I. channels 45x15x0.9 mm running at the rate of 1200 mm centre to centre to which the ceiling section 0.5 mm thick bottom wedge of 80 mm with |  |  |  |  |
|  | tapered flanges of 26 mm each having lips of 10.5 mm, at 450 mm centre to centre, shall be fixed in a direction perpendicular to G.I. intermediate channel with connecting clips made out of 2.64 mm dia x 230 mm long G.I. wire at every junction, including fixing perimeter channels 0.5 mm thick 27 mm high having flanges of 20 mm and 30 mm long, the perimeter of ceiling fixed to wall/partition with the help of rawl plugs at 450 mm centre, with 25mm long dry wall screws @ 230 mm interval, including fixing of gypsum board to ceiling section and perimeter channel with the help of dry wall screws of size 3.5 x 25 mm at 230 mm c/c, including jointing and finishing to a flush finish of tapered and square edges of the board with recommended |  |  |  |  |
|  | jointing compound , jointing tapes , finishing with jointing compound in 3 layers covering upto 150 mm or both sides of joint and two coats of primer suitable for board, all as per manufacturer’s specification and also including the cost of making openings for light fittings, grills, diffusers, cutouts made with frame of perimeter channels suitably fixed, all complete as per drawings, specification and direction of the Engineer in Charge but excluding the cost of painting with : 12.5 mm thick tapered edge gypsum fire resistant board conforming to IS: 2095- Part I. | 55.00 | m2 |  |  |
| 33 | Providing and fixing Pre-laminated (both side) flat pressed 3 layer (medium density) particle board or graded wood particle board IS : 3087 marked, Type II exterior grade IS : 12823 marked, in shelves with screws and fittings wherever required, including railing for the easy movement of drawers (fittings to be paid separately). 18 mm thick | 70.00 | m2 |  |  |
| 34 | Supplying and fixing 6mm thick glass panes for disply racks including necessary supports to the glass panes etc. complete | 14.00 | m2 |  |  |
| 35 | Supplying and fixing Mirror to the shutters 60x150cm side including fitting charges etc. complete. | 1.00 | Each |  |  |
| 36 | Providing and fixing bright finished brass handles with screws etc. complete : 125mm | 32.00 | Each |  |  |
| 37 | Design supply & installation of suspended Spider Glozing system designed to withstand the wind pressure as pr IS 875 (Part-III). The Suspended System held with Spider Fittings of SS-316 Grade The Suspended System held with Spider Fittings of SS-316 Grade Steel of approved manufacturer with glass panel having 12 mm thick clear toughened glass held together with SS- 316 Grade Stainless steel Spider & bolt assembly with laminated glass fins 21 mm thick. The Glass fins and glass panel assembly shall be connected to Slab/beams by means of SS- 316 Grade stainless steel brackets & Anchor bolts and at the bottom using SS channel of 50x25x2mm using fastener & anchor bolts, non staining weather sealants of |  |  |  |  |
|  | approved make, Teflon/ nylon bushes and separators to prevent bi-metallic contacts, all complete to perform as perspecification and approved drawings. The complete system to be designed to accommodate thermal expansion & seismic movements etc. The joints between glass panels (6 to 8 mm) and gaps at the perimeter & in U channel of the assembly to be filled with non staining weather sealant, so as to make the entire system fully water proof & dust proof. The rate shall include all design, Engineering and shop drawing including approval from structural designer, labour, T&P, scaffolding other incidental charges including wastage, enabling temporary services all fitting fixers nut bolts, washer, Buffer plates, fastener, anchors, SS channel laminated glass etc. all complete. |  |  |  |  |
|  | For the purpose of payment, actual elevation area of Glazing including thickness of joints and the portion of Glass panel inside the SS channel shall be measured. " | 17.00 | m2 |  |  |
| 38 | 1 nos stainless steel sink, water Tap and with all fittings | 1.00 | nos |  |  |
| 39 | Revolving type Office chair (make Godrej 0r featherlite) | 4.00 | nos |  |  |
| 40 | Wiring for light point/ fan point/ exhaust fan point/ call bell point with 1.5 sq.mm FRLS PVC insulated copper conductor single core cable in recessed medium class PVC conduit, with modular switch, modular plate, suitable GI box and earthing the point with 1.5 sq.mm. FR PVC insulated copper conductor single core cable etc as required. | 25 | Each |  |  |
| 41 | Wiring for circuit/submain wiring along with earth wire with the following sizes of FRLC PVC insulated copper conductor, single core cable in surface medium class PVC conduit as required. |  |  |  |  |
| 41.1 | 2 x 1.5 sq.mm + 1 x 1.5 sq.mm earth wire | 100 | m |  |  |
| 41.2 | 2 x 2.5 sq.mm + 1 x 2.5 sq mm earth wire | 200 | m |  |  |
| 41.3 | 4 x 6 sq. mm + 2 x 6sq. mm earth wire | 100 | m |  |  |
| 42 | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface / recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
| 42.1 | 20 mm | 100 | m |  |  |
| 43 | Supplying and drawing 2 pair 0.5 sq mm FR PVC insulated annealed copper conductor, unarmored telephone cable in the existing surface/ recessed steel/ PVC conduit as required. |  |  |  |  |
| 43.1 | 2 Pair | 50 | m |  |  |
| 44 | Supplying and fixing following modular socket outlet on the existing modular plate & switch box including connections but excluding modular plate etc. as required. |  |  |  |  |
| 44.1 | Telephone socket outlet | 2 | Each |  |  |
| 45 | Supplying and fixing stepped type electronic fan regulator-modular on the existing modular plate switch box including connections but excluding modular plate etc. as required. | 3 | Each |  |  |
| 46 | Supply and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. | 10 | Each |  |  |
| 47 | Supplying and fixing following size / modules, GI box alongwith modular base & cover plate for modular switches in recess etc as required. |  |  |  |  |
| 47.1 | 1 or 2 module | 2 | Each |  |  |
| 48 | Supplying and fixing suitable size GI / PVC box with modular plate and cover in front on surface or in recess, furniture including providing and fixing 3 pin 5/6 amps modular socket outlet and 5/6 amps modular switch, connection etc. as required. | 16 | Each |  |  |
| 49 | Supplying and fixing suitable size GI / PVC box with modular plate and cover in front on surface or in recess, furniture including providing and fixing 6 pin 5/6 & 15/16 amps modular socket outlet and 15/16 amps modular switch, connection etc. as required. | 6 | Each |  |  |
| 50 | Supplying and fixing 3 pin, 5 amp ceiling rose on the existing junction box/ wooden block including connection etc as required. | 4 | Each |  |  |
| 51 | Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, false ceiling with chain and hook including connection with 1.5 sq. mm FRLS PVC insulated, copper conductor, single core cable and earthing etc. as required. | 5 | Each |  |  |
| 52 | Installation, testing and commissioning of ceiling fan, including wiring the down rods of standard length (upto 30 cm) with 1.5 sq. mm FRLS PVC insulated, copper conductor, single core cable etc. as required. | 3 | Each |  |  |
| 53 | Supplying and drawing of UTP 4 pair CAT6 LAN cable in the existing surface / recessed steel / PVC conduit as required. | 50 | Each |  |  |
| 54 | Supplying and fixing following way prewired TP&N MCB distribution board of steel sheet for 415 volts on surface/ recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FRLS PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, din bar, detachable gland plate, interconnections, powder painted including earthing etc. as required.(But without MCB/ RCCB/ Isolator) |  |  |  |  |
| 54.1 | 4 way, Double door | 1 | Each |  |  |
| 55 | Supplying and fixing 5 amps to 32 amps rating, 240/415 volts, "C" curve, miniature circuit breaker suitable for inductive load of following poles in the existing MCB DB complete with connections, testing and commissioning etc. as required. |  |  |  |  |
| 55.1 | Single pole | 10 | Each |  |  |
| 55.2 | Triple pole and neutral | 1 | Each |  |  |
| 56 | Supplying and fixing single pole blanking plate in the existing MCB DB complete etc. as required. | 2 | Each |  |  |
| 57 | Supplying and fixing following rating, four pole, 415 volts, isolator in the existing MCB DB complete with connections, testing and commissioning etc. as required. |  |  |  |  |
| 57.1 | 40 amps | 1 | Each |  |  |
| 58 | Supplying and fixing following rating, four pole, (three phase and neutral), 415 volts, residual current circuit breaker (RCCB), having a sensitivity current upto 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc. as required. |  |  |  |  |
| 58.1 | 40 amps | 1 | Each |  |  |
| 59 | Earthing with GI earth pipe 4.5m long, 40mm dia including accessories, and providing masonary enclosure with cover plate having locking arrangement and watering pipe etc with charcoal / coke and salt as required. | 2 | Each |  |  |
| 60 | Supplying and laying 8 SWG GI wire at 0.5m below ground level for conductor earth electrode, including connection / termination with GI thimble etc as required. | 20 | m |  |  |
| 61 | Providing and fixing 8 SWG dia G.I. wire on surface or in recess for loop earthing as required. | 20 | m |  |  |
| 62 | Supply, installation, testing and commissioing of outdoor 3 phase direct reading energymeter board made of SMC / Polycarbonate for wall mounting, including internal wiring with suitable size wire. The meter board shall include supply and installation of following switchgear, metering instruments and with all required accessories as per statutory requirments. |  |  |  |  |
|  | 3 phase direct reading energy meter as per the requirment of KSEB with all connections, glass view panel, lockable compartment etc. |  |  |  |  |
|  | 3 nos 63A rewireable type baklite fuse unit with 1 no. Neutral link if required as per statutory. |  |  |  |  |
|  | 1 no. 40A, FP, C curve, 415V, MCB | 1 | Each |  |  |
| 63 | Supply of 28 W T5 surface mounted fluorescent luminarire CRCA sheet steel box type fitting with electronic ballast (THD less than 10%) with 28W T5 lamp with all accessories complete. (Philips TMS 122M IxTL5-28W EBE / GE GET CI 1x28 C/s HF / Crompton T5JB1328EB) | 5 | Each |  |  |
| 64 | Supply and installation of recess mounted 4x14W light fixture, complete with all accessories like electronic ballast (THD less than 10%) , 14W lamps etc complete with suitable length chain, anchor fastner, hook etc for the proper installation. (TBS 869 4xTL5-14W EBP D8 / GE SOC 414 / Crompton T5CQS11414EBB). | 2 | Each |  |  |
| 65 | Supply and installation of recess mounted LED downlight fixture 18W, with all accessories like driver, connector etc complete. (GE GDR018.40S1 / Crompton LCDH-20-CDL/Equi) | 12 | Each |  |  |
| 66 | Supply and installation of 28W T5 lamp with all accessories like lamp holder, electronic ballast, lamp, interconnecting wires etc complete with all accessories for proviging light inside the cupboard for placing opticals. (Philips TMS 095xTL5-28W / Equi) | 2 | Each |  |  |
| 67 | Supply and installation of Decorative Wall fan 400 mm sweep, 230V.1Ph, complete with Openable mesh Cover ,string Operated etc as required. (Crompton WM Wind Flo / Usha Helix high speed / Khaitan Merlin Hi speed) | 3 | Each |  |  |
| 68 | Supply of ceiling fan 1200 mm sweep,230V,1Ph, complete with double ball bearing, motor blades, down rods, canopy, capacitor etc complete as required. (Crompton Riviera / Usha Maxx Air / Khaitan Newtec) | 3 | Each |  |  |
| 69 | Supply and Installation of 10 pair MDF box, back mount frame and original krone connectors with all accessories. | 1 | Each |  |  |
| 70 | Supply & fixing modular type computer outlet (RJ-45) with modular base and cover plate, GI box complete with all accessories, connection etc. complete as required. | 3 | Each |  |  |
| 71 | Dismantling the existing light fixing, ceiling fan, exhaust fan, wall fan, DBs etc.and stacking the same to the place as per the direction of engineer in charge. | 20 | Each |  |  |
| 72 | Dismantling the existing conduiting, wiring, switch box including switches & sockets, any other electrical accessories in the area for proposed Optical shop, Pharmacy and waiting area. | 1 | Job |  |  |
| 73 | Preparation and submission of drawings, applications to KSEB for service connection, power allocation, sanction and energising of the installation complete Including providing necessary assistance to the client for obtaining sanctions. | 1 | Job |  |  |
| 74 | Supply, installation, testing and commissioning of High Wall mounted Split Air Conditioner of capacity 1.5TR 5 star rated as per BEE consisting of high wall mounted type room unit (fan coil) with cordless remote control and one number outdoor condensing unit comprising of enegy efficient suitable rotary compressor for operation on single phase, 230 V, 50Hz supply, with 5 years warrenty with copper coils, fixing the outdoor unit on a raised platform as per manufacturers recommendation and fixing of room (fan coil) unit on wall, incl 5m distance refrigerant copper piping with 13mm nitrile rubber insulation, providing and making interconnection between room & condenser as per site conditions, comprising of 1" dia drain PVC pipe with insulation 2m, canvas connections, full electrical & control cabling from power plug near indoor unit and to outdoor unit, first charge refrigarant gas, suitable stabilizer etc ODU MS Stand complete as required. | 3 | Nos |  |  |
| 75 | Supply & laying Copper Refrigerant piping suitable for Split units, duly insulated with closed cell elastomeric nitrile rubber insulation, class-o, covered with two layers of shield coating, complete with elbows, T joints, sleeves etc., with GSS slotted rail supporting arrangement and GI clamp for individual pipes. | 20 | m |  |  |
| 76 | Supply and laying of PVC drain pipe of 25mm dia ISI marked, connecting the drain lines from the indoor units to floor sump, with 9mm nitrile rubber insulation as per specification | 30 | m |  |  |
| 77 | Supply of 270 Litres 5 star rated Fridge (make LG/Whirlpool) | 1 | nos |  |  |

Place                                                          Signature of the Bidder

Date                                                           Name of the Bidder