TENDER   DOCUMENT FOR CIVIL, ELECTRICAL & INTERIOR WORKS AT LIFECARE CENTRE,

GOVT. MEDICAL COLLEGE, THRISSUR

Tender No. HLL/CHO/HCS/PROJ/2014-15/006 dated 23-02-2015

HLL LIFECARE Ltd

CORPORATE AND REGD. OFFICE

HLL BHAVAN, POOJAPPURA

THIRUVANANTHAPURAM

0471-2354949, Ext. 331,325,326,289.

CONTENTS

| SL No. | ITEM | Page No. |
| --- | --- | --- |
| 1 | Notice Inviting Tender | 3 |
| 2 | Terms and Conditions | 4 - 6 |
| 3 | Annexure A - Schedule | 7 - 19 |

NOTICE INVITING TENDER

Sub: Civil, Electrical & Interior work for Lifecare Centre at Govt. Medical College Hospital, Thrissur

Tender No. - HLL/CHO/HCS/PROJ/2014-15/006 dated 23-02-2015

HLL Lifecare Ltd, a Government of India Undertaking, invites sealed and super scribed quotation from interested contractors/firms having minimum two years of experience in the relevant field for the work at Govt. Medical College, Thrissur

Publishing date of bid: 23rd February 2015.

Last date of submission (at the office of the Group Head (HCS), Corporate and Regd. Office, HLL Bhavan, Poojappura, Thiruvananthapuram): 10th March 2015, 15.00 Hours.

Opening of bid: 10th March 2015, 16.00 Hours at HLL Lifecare Ltd, Poojappura Thiruvananthapuram)

Interested applicants can participate in the bid.

HLL reserves the right to accept or reject any application without assigning any reason or incurring any liability whatsoever. Prospective bidders are advised to regularly scan through HLL web site as corrigendum/amendments etc., if any, will be notified on the HLL web site and separate advertisement will not be made for this regard

For HLL Lifecare Ltd.

Associate Vice President (HCS)

GENERAL TERMS & CONDITIONS

- Eligibility criteria:

  - Bidders should have executed satisfactorily at least one job preferably of similar kind and contract should be valuing not less than ten lakhs during the last one year in satisfactory manner.

  - Bidders should have minimum two years’ experience in the relevant field.

Documental proof for the above eligibility criteria should be submitted along with offer.

- The schedule of quantities (ANNEXURE-A) is appended. Bidders are advised to go through the entire general terms and conditions of this contract carefully.

- Earnest money of Rs. 40,000/-(Rupees Forty thousand only) in the form of a Demand Draft of a scheduled bank issued in favor of HLL Lifecare LTD, HLL Bhavan, Poojappura, Thiruvananthapuram, which should be placed in a separate sealed cover marked “Earnest Money” shall be submitted along with the tender. EMD of the unsuccessful bidders will be returned with in 15days from the date of issue of work order. The EMD of the successful bidder will be converted into security deposit. Security deposit will be released along with the final payment.

- The filled & duly signed tender (signature on all pages of the tender) should be submitted to Associate Vice President (HCS), Healthcare Service Division, HLL Lifecare Limited, HLL Bhavan, Poojappura, Thiruvananthapuram, PH- 0471-2354949 Ext. 331, 325, 326, 289 on or before 10-03-2015 by 15.00 Hrs. The application shall be clearly marked “Tender for Civil, Electrical & Interior work at Lifecare Centre at Govt. Medical College Hospital, Thrissur”. The tender will be opened at 16.00Hrs on the same day itself in the presence of interested bidders or their authorized representatives who choose to attend at the time of opening of tender.

- The work shall be completed within 30 days from the date of receipt of Work Order or LOI

- The tender for the work shall remain open for acceptance for a period of 90days from the date of opening of tender.

- The contractor shall visit the site before quoting the rates and clarifications if any required can be had from the address mentioned above.

- Rates quoted should be inclusive of all cost of materials, Tools/Equipments labor charges, conveyance to site, handling charges, loading and unloading charges, hiring charges, clearing of debris, statutory payments etc.

- The Management reserves the right to accept or reject the Tender either partially or fully without assigning any reason what so ever.

- The work should be carried out without causing any inconvenience to the public and shall ensure that no damages are caused to the existing site premises.

- During the execution of work the contractor or authorized representative should be present at site.

- All Materials, Equipments/ Tools required for the work should be arranged by the contractor and brought to site for the timely completion of the work.

- The materials used shall be as per specification and of good quality.

- The Contractor has to arrange necessary insurance coverage for the machine, workmen etc. deployed by him. He shall arrange all safety measures to protect his workmen and also the properties of HLL.

- Retention Money and Defect liability period: Retention money will be recovered @ 5% on each bills submitted by the contractor. The retention money will be released after the defect liability period of one year from the date of issue of Work Completion Certificate. The work site safety of all employees, their ESI, PF etc will have to be borne by the contractor.

- Taxes & Duties: The amounts quoted in the tender  shall be inclusive of all taxes and duties

- Penalty for delay: Penalty shall be imposed at 0.5% per week to a maximum of 7.5% of the contract value for any delay committed by the contractor in completing the work as per the order.

- Measurement & Payment terms: The method of measurement of completed work shall be in accordance with the standard measurement. Payment will be made on satisfactorily completion of work as per the order. Interim or running account bills shall be submitted by the contractor for the work executed on the basis of such recorded measurements in the format of HLL. All such interim payments shall be regarded as payment by way of advances against final payment only, and shall not preclude the requiring of bad, unsound and imperfect or unskilled work to be rejected, removed, taken away and reconstructed or re erected. Any certificate given by the officer  relating to the work done or materials delivered forming part of such payment, may be modified, or corrected by any subsequent such certificate(s) or by the final certificate and shall not by itself be conclusive evident that any work or material so which it relates is /are in accordance with the contract and certificate. Any such interim payment,/any part  there of shall not in any respect conclude, determine or affect in any way powers of the engineer in charge under the contract or any of such payment s be treated as final settlement and adjustment of accounts or in any way vary or affect the contract.

- Payment Schedule: The contractor can submit two running account bills during the work period. 80% of the bill value will be paid on each running account bill submitted by the contractor.  The remaining 20% will be paid along with the final bill only after issue of Work Completion certificate by Engineer In Charge/Officer In Charge of HLL Lifecare Ltd  .The running account bills is to be submitted detailing the work description ,quantity and rate as per the Work Order. Payments will be made after effecting the statutory deductions of TDS, Work Contract Tax, Service Tax (reverse charge), etc .Retention money @ 5% will be deducted on each bill as per clause 15.

- The Quantity shown in the schedule is an approximate estimated quantity. No rate revision will be entertained if the quantity increases/decreases due to the site condition while executing the work.

- Final payment shall be paid only after clearing the site as per direction of Engineer-in-charge/ Officer in charge.

- Settlement of Dispute: Arbitration shall not be the means of settlement of dispute or claims or   anything on account of this Contract. If any disputes and difference arising out of the contract are to be settled by a civil court at Thiruvananthapuram.

For HLL Lifecare Ltd.

Associate Vice President (HCS)

ANNEXURE- A

| SL. No | Particulars | Quantity | Unit | Rate(Rs.) | Amount(Rs.) |
| --- | --- | --- | --- | --- | --- |
| 1 | Site clearance and levelling | 1.00 | 1 Job |  |  |
| 2 | Earth work excavation in hard soil for foundation including all cost and labour charges etc complete | 8.00 | m3 |  |  |
| 3 | Random Rubble masonry in cement mortar 1:6 for foundation and basement including simultaneous pointing to the exposed face of masonary with the same mortar including scaffolding charges and curing etc complete | 15.00 | m3 |  |  |
| 4 | Concrete hollow block of size 40x20x15 of approved quality in cement mortar 1:5 for super structure including curing, hire charges for scaffolding, cost and conveyance of all materials & labour charges etc; complete | 11.00 | m3 |  |  |
| 5 | Supplying and providing partition with Aluminium frames ,partially glazed partially prelaminate partical board etc with door accessories. | 27.00 | m2 |  |  |
| 6 | Providing and fixing 12 mm thick frameless toughened glass door shutter of approved brand and manufacture, including providing and fixing top & bottom pivot & spring type fixing arrangement and making necessary holes etc. for fixing required door fittings, all complete as per direction of Engineer-in-charge (Door handle, lock and stopper etc.to be paid separately). | 9.50 | m2 |  |  |
| 7 | Providing aluminium door of size 1m x 2.1 m height with half portion plain glass 5 mm thick and balance portion with 12 mm thick Nova pan sheet both side laminated in pivot, handle, lock, door closer etc. complete. | 1.00 | nos |  |  |
| 8 | Supplying and fixing of BTS 75 V floor spring for Aluminium door (Dorma make) as per specification and as directed by the Engineer in charge. | 1.00 | each |  |  |
| 9 | Providing and fixing SS tubular handle bar 32 mm outer dia, 3.0 mm thick & 2100 mm long with SS screws etc .complete as per direction of Engineer-in-Charge. | 2.00 | each |  |  |
| 10 | Providing and fixing factory made UPVC door frame made of UPVC extruded sections having an overall dimension as below (tolerance ±1mm), with wall thickness 2.0 mm (± 0.2mm), corners of the door frame to be Jointed with galvanized brackets and stainless steel screws, joints mitred and Plastic welded. The hinge side vertical of the frames reinforced by galvanized M.S. tube of size 19 X 19mm and 1mm (± 0.1 mm) wall thickness and 3 nos. stainless steel hinges fixed to the frame complete as per manufacturers specification and direction of Engineer-in-charge Extruded section profile size 48x40 mm. | 11.50 | m |  |  |
| 11 | Providing and fixing to existing door frames. 24 mm thick factory made PVC door shutters made of styles and rails of a UPVC hollow section of size 59x24 mm and wall thickness 2 mm (± 0.2 mm) with inbuilt edging on both sides. The styles and rails mitred and joint at the corners by means of M.S. galvanised/plastic brackets of size 75x220 mm having wall thickness 1.0 mm and stainless steel screws. The styles of the shutter reinforced by inserting galvanised M.S. tube of size 20x20 mm and 1 mm (± 0.1 mm) wall thickness . | 3.20 | m2 |  |  |
| 12 | Supplying and fixing aluminium windows/ ventilators using 1.2mm thick section including all cost of material, labour etc complete as per the instruction of EIC | 8.50 | m2 |  |  |
| 13 | Providing MS grills for windows etc; using MS flat 32x8mm in section on top and bottom horizontally fixing 10mm MS square rod vertically at 12.5cm c/c and two nos 10mm square rod at 30cm apart from top and bottom including all labour and hire charges for fabricating and cost of material & applying a coat of iron primer and as per approved drawing etc complete | 300.00 | kg |  |  |
| 14 | Providing and fixing sliding windows using powder coated Indal aluminium section 8899,8926,4404 and complete with 4mm thick plain glass EPDM glass cut, with facility for opening | 12.00 | m2 |  |  |
| 15 | Supplying and fixing fixed glass windows including all cost and labour charges etc complete | 19.00 | m2 |  |  |
| 16 | Laying the inter locking tiles of apprived size and colour over a bed of 6mm metal not less than 50mm thick including cost of all material, earth leveling, compacting and concrete locking at the edges, etc, complete | 50.00 | m2 |  |  |
| 17 | Supplying and fixing rolling shutters of approved make made of 80x1.25 mm M.S. laths inter locked together through their entire length and jointed together at the end by end locks mounted on specially designed pipe shaft with brackets, side guides and arrangements for inside and outside locking with push and pull operation etc., complete | 15.00 | m2 |  |  |
| 18 | Reinforced cement concrete M20 for sunshade, lintels, beams & slab using 20mm graded hard granite broken stone including shuttering / form works and curing etc., complete but excluding cost of reinforcement steel works |  |  |  |  |
| 18.1 | a. lintel, beam | 0.50 | m3 |  |  |
| 18.2 | b. slab | 12.50 | m3 |  |  |
| 19 | Reinforcement works for RCC including straightening, cutting, bending hooking, lapping, placing in position, tying with binding wire of approved quality and gauge including cost of binding wire and anchoring to the adjoining members wherever necessary etc, including all laps and wastages etc., complete as per design and specifications and direction with all lead and lift | 1100.00 | kg |  |  |
| 20 | Plastering with cement mortar 1:4, 12mm one coat to the interior walls & curing etc complete | 58.00 | m2 |  |  |
| 21 | Outside plastering with cement mortar 1:4, 16mm thick including all cost and labour charges etc complete | 100.00 | m2 |  |  |
| 22 | Flooring with C.C 1:4:8 using 40mm metal including all cost and labour charges etc complete | 7.50 | m3 |  |  |
| 23 | Providing and fixing Ist quality ceramic glazed wall tiles 300 x 450mm size conforming to IS : 15622 (thickness to be specified by the manufacturer ) of approved make in all colours, shades except burgundy, bottle green, black of any size as approved by Engineer-in-Charge, in skirting, risers of steps and dados over 12 mm thick bed of cement Mortar 1:3 (1 cement : 3 coarse sand) and jointing with grey cement slurry @ 3.3kg per sqm including pointing in white cement mixed with pigment of matching shade complete. | 26.00 | m2 |  |  |
| 24 | Flooring including skirting with vitrified tiles (rustic) 600x600mm in size (make of RAK/ Khajaria / Nitco), colour and quality approved by the owner / Architect with cement slurry backing on a bed of cement and sieved sand mortar in the ratio 1:4,20 mm thick including pointing the joints with the joint filler (Roff/Fosroc) to match the colour with tiles curing etc., complete | 76.00 | m2 |  |  |
| 25 | Laying the counters with mirror polished export quality granite slab 15mm thick of size , colour and quality approved by the owner / architect, with cement slurry backing of cement and seived sand mortar 1:4, 20mm thick including pointing the joints with cement to match the colour of slabs, curing etc., complete | 10.00 | m2 |  |  |
| 26 | White washing to the underside of the wall and ceiling using white cement including all cost and labour charges etc complete | 60.00 | m2 |  |  |
| 27 | Colour washing to the inner side of the wall using emulsion including applying two coats of putty with all cost and labour charges etc complete | 60.00 | m2 |  |  |
| 28 | Painting two coast to the outer faces of exterior walls with antifungal weather proof paint (Apex/ ICI weather shield ) over one coat of primer and two coat of putty as approved by the engineer in charge., complete | 100.00 | m2 |  |  |
| 29 | Providing and fixing suspended false ceiling with PVC interlocking ceiling panels (sintex or similar) 3600x250x8mm securely fixed to the soffit by suitable hangers/ clamps and fixed to the walls using PVC cornice / aluminium L angle 24mx24mm along the entire perimeter with provision for trap door using aluminium box frame 25mmx25mm and shutters fixed with T -section, hinges including cost of all materials ,labour charges, all scaffolding charges etc; complete | 79.50 | m2 |  |  |
| 30 | Roof Truss Work using truss -OUTER FRAME 40X40X16 swg ,INSIDE BRAZING-20X20X16swg PURLINGS-32X32X16 swg,, LATEES-100X,X50X12swg around all vertical legs swg PLATES, 6mm MS plate, 12mm anchor bolts,LEGS-3'' box section 3.2mm thick GUTTERS-6'' PVC fixed by using 25x6mm MSflat clamp, SHEET-40tk Trafford galwaloom sheet, 55mm long self screws anti corrosive and painting -One coat primer, one coat enamel etc complete | 95.00 | m2 |  |  |
| 31 | Providing and fixing hard drawn steel wire fabric 75x25 mm mesh of weight not less than 7.75 Kg per sqm to window frames etc. including 62x19 mm beading of second class teak wood and priming coat with approved steel primer all complete | 13.00 | m2 |  |  |
| 32 | Providing and fixing water closet squatting pan (Indian type W.C. pan ) with 100mm sand cast Iron P or S trap, 10 litre low level white P.V.C. flushing cistern, including flush pipe, with manually controlled device (handle lever) conforming to IS : 7231, with all fittings and fixtures complete including cutting and making good the walls and floors wherever required : White Vitreous china Orissa pattern W.C. pan of size 580x440mm with integral type foot rests. | 1.00 | each |  |  |
| 33 | Providing and fixing wash basin with CI / MS brackets, 15mm CP brass Pillar taps, 32mm CP brass waste of standard pattern, including painting of fittings and brackets, cutting and making good the walls wherever required.: White Vitreous China Wash basin size 630 x 450mm with a single 15mm C.P brass piller tap. Make:Hindware,Cera,Jaquar etc.., | 1.00 | each |  |  |
| 34 | supply and fixing stainless steel sink(make Prince or similar brand) with 3 zinc coat ( make Jaguar) with all necessary fixures and labour charges | 2.00 | each |  |  |
| 35 | Providing and fixing PVC pipes for water supply pipe line including earthwork excavation cost of pipes, specials and labour charges etc. complete. |  |  |  |  |
| 35.1 | 20mm dia. PVC pipes 12kg/cm3 | 20.00 | m |  |  |
| 35.2 | 25mm dia. PVC pipes 10kg/cm3 | 15.00 | m |  |  |
| 36 | Providing and fixing PVC pipes for water supply pipe line including cost of pipes, specials and labour charges etc. Concealed work including cutting chases and making good the wall etc. |  |  |  |  |
| 36.1 | 20mm dia. PVC pipes 12kg/cm3 | 20.00 | m |  |  |
| 36.2 | 25mm dia. PVC pipes 10kg/cm3 | 15.00 | m |  |  |
| 37 | Providing and fixing C.P. brass bib cock of approved quality conforming to IS:8931. Make:Hindware,Cera,Jaquar etc.., 15 mm nominal bore. | 1.00 | Each |  |  |
| 38 | Providing and fixing C.P. brass stop cock (concealed) of standard design and of approved make conforming to IS:8931: 15 mm nominal bore. | 1.00 | Nos. |  |  |
| 39 | Providing and fixing C.P. brass angle valve for basin mixer and geyser points of approved quality conforming to IS:8931 a) 15 mm nominal bore: 15mm nominal bore | 1.00 | Nos. |  |  |
| 40 | Bottle trap 38mm single piece moulded with height of 270mm, effective length of tail pipe 260mm from the centre of the waste coupling 77mm breadth with 25mm minimum water seal, weighing not less than 263gms. | 1.00 | Nos. |  |  |
| 41 | Providing and fixing SS soap Dish Holder having length of 138mm, breadth 102mm, height of 75mm with concealed fitting arrangements. Weighing not less than 106 gms. | 1.00 | Nos. |  |  |
| 42 | Providing and fixing 600 x 450mm bevelled edge mirror of superior glass (of approved quality) complete with 6mm thick hard board ground fixed to wooden cleats with C.P. brass screws and washers complete. | 1.00 | Nos. |  |  |
| 43 | Providing and fixing SS towel rail complete with brackets fixed to wooden cleats with CP brass screws with concealed fitting arrangement of approved quality and colour. 600mm long towel rail with total length of 645mm, width 78mm and effective height of 88mm, weighing not less than 190gms. | 1.00 | Nos. |  |  |
| 44 | Providing and fixing marine ply wood or of similar make, in shelves with screws and fittings wherever required, including railing for the easy movement of drawers (fittings to be paid separately). 18 mm thick | 110.00 | m2 |  |  |
| 45 | Painting wood work with Deluxe Multi Surface Paint of required shade. Two or more coat applied @ 0.90 ltr/10 sqm over one coat of primer applied @0.75 ltr/ 10 sqm of approved brand or manufacture | 20.00 | m2 |  |  |
| 46 | Supplying and fixing 6mm thick glass panes for disply racks including necessary supports to the glass panes etc. complete | 21.00 | m2 |  |  |
| 47 | Supplying and fixing Mirror to the shutters 60x150cm side including fitting charges etc. complete. | 5.00 | Each |  |  |
| 48 | Providing and fixing bright finished brass handles with screws etc. complete : 125mm | 46.00 | Each |  |  |
| 49 | Wiring for light point/ fan point/ exhaust fan point/ call bell point with 1.5 sq.mm FRLS PVC insulated copper conductor single core cable in recessed medium class PVC conduit, with modular switch, modular plate, suitable GI box and earthing the point with 1.5 sq.mm. FR PVC insulated copper conductor single core cable etc as required. | 40.00 | Each |  |  |
| 50 | Wiring for circuit/submain wiring along with earth wire with the following sizes of FRLC PVC insulated copper conductor, single core cable in surface medium class PVC conduit as required. |  |  |  |  |
| 50.1 | 2 x 1.5 sq.mm + 1 x 1.5 sq.mm earth wire | 100.00 | m |  |  |
| 50.2 | 2 x 2.5 sq.mm + 1 x 2.5 sq mm earth wire | 200.00 | m |  |  |
| 50.3 | 2 x 4 sq.mm + 1 x 4 sq mm earth wire | 75.00 | m |  |  |
| 50.4 | 4 x 6 sq. mm + 2 x 6sq. mm earth wire | 100.00 | m |  |  |
| 51 | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface / recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
| 51.1 | 20 mm | 100.00 | m |  |  |
| 52 | Supplying and drawing 2 pair 0.5 sq mm FR PVC insulated annealed copper conductor, unarmored telephone cable in the existing surface/ recessed steel/ PVC conduit as required. |  |  |  |  |
| 52.1 | 2 Pair | 50.00 | m |  |  |
| 53 | Supplying and fixing following modular socket outlet on the existing modular plate & switch box including connections but excluding modular plate etc. as required. |  |  |  |  |
| 53.1 | Telephone socket outlet | 2.00 | Each |  |  |
| 54 | Supplying and fixing stepped type electronic fan regulator-modular on the existing modular plate switch box including connections but excluding modular plate etc. as required. | 8.00 | Each |  |  |
| 55 | Supply and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. | 10.00 | Each |  |  |
| 56 | Supplying and fixing following size / modules, GI box along with modular base & cover plate for modular switches in recess etc as required. |  |  |  |  |
| 56.1 | 1 or 2 module | 2.00 | Each |  |  |
| 57 | Supplying and fixing suitable size GI / PVC box with modular plate and cover in front on surface or in recess, furniture including providing and fixing 3 pin 5/6 amps modular socket outlet and 5/6 amps modular switch, connection etc. as required. | 10.00 | Each |  |  |
| 58 | Supplying and fixing suitable size GI / PVC box with modular plate and cover in front on surface or in recess, furniture including providing and fixing 6 pin 5/6 & 15/16 amps modular socket outlet and 15/16 amps modular switch, connection etc. as required. | 4.00 | Each |  |  |
| 59 | Supplying and fixing 3 pin, 5 amp ceiling rose on the existing junction box/ wooden block including connection etc as required. | 10.00 | Each |  |  |
| 60 | Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, false ceiling with chain and hook including connection with 1.5 sq. mm FRLS PVC insulated, copper conductor, single core cable and earthing etc. as required upto 450mm sweep | 17.00 | Each |  |  |
| 61 | Supplying and drawing of UTP 4 pair CAT6 LAN cable in the existing surface / recessed steel / PVC conduit as required. | 50.00 | per m |  |  |
| 62 | Supplying and fixing following way prewired TP&N MCB distribution board of steel sheet for 415 volts on surface/ recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FRLS PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, din bar, detachable gland plate, interconnections, powder painted including earthing etc. as required.(But without MCB/ RCCB/ Isolator) |  |  |  |  |
| 62.1 | 4 way, Double door | 1.00 | Each |  |  |
| 63 | Supplying and fixing 5 amps to 32 amps rating, 240/415 volts, "C" curve, miniature circuit breaker suitable for inductive load of following poles in the existing MCB DB complete with connections, testing and commissioning etc. as required. |  |  |  |  |
| 63.1 | Single pole | 10.00 | Each |  |  |
| 64 | Supplying and fixing single pole blanking plate in the existing MCB DB complete etc. as required. | 2.00 | Each |  |  |
| 65 | Supplying and fixing following rating, four pole, 415 volts, isolator in the existing MCB DB complete with connections, testing and commissioning etc. as required. |  |  |  |  |
| 65.1 | 40 amps | 1.00 | Each |  |  |
| 66 | Supplying and fixing following rating, four pole, (three phase and neutral), 415 volts, residual current circuit breaker (RCCB), having a sensitivity current upto 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc. as required. |  |  |  |  |
| 66.1 | 40 amps | 1.00 | Each |  |  |
| 67 | Earthing with GI earth pipe 4.5m long, 40mm dia including accessories, and providing masonry enclosure with cover plate having locking arrangement and watering pipe etc with charcoal / coke and salt as required. | 2.00 | Each |  |  |
| 68 | Supplying and laying 8 SWG GI wire at 0.5m below ground level for conductor earth electrode, including connection / termination with GI thimble etc as required. | 20.00 | m |  |  |
| 69 | Providing and fixing 8 SWG dia G.I. wire on surface or in recess for loop earthing as required. | 20.00 | m |  |  |
| 70 | Supply, installation, testing and commissioning of outdoor 3 phase direct reading energy meter board made of SMC / Polycarbonate for wall mounting, including internal wiring with suitable size wire. The meter board shall include supply and installation of following switchgear, metering instruments and with all required accessories as per statutory requirements. 3 phase direct reading energy meter as per the requirement of KSEB with all connections, glass view panel, lockable compartment etc. 3 nos 63A rewireable type baklite fuse unit with 1 no. Neutral link if required as per statutory.1 no. 40A, FP, C curve, 415V, MCB | 1.00 | Each |  |  |
| 71 | Supply of 28 W T5 surface mounted fluorescent luminarire CRCA sheet steel box type fitting with electronic ballast (THD less than 10%) with 28W T5 lamp with all accessories complete. (Philips TMS 122M IxTL5-28W EBE / GE GET CI 1x28 C/s HF / Crompton T5JB1328EB) | 10.00 | Each |  |  |
| 72 | Supply and installation of recess mounted 4x14W light fixture, complete with all accessories like electronic ballast (THD less than 10%) , 14W lamps etc complete with suitable length chain, anchor fastener, hook etc for the proper installation. (TBS 869 4xTL5-14W EBP D8 / GE SOC 414 / Crompton T5CQS11414EBB). | 8.00 | Each |  |  |
| 73 | Supply of 18W surface mounted fluorescent luminarire CRCA sheet steel box type fitting with electronic ballast (THD less than 10%) with 18W lamp with all accessories complete. (Philips TMS 520/118 / GE LMS 14 DN / Crompton T5JB13114EB) | 3.00 | No |  |  |
| 74 | Supply and installation of 28W T5 lamp with all accessories like lamp holder, electronic ballast, lamp, interconnecting wires etc complete with all accessories for providing light inside the cupboard for placing opticals. (Philips TMS 095xTL5-28W / Equi.) | 8.00 | Each |  |  |
| 75 | Supply and installation of Decorative Wall fan 400 mm sweep, 230V.1Ph, complete with Openable mesh Cover ,string Operated etc as required. (Crompton WM Wind Flo / Usha Helix high speed / Khaitan Merlin Hi speed) | 1.00 | Each |  |  |
| 76 | Supply & fixing modular type computer outlet (RJ-45) with modular base and cover plate, GI box complete with all accessories, connection etc. complete as required. | 2.00 | Each |  |  |
| 77 | Preparation and submission of drawings, applications to KSEB for service connection, power allocation, sanction and energising of the installation complete Including providing necessary assistance to the client for obtaining sanctions. | 1.00 | Job |  |  |
| 78 | Supplying and fixing 5 star rated split AC for optical shop& pharmacy as per the load requirement. Make - Voltas, blue star or equivalent. |  |  |  |  |
|  | 1.5 Tr | 1.00 | Each |  |  |
|  | 1.00Tr | 1.00 | Each |  |  |
|  | 2.00Tr | 1.00 | Each |  |  |
|  | Grand Total |  |  |  |  |

Place                                                 Signature of the Bidder

Date                                                  Name of the Bidder