HLL LIFECARE LIMITED,

(A GOVERNMENT OF INDIA ENTERPRISES)

KANAGALA- 591225

TAL:HUKKERI, DIST: BELGAUM,

KARNATAKA- INDIA

TENDER ENQUIRY NO: HL: BG: PS: SN: E: 2012-13 Date: 16.10.2012

FOR SUPPLY OF SANITARY NAPKIN - PUTHUYUGAM  WITHOUT WINGS.

CORRIGENDUM – 6 DT. 26.12.2012

The following amendments have been made to the above Tender Enquiry.

- THE LAST DATE FOR SUBMISSION OF TENDER IS EXTENDED

- THE  SPECIFICATIONS OF THE PRODUCT ARE REVISED.

- THE MATERIAL SPECIFICATION OF THE THE PRODUCT IS REVISED.

- THE QUANTITY OF THE TENDER IS ENHANCED TO 600.00 M.Pcs..

- THE PRICE BID FORMAT HAS BEEN REVISED.

PLEASE REFER DETAILS BELOW.

TENDER ENQUIRY NO: HL: BG: PS: SN: E: 2012-13 Date: 16.10.2012

FOR SUPPLY OF SANITARY NAPKIN - PUTHUYUGAM  WITHOUT WINGS.

- THE LAST DATE FOR SUBMISSION OF TENDER MAY PLEASE BE READ AS  14.01.2013 INSTEAD OF 26.12.2012

- THE PRODUCT IS CLASSIFIED INTO THREE TYPES & ITS SPECIFICATION SPECIFICATIONS ARE AS FOLLOWS:

## TYPE-I - NW

(Napkin without wing + perforated poly top sheet+ higher weight)

| Sl. No. | Specification | Values |
| --- | --- | --- |
| 1 | Overall Length (mm) | 230 mm +/-5 |
| 2 | Fluff core / pad length (mm) | 210mm +-10 |
| 3 | Overall Width (mm) | 85 mm +/-5 |
| 4 | Fluff core / pad width (mm) | 65 mm +/-5 |
| 5 | Thickness of a single pad (mm) | 8 – 10 mm |
| 6 | Weight of a single pad (gm) | 8.5 – 9.5gm |
| 7 | Product Content | Perforated poly top sheet, First grade wood pulp central core. |

TENDER ENQUIRY NO: HL: BG: PS: SN: E: 2012-13 Date: 16.10.2012

FOR SUPPLY OF SANITARY NAPKIN - PUTHUYUGAM  WITHOUT WINGS.

TYPE-II- NW

(Napkin without wing + perforated poly top sheet)

| Sl. No. | Specification | Values |
| --- | --- | --- |
| 1 | Overall Length (mm) | 230 mm +/-5 |
| 2 | Fluff core / pad length (mm) | 210mm +-10 |
| 3 | Overall Width (mm) | 85 mm +/-5 |
| 4 | Fluff core / pad width (mm) | 65 mm +/-5 |
| 5 | Thickness of a single pad (mm) | 7 – 10 mm |
| 6 | Weight of a single pad (gm) | 7.5– 10gm |
| 7 | Product Content | Perforated polytop sheet, First grade wood pulp central core. |

TYPE- III - NW

(Napkin without wing + Non-woven top sheet)

| Sl. No. | Specification | Values |
| --- | --- | --- |
| 1 | Overall Length (mm) | 230 mm +/-5 |
| 2 | Fluff core / pad length (mm) | 210mm +-10 |
| 3 | Overall Width (mm) | 85 mm +/-5 |
| 4 | Fluff core / pad width (mm) | 65 mm +/-5 |
| 5 | Thickness of a single pad (mm) | 7 – 10 mm |
| 6 | Weight of a single pad (gm) | 7.5– 10gm |
| 7 | Product Content | Non-woven top sheet, First grade wood pulp central core. |

TENDER ENQUIRY NO: HL: BG: PS: SN: E: 2012-13 Date: 16.10.2012

FOR SUPPLY OF SANITARY NAPKIN - PUTHUYUGAM  WITHOUT WINGS.

C. THE MATERIAL SPECIFICATION OF ABOVE TYPES OF NAPKIN ARE AS FOLLOWS:

- TYPE-I - NW

- Detailed raw material specifications

| Raw Material | Specification |  |  |  |
| --- | --- | --- | --- | --- |
|  | Colour | Thickness | Width | Grade |
| Fluff pulp | White |  |  | Soft wood pulp, Long fiber |
| Perforated poly top sheet | Natural white | 18gsm+/- 10% | 110mm | Hydrophilic |
| Back sheet | White | 23gsm+/- 10% | 110mm | LDPE, Carona Treated or equivalent |
| Glue – Construction | Transparent |  |  | Polymer based |
| Glue – Release Paper | Transparent |  |  |  |
| Release paper | White with printing | 40gsm+/- 10% | 40mm | Silicon coated |
| Poly bag (Primary pack) | Four color printed | 40gsm+/- 10% |  | LDPE |
| Carton (Secondary packing)/ Corrugated boxes | Craft paper, Single color, printed |  |  | Made out of 5 ply (narrow flute), each ply 120 GSM virgin craft paper in the minimum burst factor of the 5 layers are 20. Internally stapled. Bursting Strength should not be less than 15 Kg/sq.cm. |

TENDER ENQUIRY NO: HL: BG: PS: SN: E: 2012-13 Date: 16.10.2012

FOR SUPPLY OF SANITARY NAPKIN - PUTHUYUGAM  WITHOUT WINGS.

TYPE-II- NW

- Detailed raw material specifications

| Raw Material | Specification |  |  |  |
| --- | --- | --- | --- | --- |
|  | Colour | Thickness | Width | Grade |
| Fluff pulp | White |  |  | Soft wood pulp, Long fiber |
| Perforated poly top sheet | Natural white | 18gsm+/- 10% | 110mm | Hydrophilic |
| Back sheet | White | 23gsm+/- 10% | 110mm | LDPE, Carona Treated or equivalent |
| Glue – Construction | Transparent |  |  | Polymer based |
| Glue – Release Paper | Transparent |  |  |  |
| Release paper | White with printing | 40gsm+/- 10% | 40mm | Silicon coated |
| Poly bag (Primary pack) | Four color printed | 40gsm+/- 10% |  | LDPE |
| Carton (Secondary packing)/ Corrugated boxes | Craft paper, Single color, printed |  |  | Made out of 5 ply (narrow flute), each ply 120 GSM virgin craft paper in the minimum burst factor of the 5 layers are 20. Internally stapled. Bursting Strength should not be less than 15 Kg/sq.cm. |

TENDER ENQUIRY NO: HL: BG: PS: SN: E: 2012-13 Date: 16.10.2012

FOR SUPPLY OF SANITARY NAPKIN - PUTHUYUGAM  WITHOUT WINGS.

TYPE-III- NW

- Detailed raw material specifications

| Raw Material | Specification |  |  |  |
| --- | --- | --- | --- | --- |
|  | Colour | Thickness | Width | Grade |
| Fluff pulp | White |  |  | Soft wood pulp, Long fiber |
| Nonwoven top sheet | Natural white | 18gsm+/- 10% | 110mm | Hydrophilic |
| Back sheet | White | 23gsm+/- 10% | 110mm | LDPE, Carona Treated or equivalent |
| Glue – Construction | Transparent |  |  | Polymer based |
| Glue – Release Paper | Transparent |  |  |  |
| Release paper | White with printing | 40gsm+/- 10% | 40mm | Silicon coated |
| Poly bag (Primary pack) | Four color printed | 40gsm+/- 10% |  | LDPE |
| Carton (Secondary packing)/ Corrugated boxes | Craft paper, Single color, printed |  |  | Made out of 5 ply (narrow flute), each ply 120 GSM virgin craft paper in the minimum burst factor of the 5 layers are 20. Internally stapled. Bursting Strength should not be less than 15 Kg/sq.cm. |

TENDER ENQUIRY NO: HL: BG: PS: SN: E: 2012-13 Date: 16.10.2012

FOR SUPPLY OF SANITARY NAPKIN - PUTHUYUGAM  WITHOUT WINGS.

- THE QUANTITY OF THE TENDER MAY BE READ AS 600.00 M.Pcs INSTEAD OF  300.00 M.PCS

- The Price Bid Format in Section 6 of the Tender documents may

please be read as follows:

Annexure – 1 in respect of INDIAN VENDORS:

F.O.R : VARIOUS DISTRICTS IN TAMILNADU

REVISED PRICE BID FORMAT

| Sl. No | F.O.R. | Qty. | Basic rate per piece | Excise duty | P&F | Local taxes (if any) | F&I | Landed price per piece |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | In Figure | In Words |
|  | Type I | 600 M.PCs |  |  |  |  |  |  |  |
|  | Type II |  |  |  |  |  |  |  |  |
|  | Type III |  |  |  |  |  |  |  |  |

# Annexure – II  IN RESPECT OF  FOREIGN VENDORS

REVISED FORMAT (A) FOR PRICE BID FOR FOREIGN VENDOR  (C.I.F: CHENNAI)

| Sl. No. | Item | Qty. | Price per piece | Landed price per piece |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | In Figure | In Words |
| 1. | Type I | 600 M.Pcs |  |  |  |
|  | Type II |  |  |  |  |
|  | Type III |  |  |  |  |

TENDER ENQUIRY NO: HL: BG: PS: SN: E: 2012-13 Date: 16.10.2012

FOR SUPPLY OF SANITARY NAPKIN - PUTHUYUGAM  WITHOUT WINGS.

REVISED FORMAT (B) FOR PRICE BID FOR FOREIGN VENDOR  (C.I.F: NHAVA SHEVA)

| Sl. No. | Item | Qty. | Price per piece | Landed price per piece |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | In Figure | In Words |
| 1. | Type I | 600 M.Pcs |  |  |  |
|  | Type II |  |  |  |  |
|  | Type III |  |  |  |  |

ALL OTHER CONTENTS OF THE TENDER REMAINS UNCHANGED.

DY.GENERAL MANAGER(S) i/c

HLL Lifecare Limited

(A Govt. of India Entp.)
                                                                               KANAGALA - 591 225.
                                                                       Belgaum Dist.

Karnataka State
                                                                              Tel: 08333 - 279239 / 44

Fax: 08333 – 279245