HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone: +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/CIVIL/2014-15/II-002		                                                 16/04/2014

## SECTION VIII

## PRICE SCHEDULE

Sub: Civil  work of Hospital Projects works at Himachal Pradesh  :   Mandi  District,:-

| Sl. No. | Details of Designated Site (Hospital Location) | Project reference |  |
| --- | --- | --- | --- |
| 1 | Public Health Centre, Nangwain, Mandi, Himachal Pradesh | Annexure 1 | a) CIVIL WORK OF NBCC |
| 2 | Community Health Centre, Rohanda, Mandi, Himachal Pradesh | Annexure 2 | a) CIVIL WORK OF NBCC |
| 3 | Community Health Centre, Dehar, Mandi, Himachal Pradesh | Annexure 3 | a) CIVIL WORK OF NBCC |
| 4 | Community Health Centre, Kottli, Mandi, Himachal Pradesh | Annexure 4 | a) CIVIL WORK OF NBCC |
| 5 | Civil Hospital, Karsog Mandi, Himachal Pradesh | Annexure 5 | a) CIVIL WORK OF NBCC |
|  |  |  | b) CIVIL WORK OF NBSU |
|  |  |  | c) CIVIL WORK OF BSC |

DRAWING ATTACHED

### The Tenderer has to submit their quote for all the schedules of the respective Tender.  If a tenderer is not quoting for all the schedules of the respective Tender, the tender will be  summarily rejected. HLL will place Work Order to the tenderer, whose total rate for all the hospital of the respective tender comes to lowest

IFB NO: HLL/AFT/TDG/HP-II/CIVIL/2014-15/II-002			                                      16/04/2014

## HLL LIFECARE LIMITED

Akkulam Factory

THIRUVANANTHAPURAM-695017

HIMACHAL PRADESH  :  MANDI DISTRICT

CIVIL  WORK

# ABSTRACT PRICE BID

| Sl. No. | Details of Designated Site (Hospital Location) |  | Amount (Rs) |
| --- | --- | --- | --- |
| 1 | Public Health Centre, Nangwain, Mandi, Himachal Pradesh | a) CIVIL WORK OF NBCC |  |
| 2 | Community Health Centre, Rohanda, Mandi, Himachal Pradesh | a) CIVIL WORK OF NBCC |  |
| 3 | Community Health Centre, Dehar, Mandi, Himachal Pradesh | a) CIVIL WORK OF NBCC |  |
| 4 | Community Health Centre, Kottli, Mandi, Himachal Pradesh | a) CIVIL WORK OF NBCC |  |
| 5 | Civil Hospital, Karsog Mandi, Himachal Pradesh | a) CIVIL WORK OF NBCC |  |
|  |  | b) CIVIL WORK OF NBSU |  |
|  |  | c) CIVIL WORK OF BSC |  |
| TOTAL Rs: |  |  |  |

The Tenderer has to submit their quote for all the schedules of the respective Tender.  If a tenderer is not quoting for all the schedules of the respective Tender, the tender will be  summarily rejected. HLL will place Work Order to the tenderer, whose total rate for all the hospital of the respective tender comes to lowest.

SEAL OF THE APPLICANT

Date:                                                                           SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone: +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/CIVIL/2014-15/II-002	                                                         16/04/2014

## PRICE SCHEDULE

## (Public Health Centre, Nangwain)

[Annexure 1 (a)]

| RENOVATION WORK OF NBCC FOR PHC NAGWAIN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SL. NO. | DESCRIPTION OF ITEM | QUANTITY | UNIT | RATE | AMOUNT |
| 1 | Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners | 68.75 | Kg |  |  |
| 2 | Providing and fixing 12mm thick prelaminated particle board flat pressed three layer or graded wood particle board conforming to IS: 12823 Grade l Type ll, in panelling fixed in aluminum doors, windows shutters and partition frames with C.P. brass / stain | 6.27 | Sqm |  |  |
| 3 | Providing and fixing glazing in aluminium door, window, ventilator shutters and partitions etc. with EPDM rubber / neoprene gasket etc. complete as per the architectural drawings and the directions of engineer-in-charge . (Cost of aluminium snap beading s | 2.22 | Sqm |  |  |
| 4 | Removing white or colour wash by scrapping and sand papering and preparing the surface smooth including necessary repairs to scratches etc. complete | 12.13 | Sqm |  |  |
| 5 | Providing and applying plaster of paris putty of 2 mm thickness over plastered surface to prepare the surface even and smooth complete | 12.13 | Sqm |  |  |
| 6 | Wall painting with plastic emulsion paint of approved brand and manufacture to give an even shade :One or more coats on old work. | 12.13 | Sqm |  |  |
| 7 | White washing with lime to give an even shade :Old work (one or more coats) | 2.73 | Sqm |  |  |
| TOTAL |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/CIVIL/2014-15/II-002			                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Rohanda)

[Annexure 2 (a)]

| RENOVATION WORK OF NBCC FOR CHC ROHANDA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SL. NO. | DESCRIPTION OF ITEM | QUANTITY | UNIT | RATE | AMOUNT |
| 1 | Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners | 68.75 | Kg |  |  |
| 2 | Providing and fixing 12mm thick prelaminated particle board flat pressed three layer or graded wood particle board conforming to IS: 12823 Grade l Type ll, in panelling fixed in aluminum doors, windows shutters and partition frames with C.P. brass / stain | 6.27 | Sqm |  |  |
| 3 | Providing and fixing glazing in aluminium door, window, ventilator shutters and partitions etc. with EPDM rubber / neoprene gasket etc. complete as per the architectural drawings and the directions of engineer-in-charge . (Cost of aluminium snap beading s | 2.22 | Sqm |  |  |
| 4 | Removing white or colour wash by scrapping and sand papering and preparing the surface smooth including necessary repairs to scratches etc. complete | 12.13 | Sqm |  |  |
| 5 | Providing and applying plaster of paris putty of 2 mm thickness over plastered surface to prepare the surface even and smooth complete | 12.13 | Sqm |  |  |
| 6 | Wall painting with plastic emulsion paint of approved brand and manufacture to give an even shade :One or more coats on old work. | 12.13 | Sqm |  |  |
| 7 | White washing with lime to give an even shade :Old work (one or more coats) | 2.73 | Sqm |  |  |
| TOTAL |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/CIVIL/2014-15/II-002			                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Dehar)

[Annexure 3 (a)]

| RENOVATION WORK OF NBCC FOR CHC DEHAR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SL. NO. | DESCRIPTION OF ITEM | QUANTITY | UNIT | RATE | AMOUNT |
| 1 | Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners | 68.75 | Kg |  |  |
| 2 | Providing and fixing 12mm thick prelaminated particle board flat pressed three layer or graded wood particle board conforming to IS: 12823 Grade l Type ll, in panelling fixed in aluminum doors, windows shutters and partition frames with C.P. brass / stain | 6.27 | Sqm |  |  |
| 3 | Providing and fixing glazing in aluminium door, window, ventilator shutters and partitions etc. with EPDM rubber / neoprene gasket etc. complete as per the architectural drawings and the directions of engineer-in-charge . (Cost of aluminium snap beading s | 2.22 | Sqm |  |  |
| 4 | Removing white or colour wash by scrapping and sand papering and preparing the surface smooth including necessary repairs to scratches etc. complete | 12.13 | Sqm |  |  |
| 5 | Providing and applying plaster of paris putty of 2 mm thickness over plastered surface to prepare the surface even and smooth complete | 12.13 | Sqm |  |  |
| 6 | Wall painting with plastic emulsion paint of approved brand and manufacture to give an even shade :One or more coats on old work. | 12.13 | Sqm |  |  |
| 7 | White washing with lime to give an even shade :Old work (one or more coats) | 2.73 | Sqm |  |  |
| TOTAL |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/CIVIL/2014-15/II-002		                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Kottli)

[Annexure 4 (a)]

| RENOVATION WORK OF NBCC FOR CHC KOTTLI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SL. NO. | DESCRIPTION OF ITEM | QUANTITY | UNIT | RATE | AMOUNT |
| 1 | Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners | 68.75 | Kg |  |  |
| 2 | Providing and fixing 12mm thick prelaminated particle board flat pressed three layer or graded wood particle board conforming to IS: 12823 Grade l Type ll, in panelling fixed in aluminum doors, windows shutters and partition frames with C.P. brass / stain | 6.27 | Sqm |  |  |
| 3 | Providing and fixing glazing in aluminium door, window, ventilator shutters and partitions etc. with EPDM rubber / neoprene gasket etc. complete as per the architectural drawings and the directions of engineer-in-charge . (Cost of aluminium snap beading s | 2.22 | Sqm |  |  |
| 4 | Removing white or colour wash by scrapping and sand papering and preparing the surface smooth including necessary repairs to scratches etc. complete | 12.13 | Sqm |  |  |
| 5 | Providing and applying plaster of paris putty of 2 mm thickness over plastered surface to prepare the surface even and smooth complete | 12.13 | Sqm |  |  |
| 6 | Wall painting with plastic emulsion paint of approved brand and manufacture to give an even shade :One or more coats on old work. | 12.13 | Sqm |  |  |
| 7 | White washing with lime to give an even shade :Old work (one or more coats) | 2.73 | Sqm |  |  |
| TOTAL |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/CIVIL/2014-15/II-002			                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Karsog)

[Annexure 5 (a)]

| RENOVATION WORK OF NBCC FOR CH KARSOG |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SL. NO. | DESCRIPTION OF ITEM | QUANTITY | UNIT | RATE | AMOUNT |
| 1 | Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners | 68.75 | Kg |  |  |
| 2 | Providing and fixing 12mm thick prelaminated particle board flat pressed three layer or graded wood particle board conforming to IS: 12823 Grade l Type ll, in panelling fixed in aluminum doors, windows shutters and partition frames with C.P. brass / stain | 6.27 | Sqm |  |  |
| 3 | Providing and fixing glazing in aluminium door, window, ventilator shutters and partitions etc. with EPDM rubber / neoprene gasket etc. complete as per the architectural drawings and the directions of engineer-in-charge . (Cost of aluminium snap beading s | 2.22 | Sqm |  |  |
| 4 | Removing white or colour wash by scrapping and sand papering and preparing the surface smooth including necessary repairs to scratches etc. complete | 12.13 | Sqm |  |  |
| 5 | Providing and applying plaster of paris putty of 2 mm thickness over plastered surface to prepare the surface even and smooth complete | 12.13 | Sqm |  |  |
| 6 | Wall painting with plastic emulsion paint of approved brand and manufacture to give an even shade :One or more coats on old work. | 12.13 | Sqm |  |  |
| 7 | White washing with lime to give an even shade :Old work (one or more coats) | 2.73 | Sqm |  |  |
| TOTAL |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/CIVIL/2014-15/II-002			                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Karsog)

[Annexure 5 (b)]

| RENOVATION WORK OF NBSU FOR CH KARSOG |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SL. NO. | DESCRIPTION OF ITEM | QTY | UNIT | RATE | AMOUNT |
| 1 | Dismantling aluminium/ Gypsum partitions, doors, windows, fixed glazing and false ceiling including disposal of unserviceable surplus material and stacking of serviceable material with in 200 meters lead as directed by Engineer-in-charge. | 0.00 | Sqm |  |  |
| 2 | Dismantling tile work in floors and roofs laid in cement mortar including stacking material within 200 metres lead, for thickness of tiles 10 mm to 25 mm. | 28.14 | Sqm |  |  |
| 3 | Half brick masonry with common burnt clay F.P.S ( non modular ) bricks of class designation 7.5 in superstructure above plinth level upto 2 M height in all shades and sizes in Cement mortar 1:4( 1 cement : 4 coarse sand | 0.00 | Sqm |  |  |
| 4 | 12 mm cement plaster of mix1:4 (1 cement: 4 coarse sand) | 2.00 | Sqm |  |  |
| 5 | Providing and fixing 18mm thick gang saw cut mirror polished premoulded and prepolished ) machine cut for kitchen platforms, vanity counters, window sills, facias and similar locations of required size of approved shade, colour and texture laid over 20mm thick base cement mortar 1:4 (1 cemnet :4 course sand) with joints treated with white cement, mixed with matching pigment, epoxy touch ups, including rubbing, curing, moulding and polishing to edges to give high gloss finish etc complete at all levels.Granite of any colour and shade. Area of slab over 0.5 sqm | 0.00 | Sqm |  |  |
| 6 | Providing edge moulding to 18mm thick marble stone counters, Vanities etc. including machine polishing to edge to give high gloss finish etc. complete as per design approved by Engineer-in-Charge. Granite work | 0.00 | metre |  |  |
| 7 | Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners of required dia and size, including necessary filling up the gaps at junctions, i.e. at top, bottom and sides with required EPDM rubber/ neoprene gasket etc. Aluminium sections shall be smooth, rust free, straight, mitred and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing / paneling, C.P. brass / stainless steel screws, all complete as per architectural drawings and the directions of Engineer-in-charge. (Glazing, paneling and dash fasteners to be paid for separately) :For Fixed Portion , Powder coated aluminium (minimum thickness of powder coating 50 micron) | 117.50 | Kg |  |  |
| 8 | Providing and fixing 12mm thick prelaminated particle board flat pressed three layer or graded wood particle board conforming to IS: 12823 Grade l Type ll, in panelling fixed in aluminum doors, windows shutters and partition frames with C.P. brass / stainless steel screws etc. complete as per architectural drawings and directions of engineer-in-charge.Pre-laminated particle board with decorative lamination on both sides. | 4.12 | Sqm |  |  |
| 9 | Providing and fixing glazing in aluminium door, window, ventilator shutters and partitions etc. with EPDM rubber / neoprene gasket etc. complete as per the architectural drawings and the directions of engineer-in-charge . (Cost of aluminium snap beading shall be paid in basic item)With TINTED float glass panes of 5.5 mm thickness : | 10.31 | Sqm |  |  |
| 10 | Providing and laying vitrified floor tiles in different sizes (thickness to be specified by the manufacturer) with water absorption's less than 0.08% and conforming to IS : 15622 of approved make in Off white colours and shades, laid on 20mm thick cement mortar 1:4 (1 cement : 4 coarse sand) / adhesive (Johnson or approved make) including grouting the joints with white cement and matching pigments etc., complete.Size of Tile 600x600 mm | 29.61 | Sqm |  |  |
| 11 | Providing and fixing ceramic wall tile of Johnon or equivalent , white colour and size (300x200mm) as approved by Engineer in charge in dados over 12 mm thick bed of cement mortar (1 cement : 3 coarse sand)and jointing with filler of epoxy grouting etc complete. | 52.54 | Sqm |  |  |
| 12 | Providing and fixing Stainless Steel A ISI 304 (18/8) kitchen sink as per IS 13983 with C.I. brackets and stainless steel plug 40 mm including painting of fittings and brackets, cutting and making good the walls wherever required :600X400X250mm | 1.00 | Each |  |  |
| 13 | Providing and fixing C.P brass elbow action pillar cock : 15mm nominal bore | 1.00 | Each |  |  |
| 14 | Providing and fixing P.V.C. waste pipe for sink or wash basin including P.V.C. waste fittings complete. Flexible pipe, 32 mm dia | 0.00 | Each |  |  |
| 15 | Providing and fixing Chlorinated Polyvinyl Chloride (CPVC) pipes, having thermal stability for hot & cold water supply including all CPVC plain & brass threaded fittings i/c fixing the pipe with clamps at 1.00 m spacing. This includes jointing of pipes & fittings with one step CPVC solvent cement and the cost of cutting chases and making good the same including testing of joints complete as per direction of Engineer in Charge. |  |  |  |  |
|  | Concealed work including cutting chases and making good the walls etc |  |  |  |  |
|  | 20 mm nominal outer dia .Pipes | 40.00 | metre |  |  |
|  | 25 mm nominal outer dia .Pipes | 45.00 | metre |  |  |
| 16 | Providing and fixing 600x450 mm beveled edge mirror of superior glass (of approved quality) complete with 6 mm thick hard board ground fixed to wooden cleats with C.P. brass screws and washers complete. | 1.00 | Each |  |  |
| 17 | Providing and fixing PTMT towel rail complete with brackets fixed to wooden cleats with CP brass screws with concealed fitting arrangement of approved quality and colour.600mm long towel rail with total length of 645mm, width 78mm and effective height of 88mm, weighing not less than 190gms. | 1.00 | Each |  |  |
| 18 | Providing and fixing PTMT liquid soap container 109mm wide, 125mm high and 112mm distance from wall of standard shape with bracket of the same materials with snap fittings of approved quality and colour. weighing not less than 105 gms. | 1.00 | Each |  |  |
| 19 | Removing white or colour wash by scrapping and sand papering and preparing the surface smooth including necessary repairs to scratches etc. complete | 53.06 | Sqm |  |  |
| 20 | Providing and applying plaster of paris putty of 2 mm thickness over plastered surface to prepare the surface even and smooth complete | 42.21 | Sqm |  |  |
| 21 | Wall painting with plastic emulsion paint of approved brand and manufacture to give an even shade :One or more coats on old work. | 63.69 | Sqm |  |  |
| 22 | Providing and fixing tiled false ceiling of approved materials of size 595x595 mm in true horizontal level suspended on inter locking metal grid of hot dipped galvanized steel sections ( galvanized @ 120 gsm/sqm, both side inclusive) consisting of main "T" runner with suitably spaced joints to get required length and of size 24x38mm made from 0.30mm thick (minimum) sheet, spaced at 1200mm center to center and cross "T" of size 24x25mm made of 0.30mm | 29.61 | Sqm |  |  |
| TOTAL |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/CIVIL/2014-15/II-002			                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Karsog)

[Annexure 5 (c)]

| RENOVATION WORK OF BSC FOR CHC KARSOG |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SL. NO. | DESCRIPTION OF ITEM | QTY | UNIT | RATE | AMOUNT |
| 1 | Demolishing R.C.C. work manually/ by mechanical means including stacking of steel bars and disposal of unserviceable material within 50 metres lead as per direction of Engineer - in- charge. | 0.00 | Cum |  |  |
| 2 | Extra for cutting reinforcement bars manually/ by mechanical means in R.C.C. or R.B. work (Payment shall be made on the cross sectional area of R.C.C. or R.B. work) as per direction of Engineer - in -charge. | 0.00 | Sqm |  |  |
| 3 | Extra for scrapping, cleaning and straightening reinforcement from R.C.C. or R.B. work | 0.00 | Kg |  |  |
| 4 | Half brick masonry with common burnt clay F.P.S ( non modular ) bricks of class designation 7.5 in superstructure above plinth level upto 2 M height in all shades and sizes in Cement mortar 1:4( 1 cement : 4 coarse sand | 0.00 | Sqm |  |  |
| 5 | 12 mm cement plaster of mix1:4 (1 cement: 4 coarse sand) | 1.20 | Sqm |  |  |
| 6 | Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners of required dia and size, including necessary filling up the gaps at junctions, i.e. at top, bottom and sides with required EPDM rubber/ neoprene gasket etc. Aluminium sections shall be smooth, rust free, straight, mitred and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing / paneling, C.P. brass / stainless steel screws, all complete as per architectural drawings and the directions of Engineer-in-charge. (Glazing, paneling and dash fasteners to be paid for separately) :For Fixed Portion , Powder coated aluminium (minimum thickness of powder coating 50 micron) | 51.00 | Kg |  |  |
| 7 | Providing and fixing 12mm thick prelaminated particle board flat pressed three layer or graded wood particle board conforming to IS: 12823 Grade l Type ll, in panelling fixed in aluminum doors, windows shutters and partition frames with C.P. brass / stainless steel screws etc. complete as per architectural drawings and directions of engineer-in-charge.Pre-laminated particle board with decorative lamination on both sides. | 1.49 | Sqm |  |  |
| 8 | Providing and fixing 18mm thick gang saw cut mirror polished premoulded and prepolished ) machine cut for kitchen platforms, vanity counters, window sills, facias and similar locations of required size of approved shade, colour and texture laid over 20mm thick base cement mortar 1:4 (1 cement : 4 coarse sand), joints treated with white cement, mixed with matching pigment, epoxy touch ups, including rubbing, curing, moulding and polishing to edges to give high gloss finish etc. complete at all levels. | 0.00 | Sqm |  |  |
| 9 | Providing edge moulding to 18mm thick marble stone counters, Vanities etc. including machine polishing to edge to give high gloss finish etc. complete as per design approved by Engineer-in-Charge. Granite work | 3.35 | metre |  |  |
| 10 | Providing and fixing glazing in aluminium door, window, ventilator shutters and partitions etc. with EPDM rubber / neoprene gasket etc. complete as per the architectural drawings and the directions of engineer-in-charge . (Cost of aluminium snap beading shall be paid in basic item)With float glass panes of 5.5 mm thickness : | 4.29 | Sqm |  |  |
| 11 | Providing and fixing ceramic wall tile of Johnon or equivalent , white colour and size (300x200mm) as approved by Engineer in charge in dados over 12 mm thick bed of cement mortar (1 cement : 3 coarse sand)and jointing with filler of epoxy grouting etc co | 2.04 | Sqm |  |  |
| 12 | Providing and fixing wash basin including painting of fittings and brackets, cutting and making good the walls wherever required :400X400X250mm | 1.00 | Each |  |  |
| 13 | Providing and fixing C.P brass elbow action pillar cock : 15mm nominal bore | 0.00 | Each |  |  |
| 14 | Providing and fixing P.V.C. waste pipe for sink or wash basin including P.V.C. waste fittings complete. Flexible pipe, 32 mm dia | 0.00 | Each |  |  |
| 15 | Providing and fixing Chlorinated Polyvinyl Chloride (CPVC) pipes, having thermal stability for hot & cold water supply including all CPVC plain & brass threaded fittings i/c fixing the pipe with clamps at 1.00 m spacing. This includes jointing of pipes & Concealed work including cutting chases and making good the walls etc |  |  |  |  |
|  | 20 mm nominal outer dia .Pipes | 40.00 | metre |  |  |
|  | 25 mm nominal outer dia .Pipes | 35.00 | metre |  |  |
| 16 | Providing and fixing 600x450 mm beveled edge mirror of superior glass (of approved quality) complete with 6 mm thick hard board ground fixed to wooden cleats with C.P. brass screws and washers complete. | 0.00 | Each |  |  |
| 17 | Providing and fixing PTMT towel rail complete with brackets fixed to wooden cleats with CP brass screws with concealed fitting arrangement of approved quality and colour.600mm long towel rail with total length of 645mm, width 78mm and effective height of 88 mm, weighing not less than 190 gms | 0.00 | Each |  |  |
| 18 | Providing and fixing PTMT liquid soap container 109mm wide, 125mm high and 112mm distance from wall of standard shape with bracket of the same materials with snap fittings of approved quality and colour. weighing not less than 105 gms. | 0.00 | Each |  |  |
| 19 | Removing white or colour wash by scrapping and sand papering and preparing the surface smooth including necessary repairs to scratches etc. complete | 64.06 | Sqm |  |  |
| 20 | Providing and applying plaster of paris putty of 2 mm thickness over plastered surface to prepare the surface even and smooth complete | 41.58 | Sqm |  |  |
| 21 | Wall painting with plastic emulsion paint of approved brand and manufacture to give an even shade :One or more coats on old work. | 64.06 | Sqm |  |  |
| 22 | Providing and fixing of MS angles and frames to hold the granite slab including bolts | 70 | kg |  |  |
| TOTAL |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT