TENDER   DOCUMENT FOR CONTRUCTION OF TRANSFORMER ROOM FOR HLL’s UNIT,

GOVT. MEDICAL COLLEGE, KOZHIKODE

Tender No. HLL/CHO/HCS/PROJ/2015-16/003 dated 22-05-2015

HLL LIFECARE Ltd

CORPORATE AND REGD. OFFICE

HLL BHAVAN, POOJAPPURA

THIRUVANANTHAPURAM

0471-2354949, Ext. 331,325,326,289.

CONTENTS

| SL No. | ITEM | Page No. |
| --- | --- | --- |
| 1 | Notice Inviting Tender | 3 |
| 2 | Terms and Conditions | 4 - 6 |
| 3 | Annexure A - Schedule | 7 - 19 |

NOTICE INVITING TENDER

Sub: Construction of Transformer Room for HLL’s unit at Govt. Medical College Hospital, Kozhikode

Tender No. - HLL/CHO/HCS/PROJ/2015-16/003 dated 22-05-2015

HLL Lifecare Ltd, a Government of India Undertaking, invites sealed and super scribed quotation from interested contractors/firms having minimum two years of experience in the relevant field for the work at Govt. Medical College, Kozhikode

Publishing date of bid: 22nd May 2015.

Last date of submission (at the office of the Group Head (HCS), Corporate and Regd. Office, HLL Bhavan, Poojappura, Thiruvananthapuram): 2nd June 2015, 15.00 Hours.

Opening of bid: 2nd June 2015, 16.00 Hours at HLL Lifecare Ltd, Poojappura Thiruvananthapuram)

Interested applicants can participate in the bid.

HLL reserves the right to accept or reject any application without assigning any reason or incurring any liability whatsoever. Prospective bidders are advised to regularly scan through HLL web site as corrigendum/amendments etc., if any, will be notified on the HLL web site and separate advertisement will not be made for this regard

For HLL Lifecare Ltd.

ASSOCIATE VICE PRESIDENT

HEALTHCARE SERVICE DIVISION

HLL LIFECARE LTD

CORPORATE AND REGD. OFFICE

HLL BHAVAN, POOJAPPURA

THIRUVANANTHAPURAM

0471-2354949, EXT. 331,325,326,289.

GENERAL TERMS & CONDITIONS

- Eligibility criteria:

  - Bidders should have executed satisfactorily at least one job preferably of similar kind and contract should be valuing not less than five lakhs during the last one year in satisfactory manner.

  - Bidders should have minimum two years’ experience in the relevant field.

Documental proof for the above eligibility criteria should be submitted along with offer.

- The schedule of quantities (ANNEXURE-A) is appended. Bidders are advised to go through the entire general terms and conditions of this contract carefully.

- Earnest money of Rs.10000/-(Rupees Ten thousand only) in the form of a Demand Draft of a scheduled bank issued in favor of HLL Lifecare LTD, HLL Bhavan, Poojappura, Thiruvananthapuram, which should be placed in a separate sealed cover marked “Earnest Money” shall be submitted along with the tender. EMD of the unsuccessful bidders will be returned with in 15days from the date of issue of work order. The EMD of the successful bidder will be converted into security deposit. Security deposit will be released along with the final payment.

- The filled & duly signed tender (signature on all pages of the tender) should be submitted to Associate Vice President (HCS), Healthcare Service Division, HLL Lifecare Limited, HLL Bhavan, Poojappura, Thiruvananthapuram, PH- 0471-2354949 Ext. 331, 325, 326, 289 on or before 02-06-2015 by 15.00 Hrs. The application shall be clearly marked “Tender for construction of Transformer Room for HLL’s unit at Govt. Medical College Hospital, Kozhikode”. The tender will be opened at 16.00Hrs on the same day itself in the presence of interested bidders or their authorized representatives who choose to attend at the time of opening of tender.

- The work shall be completed within 30 days from the date of receipt of Work Order or LOI

- The tender for the work shall remain open for acceptance for a period of 90days from the date of opening of tender.

- The contractor shall visit the site before quoting the rates and clarifications if any required can be had from Centre Manger, Lifecare Centre Kozhikode. Mob 9400027933, 9400027932

- Rates quoted should be inclusive of all cost of materials, Tools/Equipments labor charges, conveyance to site, handling charges, loading and unloading charges, hiring charges, clearing of debris, statutory payments etc.

- The Management reserves the right to accept or reject the Tender either partially or fully without assigning any reason what so ever.

- The work should be carried out without causing any inconvenience to the public and shall ensure that no damages are caused to the existing site premises.

- During the execution of work the contractor or authorized representative should be present at site.

- All Materials, Equipment’s/ Tools required for the work should be arranged by the contractor and brought to site for the timely completion of the work.

- The materials used shall be as per specification and of good quality.

- The Contractor has to arrange necessary insurance coverage for the machine, workmen etc. deployed by him. He shall arrange all safety measures to protect his workmen and also the properties of HLL.

- Retention Money and Defect liability period: Retention money will be recovered @ 5% on each bills submitted by the contractor. The retention money will be released after the defect liability period of one year from the date of issue of Work Completion Certificate. The work site safety of all employees, their ESI, PF etc will have to be borne by the contractor.

- Taxes & Duties: The amounts quoted in the tender  shall be inclusive of all taxes and duties

- Penalty for delay: Penalty shall be imposed at 0.5% per week to a maximum of 7.5% of the contract value for any delay committed by the contractor in completing the work as per the order.

- Measurement & Payment terms: The method of measurement of completed work shall be in accordance with the standard measurement. Payment will be made on satisfactorily completion of work as per the order. Interim or running account bills shall be submitted by the contractor for the work executed on the basis of such recorded measurements in the format of HLL. All such interim payments shall be regarded as payment by way of advances against final payment only, and shall not preclude the requiring of bad, unsound and imperfect or unskilled work to be rejected, removed, taken away and reconstructed or re-erected. Any certificate given by the officer  relating to the work done or materials delivered forming part of such payment, may be modified, or corrected by any subsequent such certificate(s) or by the final certificate and shall not by itself be conclusive evident that any work or material so which it relates is /are in accordance with the contract and certificate. Any such interim payment,/any part  there of shall not in any respect conclude, determine or affect in any way powers of the engineer in charge under the contract or any of such payment s be treated as final settlement and adjustment of accounts or in any way vary or affect the contract.

- Payment Schedule: 50% of the bill value will be paid on submission of final bill submitted by the contractor.  The remaining 50% will be paid after issue of Work Completion certificate by Engineer in Charge/Officer In charge of HLL Lifecare Ltd. Payments will be made after effecting the statutory deductions of TDS, Work Contract Tax, Service Tax (reverse charge), etc .Retention money @ 5% will be deducted on each bill as per clause 15.,

- The Quantity shown in the schedule is an approximate estimated quantity. No rate revision will be entertained if the quantity increases/decreases due to the site condition while executing the work.

- Final payment shall be paid only after clearing the site as per direction of Engineer-in-charge/ Officer in charge.

- Settlement of Dispute: Arbitration shall not be the means of settlement of dispute or claims or   anything on account of this Contract. If any disputes and difference arising out of the contract are to be settled by a civil court at Thiruvananthapuram.

For HLL Lifecare Ltd.

ASSOCIATE VICE PRESIDENT

HEALTHCARE SERVICE DIVISION

HLL LIFECARE LTD

CORPORATE AND REGD. OFFICE

HLL BHAVAN, POOJAPPURA

THIRUVANANTHAPURAM

0471-2354949, EXT. 331,325,326

ANNEXURE- A

| SL No | Description | Unit | Quantity | Rate (Rs.) | Amount (Rs.) |
| --- | --- | --- | --- | --- | --- |
| CIVIL WORK |  |  |  |  |  |
|  | EARTH WORK |  |  |  |  |
| 1 | Earth work in excavation by mechanical means (Hydraulic Excavator )/ manual means in foundation trenches or drains (not exceeding 1.5 m in width or 10 sqm on plan) including dressing of sides and ramming of bottoms, lift upto 1.5 m, including getting out the excavated soil anddisposal of surplus excavated soil as directed, within a lead of 50 m. All kinds of soil. | Cum | 12 |  |  |
| 2 | Filling available excavated earth (excluding rock) in trenches, plinth, sides of foundations etc. in layers not exceeding 20cm in depth, consolidating each deposited layer by ramming and watering, lead up to 50 m and lift upto 1.5 m. | Cum | 10 |  |  |
| 3 | Supplying and filling in plinth with quarry dust under floors including watering, ramming, consolidating and dressing complete. | Sqm | 1.6 |  |  |
|  | PLAIN CEMENT CONCRETE |  |  |  |  |
| 4 | Cement concrete(Cast-in-situ) Providing and laying in position cement concrete of specified grade excluding the cost of centring and shuttering- All work upto plinth level 1:5:10 (1 cement : 5 coarse sand : 10 graded stone aggregate 40mm nominal size) |  |  |  |  |
| 5 | Providing and laying damp proof coarse with cement concrete 1:2:4 (1 cement : 2 coarse sand: 4 graded stone aggregate 20mm nominal size) including and applying a coat of residual petroleum bitumen of penetration of penetration 80/100 of approved quality using 1.7 kg per square metre on damp proof coarse after cleaning the surface with brushes and finally with a piece of cloth lightly soaked in kerosene oil 50mm thick | Sqm | 9 |  |  |
|  | REINFORCED CEMENT CONCRETE |  |  |  |  |
| 6 | Providing and laying in position specified grade of reinforced cementconcrete, excluding the cost of centering, shuttering, finishing and reinforcement - All work up to plinth level 1:1.5:3 (1 cement : 1.5 coarse sand : 3 graded stone aggregate 20 mm nominal size) | Cum | 2.5 |  |  |
| 7 | Centering and shuttering including strutting, propping etc. and removal ofform for : Foundations, footings, bases of columns, etc. for mass concrete | Sqm | 26 |  |  |
| 8 | Reinforced cement concrete work in beams, suspended floors, roofs having slope upto 15degree, landings, balconies, shelves, chajjas, lintels, bands, plain window sills, staircases and spiral stair cases upto floor V level excluding cost of centering, shuttering, finishing and excluding the cost of reinforcement in 1:1.5:3 (1 cement: 1.5 coarse sand : 3 graded stone aggregate 20mm nominal size) | Cum | 2.3 |  |  |
| 9 | Centering and shuttering including strutting, propping etc. and removal ofform for : Suspended floors, roofs, landings, balconies and access platform Stairs, (excluding landings) except spiral-staircases | Sqm | 25 |  |  |
| 10 | Reinforcement for RCC works including straightening, cutting, bending, placing in position and binding all complete. Thermo - Mechanically treated bars (Using Vizac, Tata or SAIL) | Kg | 605 |  |  |
|  | BRICK WORK / LATERITE WORK |  |  |  |  |
| 11 | Stone work with neatly dressed selected hard laterite stones of of good quality size 30X20X19 cm in foundation and plinth cement mortar 1:6 (1 cement : 6 course sand) including extra for super structure above plinth level and upto floor V level | Cum | 12.2 |  |  |
| 12 | Half Brick masonry with FPS bricks of class designation 75 in superstructure above plinth level upto floor V level in cement mortar 1:4 (1 cement: 4 coarse sand) | Sqm | 3 |  |  |
| 13 | Extra providing and placing in position 2 nos.6mm dia MS bars at every third course of half brick masonry (with F P S Bricks) | Sqm | 3 |  |  |
|  | STEEL WORK |  |  |  |  |
| 14 | Providing and fixing M.S grills of required pattern in frames of windows etc with M.S.flats,square or round bars etc all complete:- Fixed to openings/wooden frames with rawl plugs screws etc | Kg | 68 |  |  |
| 15 | Steel work in built up tubular ( round, square or rectangular hollow tubes etc.) trusses etc. including cutting, hoisting, fixing in position and applying a priming coat of approved steel primer, including welding and bolted with special shaped washers etc. complete.Hot finished welded type tubes | Kg | 906 |  |  |
| 16 | Supplying and fixing rolling shutters of approved make, made of required size M.S. laths, interlocked together through their entire length and jointed together at the end by end locks, mounted on specially designed pipe shaft with brackets, side guides and arrangements for inside and outside locking with push and pull operation complete, including the cost of providing and fixing necessary 27.5 cm long wire springs manufactured from high tensile steel wire of adequate strength conforming to IS: 4454 - part 1 and M.S. top cover of required thickness for rolling shutters. 80x0.90 mm M.S. laths with 0.90 mm thick top cover | Sqm | 6 |  |  |
|  | ALUMINIUM WORKS |  |  |  |  |
| 17 | Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixed with rawl plugs and screws or with fixing clips, or with expansion hold fasteners including necessary filling up of gaps at junctions, at top, bottom and sides with required PVC/ neoprene felt etc. Aluminium sections shall be smooth, rust free, straight, mitred and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing/panelling. C.P. brass/ stainless steel screws, all complete as per architectural drawings and the directions of Engineer-in-charge. (Glazing and panelling to be paid for separately):For fixed portion Powder coated aluminium (minimum thickness of powder coating 50 micron | Kg | 25.5 |  |  |
| 18 | Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixed with rawl plugs and screws or with fixing clips, or with expansion hold fasteners including necessary filling up of gaps at junctions, at top, bottom and sides with required PVC/ neoprene felt etc. Aluminium sections shall be smooth, rust free, straight, mitred and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing/panelling. C.P. brass/ stainless steel screws, all complete as per architectural drawings and the directions of Engineer-in-charge. (Glazing and panelling to be paid for separately):For shutters of doors, windows & ventilators including providing and fixing hinges / pivots and making provision for fixing of fittings wherever required including the cost of EPDM rubber / neoprene gasket required (Fittings shall be paid for separately).Powder coated aluminium (minimum thickness of powder coating 50 micron | Kg | 24.8 |  |  |
| 19 | Providing and fixing 100mm brass locks (best make of approved quality) for aluminium doors including necessary cutting and making good etc. complete. | Each | 1 |  |  |
| 20 | Providing and fixing glazing in aluminium door, window, ventilator shutters and partitions etc. with PVC/neoprene gasket etc. complete as per the architectural drawings and directions of engineer-in-charge. (Cost of aluminium snap beading shall be paid in basic item): With float glass panes of 5.5 mm thickness | Sqm | 0.9 |  |  |
| 21 | Providing and fixing glazing in aluminium door, window, ventilator shutters and partitions etc. with PVC/neoprene gasket etc. complete as per the architectural drawings and directions of engineer-in-charge. (Cost of aluminium snap beading shall be paid in basic item): With float glass panes of 4.0 mm thickness | Sqm | 4 |  |  |
| 22 | Providing and fixing12mm thick prelaminated particle board flat pressed three layer or graded wood particle board conforming to IS12823 Grade 1 type II, in panelling fixed in aluminium doors, windows shutters and partition frames with C.P brass/ stainless steel screws etc. complete as per architectural drawings and directions of engineer in charge Pre laminated particle board with decorative lamination on both sides | Sqm | 1.1 |  |  |
| 23 | Providing nad fixing aluminium hanging floor door stopper ISImarked anodised (anodic coating not less than grade AC 10 as per IS : 1868) transparant or dyed to required colour and shade with necessary screws etc. complete. Single rubber stopper | Each | 1 |  |  |
| 24 | providing and fixing aluminium tower bolts ISI marked anodised (anodic coating not less than grade AC 10 as per IS : 1868) transparent od dyed to required colour or shade with necessary screws etc. complete: 250 x 10 mm | Each | 2 |  |  |
| 25 | Providing and fixing window handles with necessary screws etc. complete- | Each | 6 |  |  |
| 26 | Providing and fixing bright finished stay-cum friction Hymax hinges with necessary screws etc. complete-- 250 mm (Heavy type) | Each | 12 |  |  |
|  | ROOFING WORKS |  |  |  |  |
| 27 | Providing corrugated G.S. sheet roofing including vertical / curved surface fixed with polymer coated J or L hooks, bolts and nuts 8 mm diameter with bitumen and G.I. limpet washers or with G.I. limpet washers filled with white lead, including a coat of approved steel primer and two coats of approved paint on overlapping of sheets complete (up to any pitch in horizontal/ vertical or curved surfaces), excluding the cost of purlins, rafters and trusses and including cutting to size and shape wherever required. | cum | 50.4 |  |  |
| 28 | Providing and fixing 15 cm wide,20cm overall semi-circular plain G.S. sheet gutter with iron brackets suitable size, bolts, nuts and washers etc., including making necessary connections with rain water pipes complete. |  |  |  |  |
|  | FLOORING WORKS |  |  |  |  |
| 29 | Providing and laying in position cement concrete of specified grade excluding 1:2:4 (1 cement : 2 coarse sand : 4 graded stone aggregate 20 mm nominal size) | Sqm | 1.4 |  |  |
| 30 | 12 mm cement plaster finished with a floating coat of neat cement of mix :1:3 (1 cement: 3 fine sand) | Sqm | 27.7 |  |  |
|  | FINISHING WORKS |  |  |  |  |
| 31 | 6mm cement plaster to ceiling of mix 1 : 3 (1 cement : 3 fine sand ) | Sqm | 32 |  |  |
| 32 | Extra for plastering on circular work not exceeding 6 m in radius:In two coats | Sqm | 10 |  |  |
| 33 | 12mm cement plaster on rough side of single or half brick wall of mix 1:4 (1 cement : 4 fine sand) | Sqm | 61 |  |  |
| 34 | 15 mm cement plaster on rough side of single or half brick wall of mix:1:4 (1 cement : 4 coarse sand) | Sqm | 72 |  |  |
| 35 | Wall painting with acrylic emulsion paint of approved brand and manufacture to give an even shade in two or more coats on new work, including finishing walls with Deluxe Multi surface paint system for interiors and exteriors using Primer as per manufacturers specifications: Two or more coats applied on walls @ 1.25 ltr/10 sqm over and including one coat of Special primer applied @ 0.75 ltr / 10 sqm | Sqm | 78 |  |  |
| 36 | Finishing walls with Acrylic Smooth exterior paint of required shade: —New work (Two or more coat applied @ 1.67 ltr/10 sqm over and including basecoat of water proofing cement paint applied @ 2.20 kg/ 10 sqm) | Sqm | 72 |  |  |
| 37 | Steel Work painting- Painting with synthetic enamel paint of approved brand and manufacture to give an even shade. Two or more coats on new work | Sqm | 17 |  |  |
|  | INTERNAL WATER SUPPLY & SANITORY INSTALLATIONS |  |  |  |  |
| 38 | Providing and jointing PVC110mm PVC pipes of (pressure 4 kg/cm2) for drainage line including testing of joints etc complete. (Rain water Down pipe) 110mm diameter (OD) | RM | 16.2 |  |  |
| 39 | Providing and jointing PVC110mm PVC bend of (pressure 4 kg/cm2) for drainage line including testing of joints etc complete. (Rain water Down pipe) 110mm diameter (OD) | Each | 12 |  |  |
| 40 | Providing and fixing unplasticised-P.V.C pipes clips of approved design to unplasticised-P.V.C rain water pipes by means of 50 x 50 x 50 mm hard wood plugs, screwed with M.S screws of required length including cutting brick work and fixing in cement mortar 1 : 4 (1 cement : 4 coarse sand) and making good the walls etc. complete 110mm | Each | 16 |  |  |
| ELECTRICAL WORK |  |  |  |  |  |
| 1 | Wiring for light point/ fan point/ exhaust fan point/ call bell point with 1.5 sq.mm FRLS PVC insulated copper conductor single core cable in surface / recessed medium class PVC conduit, with modular switch, modular plate, suitable GI box and earthing the point with 1.5 sq.mm. FRLS PVC insulated copper conductor single core cable etc. as required. |  |  |  |  |
| a | Group C | Point | 10 |  |  |
| 2 | Wiring for circuit/ submain wiring alongwith earth wire with the following sizes of FRLS PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required |  |  |  |  |
| a | 2x1.5 sq mm +1x1.5 sq. mm earth wire | Rm | 30 |  |  |
| b | 2x2.5 sq mm +1x 2. 5 sq.mm earth wire | Rm | 55 |  |  |
| 3 | Supplying and fixing stepped type electronic fan regulator on the existing modular plate switch box including connections but excluding modular plate etc. as required | Each | 1 |  |  |
| 4 | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. | Each | 2 |  |  |
| 5 | Supplying and fixing following size/ modules,GI box along with modular base, cover plate for modular switches in recess etc as required. |  |  |  |  |
| a | 1 or 2 module( 75 mmx75 mm) | Each | 1 |  |  |
| 6 | Supplying and fixing suitable size G. I box with modular plate and cover in front on surface or in recess, including providing and fixing 3 pin 5/6 amps modular socket outlet and 5/6 amps modular switch, connection,painting etc as required, (For light plugs to be used in non residential buildings }. | Each | 3 |  |  |
| 7 | Supplying and fixing suitable size G. I box with modular plate and cover in front on surface or in recess, including providing and fixing 6 pin 15/16& 5/6 amps modular socket outlet and 15/16 amps modular switch, connection,painting etc as required. | Each | 2 |  |  |
| 8 | Supplying and fixing 3 pin , 5 amps ceiling rose on the existing junction box / wooden block including connection etc as required. | Each | 10 |  |  |
| 9 | Installation testing and commissioning of pre- wired, fluroscent fitting / compact fluroscent fitting of all types, complete with all accessories and tube etc. directly on ceiling / wall, including connection with 1.5 sq mm FRLS PVC insulated,copper conductor, single core cable and earthing etc as required. | Each | 8 |  |  |
| 10 | Installation, testing and commissioning of ceiling fan, including wiring the down rods of standard length { Up to 30 Cm } with 1.5 sq mm FR PVC insulated, copper conductor, single core cable as required. | Each | 1 |  |  |
| 11 | Supplying and fixing extra down rod of 10 Cm length G. I pipe, 15 mm dia, heavy guage including painting etc. as required ( Note: More than 5 Cm length shall be rounded to the nearest 10 Cm and 5 Cm or less shall be ignored) | Each | 5 |  |  |
| 12 | ITC Exhaust fan up to 450 mm sweep: Installation of exhaust fan in the existing opening, including making good the damage, connection, testing, commissioning etc. as required. | Each | 1 |  |  |
| 13 | Supplying and fixing following way, horizontal type three pole and neutral, sheet steel, MCB distribution board, 415 volts, on surface/ recess, complete with tinned copper bus bar, neutral bus bar, earth bar, din bar, interconnections, powder painted including earthing etc. as required. (But without MCB/RCCB/Isolator) |  |  |  |  |
| a | 4 way ( 4+12 ), Double door | Each | 1 |  |  |
| 14 | Supplying and fixing 5 amps to 32 amps rating, 240/415 volts, "C" curve, miniature circuit breaker suitable for inductive load of following poles in the existing MCB DB complete with connections, testing and commissioning etc. as required. |  |  |  |  |
| a | Single pole | Each | 4 |  |  |
| b | Triple Pole and neutral | Each | 1 |  |  |
| 15 | Supplying and fixing single pole blanking plate in the existing MCB DB complete etc as required. | Each | 8 |  |  |
| 16 | Supplying and fixing following rating, four pole, 415 volts, residual current circuit breaker { RCCB }, having a sensitivity current up to 300 milli amperes in the existing MCB DB complete with connections, testing and commissioning etc. as required. |  |  |  |  |
| a | 25 amps | Each | 1 |  |  |
| 17 | Laying and fixing of one number PVC insulated and PVC sheathed / XLPE power cable of 1.1 KV grade of following size on wall surface as required. |  |  |  |  |
| a | Upto 35 sq. mm (clamped with 1mm thick saddle) | Rm | 25 |  |  |
| 18 | Supplying and making end termination with brass compression gland and aluminium lugs for following size of PVC insulated and PVC sheathed / XLPE aluminium conductor cable of 1.1 KV grade as required. |  |  |  |  |
| a | 4 C x 10 sqmm | Each | 2 |  |  |
| 19 | Supply of following size 1.1 KV grade XLPE insulated, PVC sheathed, armoured Aluminium conductor power cable conforming to IS 7098 (Part 1) amended up to date. |  |  |  |  |
| a | 4 C x 10 sqmm | Rm | 25 |  |  |
| 20 | Supplying and laying/fixing 12 SWG copper wire along with MV cales in ground/wall surface for conductor earth electrode, including soldering etc as required | Rm | 50 |  |  |
| 21 | Supply of 1X28 W compact energy saving industrial Fitting with CRCA sheet Housing suitable for 28 W T5 Lamp, 28 W T5 lamp, suitable ballast etc( Wipro WIF 90128 SG) or equi. | Each | 8 |  |  |
| 22 | Supply of ceiling fan 1200 mm sweep, double ball bearing, approved colour and quality with original down rod,suspension shackle,canopy etc with out regulator ( Make: Usha Technix flair star rated ) or equi. | Each | 1 |  |  |
| 23 | Supply of heavy duty exhaust fan 300 mm sweep, 900 rpm, metal blades of approved colour, for single phase 230 volt, 50Hz AC supply complete with motor, capacitor and all accessories as required complete ( Almonard or equi). | Each | 1 |  |  |
| GRAND TOTAL |  |  |  |  |  |

Place                                                                      Signature of the Bidder

Date                                                                       Name of the Bidder