30.11.2012

AMENDMENT-I

Name of Work: Supply, Installation, Testing and Commissioning of  Canteen/  Kitchen Equipments

- Interior and  Accessories table in tender page no 68 to 70 in the tender document is modified as

|  |  |  |  | Interior and Accessories (Table) - Kitchen |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Room No | Description | Flooring | Walls | Drains | Exhaust | Ventilation | Power Isolator | Power Sockets | Lighting | LPG | Extras |
| 1 | Steam cooking area | 2 feet x 2 feet Vitrified tiles Matt finish | Joint free glazed tiles 600x 300x 8mm –full height | 6" drain with cover, inspection trap | Exhaust Hood | positive pressure ventilation | Yes | 15 A x 8 | Fluroscent with louvres | Yes |  |
| 2 | Vegetarian Kitchen-1 | 2 feet x 2 feet Vitrified tiles Matt finish | Joint free glazed tiles 600x 300x 8mm –full height | 6" drain with cover, inspection trap; Grease trap | Exhaust Hood | positive pressure ventilation | Yes | 15A x6 | Fluroscent with louvres | Yes |  |
| 3 | Vegetarian Kitchen-2 | 2 feet x 2 feet Vitrified tiles Matt finish | Joint free glazed tiles 600x 300x 8mm –full height | 6" drain with cover, inspection trap; Grease trap | Exhaust Hood | positive pressure ventilation | Yes | 15A x 6 | Fluroscent with louvres | Yes |  |
| 4 | Pot wash area | 2 feet x 2 feet Vitrified tiles Matt finish | Joint free glazed tiles 600x 300x 8mm –full height | 6" drain with inspection trap; | Exhaust Fans | Wall/Ceiling fans | Yes | 15A x | Fluroscent/CFL lamps |  |  |
| 5 | Dish wash area | 2 feet x 2 feet Vitrified tiles Matt finish. | Joint free glazed tiles 600x 300x 8mm –full height | 6" drain with inspection trap; | Exhaust Fans | Ceiling fans | Yes | 15A X 4 | Concealed CFL bulk head fitting |  |  |
| 6 | Refrigerator room | 2 feet x 2 feet Vitrified tiles Matt finish | Joint free glazed tiles 600x 300x 8mm –full height | 2" drains |  | Ceiling fans | yes | 15A X 5 | Concealed CFL bulk head fitting |  |  |
| 7 | Dry store | 2 feet x 2 feet Vitrified tiles Matt finish | Joint free glazed tiles 600x 300x 8mm –full height | 4" drain with trap; | Yes | Yes |  | 15A X 4 | Concealed CFL Lighting |  |  |
| 8 | Storage area | 2 feet x 2 feet Vitrified tiles Matt finish | Joint free glazed tiles 600x 300x 8mm –full height | 4" drain with trap; | Yes | Yes |  | 15 Ax 5 | Concealed CFL lighting |  |  |
| 9 | Wet processing area | 2 feet x 2 feet Vitrified tiles Matt finish | Joint free glazed tiles 600x 300x 8mm –full height | 4" drain with trap; | Yes | Yes |  | 15 Ax 6 | Flourescent fitting |  |  |
| 10 | Dietary service counter | 2 feet x 2 feet Vitrified tiles Matt finish | Joint free glazed tiles 600x 300x 8mm –full height | 4" drain with trap | Yes | Yes, Fans |  | 15 Ax 6 | Fluorescent louvres | Yes |  |
| 11 | Vending Counter | 2 feet x 2 feet Vitrified tiles Matt finish | Decorative plastic emulsion painting | 4" drain with trap | Yes | Yes |  | 15 Ax 6 | Concealed CFL | Yes | Counter top should be granite |
| 12 | Dining Area | 2 feet x 2 feet Vitrified tiles Matt finish | Decorative plastic emulsion painting |  | Yes | Fans |  | 15 Ax 2 | Concealed decorative fittings |  | Cash reports/ Computer |
| 13 | Waiting area cum Office | 2 feet x 2 feet Vitrified tiles | Joint free glazed tiles 600x 300x 8mm –full height) |  |  | Fans |  | 15 Ax 3 |  |  |  |
| 14 | Garbage Storage Area | 2 feet x 2 feet Vitrified tiles Matt finish | Joint free glazed tiles 600x 300x 8mm –full height | Yes | Yes |  |  | 15 Ax 3 | With mobile garbage trolleys 200 l |  |  |
| 15 | Gas Bank | Bituminous hard flooring | Plastered walls |  | Yes |  | Outside | Outside | 12 cylinder LPG manifold |  |  |

- “Exterior wall finishes with aluminum facade-cladded” in tender page no 71 should modified as “Exterior wall of whole building should be coated with weather proof dulux or asian paint.”

3) Another amendment under turnkey job in tender page no 71 is “Internal walls of ground floor should be double coated with white plastic emulsion wherever exposed”.

4) “The dining area should be air conditioned or well ventilated. Quote optional price for high quality split A/C system for the dining area (This is in addition to the ceiling fans provided)” in tender page 71 is modified as “The dining area should be well ventilated with fans”

5) “Provision of suitable cable TV display and music are the responsibility of the vendor” in tender page No 72 is deleted.

6) “Roof should be modified to locate water storage tanks suitably” in tender page 72 is modified as “Roof should be modified to locate two water storage tanks of 2000 liters”

7) Another amendment under turnkey job in tender page no 72 is “Upstair entry should be blocked with suitable lockable doors. A separate stair of 90 cm width should be created to first floor on the rear side of the building”

8) Tiles shall be from approved make such as Johnson/ Kajaria/ Somany/ Nitco. Premium quality tiles to be used.

9) Water supply items and electrical items should be that of approved brands.

10) The last date of submission of tender: 10.12.2012 at 3:00 PM

11) Date and time of tender opening: 10.12.2012 at 3:30 PM