TENDER   DOCUMENT FOR INTERIOR AND FURNISHING WORKS

AT HINDLABS, AIIMS-Raipur

HLL LIFECARE LTD

HINDLABS,

Trauma Building, AIIMS-Raipur,

Tatibandh, Raipur, Chhattishgarh-492099

Mob: 7415333044, 7205870356

CONTENTS

| Sl. No. | Item | Page No. |
| --- | --- | --- |
| 01. | Notice Inviting Tender | 3 |
| 02. | Terms and Condition | 4 - 7 |
| 03. | Annexure A – Schedule | 8 - 13 |

NOTICE INVITING TENDER

Tender No.: HLL/HIND/R/CG/2013-14/Tend-02			Dated 	09/10/2013

Sub: Furnishing and Interior works for Hind labs, AIIMS-Raipur, Tatibandh, Raipur

HLL Lifecare Ltd, a Government of India Undertaking, invites sealed and superscribed quotation from interested contractors having minimum One year of experience in the relevant field for the following works at HINDLABS (PROPOSED DIAGNOSTIC LABAROTARY) at AIIMS-Raipur, Tatibandh, Raipur.

The estimated value of the present purchase is around Rs.20,00,000.00 (Rupees Twenty Lakhs only).

Publishing date of the bid	:

Last Date of submission	:

Address:				Centre Manager

HLL LIFECARE LTD

Flat No: 1A, 2nd Block

Singapore City, Behind R.K Mall

Mahoba Bazar, Raipur- 492001

Mob: 7415333044, 7205870356

Opening of Bid		:

HLL LIFECARE LTD

Flat No: 1A, 2nd Block

Singapore City, Behind R.K. Mall

Mahoba Bazar, Raipur- 492001

Mob: 7415333044, 7205870356

Interested applicants may download the tender from company website or collect the tender documents from the Centre Manager, HLL Life Care Limited, Flat No. 1A, 2nd Block, Singapore City, Behind R. K. Mall, Mahoba Bazar, Raipur 492 001, from 09.10.2013 till 15.10.2013 between 9.30 AM to 4.30 PM.

Interested applicants can participate in the bid.

HLL reserves the right to accept or reject any application without assigning any reason or incurring any liability whatsoever.  Prospective bidders are advised to regularly scan through HLL’s website as corrigendum / amendments etc., if any, will be notified on the HLL’s website and separate advertisement will not be made in this regard.

GENERAL TERMS & CONDITIONS

- The estimated cost of the works is Rs.20,00,000.00 (Rupees Twenty Lakhs Only)

- Minimum Eligibility criteria:

Experience of having successfully completed works during the last 1 year ending last day of the month previous to the one in which applications are invited:-

  - Two similar completed works each consist not less than the amount equal to 50% of the estimated cost of the work for which the bid is submitted.

  - One similar completed work consisting not less than the amount equal to 60% of the estimated cost of the work for which the bid is submitted.

  - Documentary proof for the above eligibility criteria should be submitted along with the offer.

- The schedule of works (ANNEXURE - A) is appended.  Tenderers are advised to go through the entire general terms and conditions of this contract carefully.

- Earnest money of Rs.10,000/- (Rupees ten thousand only) in the form of a Demand Draft of a scheduled bank issued in favor of HLL Life care Limited, Bangalore, which should be placed in a separate sealed cover marked “Earnest money” shall be submitted along with the tender.

- The filled & duly signed sealed tender should be addressed to Centre Manager (HCS-AIIMS Raipur) and should be submitted at following address

HLL LIFECARE LTD

Flat No:1A, 2nd Block

Singapore City, Behind R.K Mall

Mahoba Bazar, Raipur- 492001

Mob: 7415333044, 7205870356

on or before 14.00Hrs on 16.10.2013.  The tender will be opened at 14.30Hrs on 16.10.2013 in the presence tender opening committee and interested bidders or their authorized representatives who choose to attend at the time of opening of tender.

- The work shall be completed within 30days (1 month) from the date of receipt of Work Order or LOA (Letter of acceptance).

- The tender for the work shall remain open for acceptance for a period of 7days from the date of opening of tender.

- The contractor shall visit the site before quoting the rates and clarifications if any required can be had from the undersigned before submitting the tenders.

Centre manager

HLL LIFECARE LTD

HINDLABS, Trauma Building

AIIMS –Raipur, G.E Road,

Tatibandh, Raipur-492099

Mob: 7415333044, 7205870356

- Rates quoted should be inclusive of all taxes, cost of materials, Tools/Equipment’s, labor charges, conveyance to site, handling charges, loading and unloading charges, hiring charges,  statutory payments etc complete.

- The Management reserves the right to accept or reject the Tender either partially or fully without assigning any reason what so ever.

- The work should be carried out without causing any inconvenience to the public and shall ensure that no damages are caused to the existing site premises.

- During the execution of work the contractor or authorized representative should be present at site.

- All Materials, Equipment’s/ Tools required for the work should be arranged by the contractor and brought to site for the timely completion of the work.

- The materials used shall be as per specification and of good quality.

- The Contractor has to arrange necessary insurance coverage for the machine, workmen etc. deployed by him. He shall arrange all safety measures to protect his workmen and also the properties of HLL.

- Security Deposit and Defect liability period: Shortlisted bidder is required to furnish Security Deposit @5% of the tendered amount for execution of the work along with the EMD submitted.  The security deposit will be released after the defect liability period of one year.  Statutory deductions like Income tax (2%), work contract tax (4%) etc will be deducted from the bill as per the rules of the company.  The work site safety of all employees, their ESI, PF etc will have to be borne by the contractor.

- Penalty for delay: Penalty shall be imposed at 0.5% per week to a maximum of 7.5% of the contract value for any delay committed by the contractor in completing the work as per the order.

- Measurement & Payment terms: The method of measurement of completed work shall be in accordance with the standard measurement. Payment will be made on satisfactorily completion of work as per the order. The method of measurement of completed work shall be in accordance with the standard measurement. Interim or running account bills shall be submitted by the contractor for the work executed on the basis of such recorded measurements in the format of HLL. All such interim payments shall be regarded as payment by way of advances against final payment only, and shall not preclude the requiring of bad, unsound and imperfect of unskilled work to be rejected, removed, taken away and reconstructed or re-erected. Any certificate given by the engineer in charge relating to the work done or materials delivered forming part of such payment, may be modified, or corrected by any subsequent such certificate(s) or by the final certificate and shall not by itself be conclusive evident that any work or material so which it relates is /are in accordance with the contract and certificate. Any such interim payment,/any part  there of shall not in any respect conclude, determine or affect in any way powers of the engineer in charge under the contract or any of such payment s be treated as final settlement and adjustment of accounts or in any way vary or affect the contract.

- Payment Schedule:  20% of the total work order amount will be released on supply of materials on the site with subject to the HLL officers verification of the items, 40% will be paid subjected to the completion of work of the technical area and the remaining percentage of amount will be paid after the deductions shown in Clause 16.

- The Quantity shown in the schedule is an approximate estimated quantity.  No extra rate will be entertained if the quantity exceeds due to the site condition while executing the work.

- Final payment shall be paid only after clearing the site as per direction of Engineer-in-charge/ officer in charge.

- Settlement of Dispute: Arbitration shall not be the means of settlement of dispute or claims or   anything on account of this Contract. If any disputes and difference arising out of the contract are to be settled by a civil court at Thiruvananthapuram.

For HLL Lifecare Limited

SVP & Group Head (HCS)

| ANNEXURE - A |  |  |  |
| --- | --- | --- | --- |
| Interior and Furnishing work at HINDLABS, AIIMS-Raipur |  |  |  |
| SN | ITEMS | UNIT | QTY |
| 1 | FULL HT. PARTITIONS |  |  |
|  | Providing fabrication and fixing of flush type partition |  |  |
|  | made of 18mm plywood frame work, 6mm thk commercial |  |  |
|  | plywood of ISI brand ply make and 6mm thk etcthing |  |  |
|  | glass. The horiontal and vertical members of frame of plywood |  |  |
|  | (section 65mm x50mm) muts be farmed 600mmx600mm c\c and 6 |  |  |
|  | mm thk plywood must be fixed on both sides of the formed grid up to |  |  |
|  | 900mm ht. from floor , the galss of modi, asahi or saint Gobain make |  |  |
|  | between 900mm & 2100mm, and 6mm thk plywood,on both sides of |  |  |
|  | the wooden grid, above 2100mm upto the celling. Skriting must be |  |  |
|  | fixed on both sides of the partition. The exposed |  |  |
|  | surfaces of plywood must be covered with 1.0mm thk laminate of |  |  |
|  | good quality branded ISI ISO certified brand |  |  |
|  | The glass must be held in position with 25mm decorative |  |  |
|  | quarter bead of teak wood on sides. All the exposed teak wood must |  |  |
|  | be finished with 3 coats of melamine polish. |  |  |
|  | LAB |  |  |
|  | 16'6"(L) X 10'0" (H) | SQFT | 165 |
|  | 3'0"(L) X 10'0" (H) x4 | SQFT | 120 |
|  | 3'0"(L) X 3'0" (H) x2 | SQFT | 18 |
|  | 7'0"(L) X 10'0" (H) | SQFT | 70 |
|  | OFFICE GF |  |  |
|  | 25'0"(L) X 8'0" (H) |  |  |
|  | 3'0"(L) X 1'0" (H) x 2 | SQFT | 200 |
|  |  | SQFT | 6 |
|  | OFFICE FF |  |  |
|  | 7'0"(L) X 10'0" (H) | SQFT | 70 |
|  | 3'0"(L) X 3'0" (H) | SQFT | 9 |
| 2 | LOW HT. PARTITIONS |  |  |
|  | Providing fabrication and fixing of low ht.partition |  |  |
|  | made of 18mm ply frame work, 6mm thk commercial |  |  |
|  | plywood of ISI Ply make and 8mm thk etcthing |  |  |
|  | glass. The horiontal and vertical members of frame of |  |  |
|  | 18 mm ply(section 65mm x50mm) muts be farmed 600mmx600mm c\c and |  |  |
|  | 6mm thk plywood must be fixed on both sides of the formed grid up to |  |  |
|  | 900mm ht. from floor , the galss of modi, asahi or saint Gobain make |  |  |
|  | fixed with D-brackets and 6mm thk plywood,on both sides of |  |  |
|  | the wooden grid, Skriting must be fixed on both sides of the partition. |  |  |
|  | The exposed surfaces of plywood must be covered with 1.0mm thk laminate |  |  |
|  | good quality branded ISI ISO certified brand make |  |  |
|  | The glass must be held in position with 25mm decorative |  |  |
|  | quarter bead of teak wood on sides. All the exposed teak wood must |  |  |
|  | be finished with 3 coats of melamine polish. |  |  |
|  | LAB |  |  |
|  | 6'0" (L) X 4'6" (H) x2 |  |  |
|  |  | SQFT | 54 |
|  | OFFICE FF |  |  |
|  | 7'0" (L) X 4'6" (H) | SQFT | 31.5 |
| 3 | SWING DOORS |  |  |
|  | Providing & fixing 40 mm thk flush doors of commercial ISI make |  |  |
|  | The exposed surfaces of flush door must be covered with 1.0mm thk |  |  |
|  | laminate of good quality branded ISI ISO certified brand and 6mm glass |  |  |
|  | of modi,asahi or saint gobin make The glass must be held in position |  |  |
|  | With 25mm decorative quarter bead of teak wood on sides. All the exposed teak |  |  |
|  | wood must be finished with 3 coats of melamine polish. The flush door be |  |  |
|  | provided with hydraulic door closer, tower bolt ,mortise lock door stopper etc. |  |  |
|  | LAB |  |  |
|  | 3'0" X 7'0" x2 | SQFT | 21 |
|  | OFFICE GF |  |  |
|  | 3'0"(L) X7'0" (H) X 2 | SQFT | 42 |
|  | OFFICE FF |  |  |
|  | 3'0"(L) X7'0" (H) | SQFT | 21 |
| 4 | SLIDING DOORS |  |  |
|  | Providing & fixing 40 mm thk sliding flush doors of commercial ISI make |  |  |
|  | The exposed surfaces of flush door must be covered with 1.0mm thk |  |  |
|  | laminate of good quality branded ISI ISO certified brand and 6mm glass |  |  |
|  | of modi,asahi or saint gobin make The glass must be held in position |  |  |
|  | with 25mm decorative quarter bead of teak wood on sides. All the exposed teak |  |  |
|  | wood must be finished with 3 coats of melamine polish. The flush door be |  |  |
|  | provided with sliding fitting, tower bolt ,mortise lock door stopper etc. |  |  |
|  | LAB |  |  |
|  | 2'6" X 7'0" | SQFT | 17.5 |
| 5 | OFFICE TABLES |  |  |
|  | Proviing and fabrication of table as per design, |  |  |
|  | having table body of 18 mm, 12 mm & 6 mm thk commercial plywood |  |  |
|  | of ISI make and table top &shutters of 19 mm thk block board of |  |  |
|  | ISI make, drawer unit with one drawer |  |  |
|  | mounted on teliscopic drawer slider of Ebco make and shuttered |  |  |
|  | shelf under it, Keyboard tray fixed under the table top with telescopic |  |  |
|  | keyboard slider of Ebco make foot rest and CPU carrier (on the foot |  |  |
|  | rest level) with 19 mm thk block board the table top and all other |  |  |
|  | exposed surfaces covered with, 1 mm think Natural Teak laminate of |  |  |
|  | good quality branded ISI ISO certified brand, all inside/hidden surfaces |  |  |
|  | finished with one coat of primer and three coats of synthetic enamel |  |  |
|  | paint of berger Nerolac. Asion Paints all the edges and corners of |  |  |
|  | ply & block board protected with 6 mm thk plain teak wood bead all the |  |  |
|  | teak wood finished with 3 coats of melaminne polish, locks and |  |  |
|  | handles fixed in drawers &shutter, complete work. |  |  |
|  | LAB |  |  |
|  | 5'6" (L) X 2'0" (w) X 2'6"(H) x 4 | NO. | 4 |
|  | OFFICE GF |  |  |
|  | 3'6" (L) X 1'6" (w) X 2'6"(H) | NO. | 1 |
|  | OFFICE FF |  |  |
|  | 5'6" (L) X 2'0" (w) X 2'6"(H) x 2 | NO. | 2 |
|  | 7'6" (L) X 2'0" (w) X 2'6"(H) X2 | NO. | 2 |
| 6 | RECEPTION TABLE |  |  |
|  | Proviing and fabrication of table as per design, of 750 mm high & 600 mm |  |  |
|  | deep having table body of 18 mm, 12 mm & 6 mm thk commercial plywood |  |  |
|  | of ISI make and table top &shutters of 19 mm thk block board of |  |  |
|  | ISI make, drawer unit with 4 ROW of one drawer |  |  |
|  | mounted on teliscopic drawer slider of Ebco make and shuttered |  |  |
|  | shelf under it, Keyboard tray fixed under the table top with telescopic |  |  |
|  | keyboard slider of Ebco make foot rest and CPU carrier (on the foot |  |  |
|  | rest level) with 19 mm thk block board the table top and all other |  |  |
|  | exposed surfaces covered with, 1 mm think Natural Teak laminate of |  |  |
|  | good quality branded ISI ISO certified brand, all inside/hidden surfaces |  |  |
|  | finished with one coat of primer and three coats of synthetic enamel |  |  |
|  | paint of berger Nerolac. Asion Paints all the edges and corners of |  |  |
|  | ply & block board protected with 6 mm thk plain teak wood bead all the |  |  |
|  | teak wood finished with 3 coats of melaminne polish, locks and |  |  |
|  | handles fixed in drawers &shutter, complete work. |  |  |
|  | LAB |  |  |
|  | 22'0" (L) X 2'0" (W) X 4'0" (H) | NOS | 1 |
| 7 | SIDE TABLE |  |  |
|  | providing and fabrication of side tables as per design, with table body of |  |  |
|  | 18 mm,12 mm & thk commcercial plywood of ISI ISO make |  |  |
|  | and the table top & shutters of 19 mm thk block board |  |  |
|  | of ISI ISO certified make darwers mounted on the teliscopic |  |  |
|  | darwer slider of Ebco make the table top and all other exposed |  |  |
|  | surfaces covered with 1 mm think Natural Teak laminate of good quality |  |  |
|  | branded ISI ISO certified brand all inside/hidding surfaces finished with |  |  |
|  | one coat of primer and three coats of synthetic enamel paint of berger |  |  |
|  | Nerolac Asian paints make. All the edges and corners of plywood & |  |  |
|  | block borad protected with 6 mm thk plain teak wood dead all exposed |  |  |
|  | teak wood finised with 3 coats of melamine polish,locks and handles |  |  |
|  | must be fixed in drawers, complete work. |  |  |
|  | LAB |  |  |
|  | 3'0" (L) X 1'6" (W) X 2'6" (H) | NOS | 4 |
|  | OFFICE FF |  |  |
|  | 3'0"(L) X 1'6"(W) X 2'6" (H) | NO. | 2 |
| 8 | BILLING COUNTER |  |  |
|  | Proviing and fabrication of table as per design, of 750 mm high & 600 mm |  |  |
|  | deep having table body of 18 mm, 12 mm & 6 mm thk commercial plywood |  |  |
|  | of ISI make and table top &shutters of 19 mm thk block board of |  |  |
|  | ISI make, drawer unit with 3 units of one drawer |  |  |
|  | mounted on teliscopic drawer slider of Ebco make and shuttered |  |  |
|  | shelf under it, Keyboard tray fixed under the table top with telescopic |  |  |
|  | keyboard slider of Ebco make foot rest and CPU carrier (on the foot |  |  |
|  | rest level) with 19 mm thk block board the table top and all other |  |  |
|  | exposed surfaces covered with, 1 mm think Natural Teak laminate of |  |  |
|  | good quality branded ISI ISO certified brand, all inside/hidden surfaces |  |  |
|  | finished with one coat of primer and three coats of synthetic enamel |  |  |
|  | paint of berger Nerolac. Asion Paints all the edges and corners of |  |  |
|  | ply & block board protected with 6 mm thk plain teak wood bead all the |  |  |
|  | teak wood finished with 3 coats of melaminne polish, locks and |  |  |
|  | handles fixed in drawers &shutter, complete work. |  |  |
|  | OFFICE GF |  |  |
|  | 10'6" (L) X 2'0" (W) X 4'0" (H) | NOS | 1 |
| 9 | FILE CABINET |  |  |
|  | Providing and fabrication of 1'6" deep file/storage |  |  |
|  | cabinet with cabinet body of 18 mm,12 mm & 6 mm thk commercial |  |  |
|  | plywood of ISI make,with one row of drawers mounted |  |  |
|  | on the teliscopic drawer slider of Ebco make and shuttered shelf under |  |  |
|  | the drawer, all the external exposed surfaces of the cabinet covered |  |  |
|  | with 1.0 mm thk laminate ofgood quality branded ISI ISO certified brand, |  |  |
|  | all the internal/hidden surfaces finished with one coat of primer and three |  |  |
|  | coats of synthetic enamel paint of Berger, Nerolac, Asian paints make |  |  |
|  | all the edges/corners of plywood protected. With 6 mm thk plain teak |  |  |
|  | wood bead.and all the exposed teak wood surfaces must be finished |  |  |
|  | with three coats of melamine polish, including fixing lock and handle |  |  |
|  | on shutters and drawers complete work.(front side to be measured) |  |  |
|  | LAB |  |  |
|  | 4'0" (L) X 7'0" (H) | SQFT | 28 |
|  | 3'6" (L) X 7'0" (H) | SQFT | 24.5 |
|  | RECEPTION |  |  |
|  | 8'0" (L) X 3'0" (H) | SQFT | 24 |
|  | OFFICE GF |  |  |
|  | 7'0" (L) X 7'0" (H) | SQFT | 49 |
|  | 3'6" (L) X 7'0" (H) X2 | SQFT | 49 |
|  | OFFICE FF |  |  |
|  | 4'0" (L) X 2'6" (H) | SQFT | 10 |
|  | 3'6" (L) X 7'0" (H) X 10 | SQFT | 245 |
|  | PANTRY |  |  |
|  | 6'0" (L) X 3'0" (H) | SQFT | 18 |
| a | 18 mm table rubbed granite in dark color (black,grey) |  |  |
|  | with edges moulded complete with fitting etc. | SQFT | 12 |
|  | 6'0" (L) X 2'0" (W) |  |  |
| 10 | WALL MOUNTED OVERHEAD FILE CABINET |  |  |
|  | Providing and fabrication 13" deep file/storage |  |  |
|  | wall mounted cabinet with cabinet body of 18 mm,12 mm & 6 mm thk |  |  |
|  | commercial plywood of ISI make ,with one row of drawers mounted |  |  |
|  | on the teliscopic drawer slider of Ebco make and shuttered shelf under |  |  |
|  | the drawer, all the external exposed surfaces of the cabinet covered |  |  |
|  | with 1.0 mm thk laminate of good quality branded ISI ISO certified brand, |  |  |
|  | all the internal/hidden surfaces finished with one coat of primer and three |  |  |
|  | coats of synthetic enamel paint of Berger, Nerolac, Asian paints make |  |  |
|  | all the edges/corners of plywood protected. With 6 mm thk plain teak |  |  |
|  | wood bead.and all the exposed teak wood surfaces must be finished |  |  |
|  | with three coats of melamine polish, including fixing lock and handle |  |  |
|  | on shutters and drawers complete work.(front side to be measured) |  |  |
|  | LAB |  |  |
|  | 95'0" (L) X 2'6" (H) | SQFT | 237.5 |
|  | OFFICE FF |  |  |
|  | 7'6" (L) X 2'6" (H) X 2 | SQFT | 37.5 |
| 11 | CABINATE UNDER PLATFORM |  |  |
| A | Providing and fabrication front shutters with frame under platform |  |  |
|  | with shutter & frame of 18 mm, thk commercial |  |  |
|  | plywood of ISI make, |  |  |
|  | all the external exposed surfaces of the cabinet covered |  |  |
|  | with 1.0 mm thk laminate of good quality branded ISI ISO certified brand, |  |  |
|  | all the internal/hidden surfaces finished with one coat of primer and three |  |  |
|  | coats of synthetic enamel paint of Berger, Nerolac, Asian paints make |  |  |
|  | all the edges/corners of plywood protected. With 6 mm thk plain teak |  |  |
|  | wood bead.and all the exposed teak wood surfaces must be finished |  |  |
|  | with three coats of melamine polish, including fixing magnet and handle |  |  |
|  | on shutters complete work. |  |  |
|  | LAB |  |  |
|  | 132'0" (L) X 2'3" (H) | SQFT | 297 |
| B | DRAWER CELL |  |  |
|  | Providing and fabrication 2'0" WIDE & 20" deep file/storage drawer unit |  |  |
|  | cabinet with cabinet body of 18 mm,12 mm & 6 mm thk |  |  |
|  | commercial plywood of ISI make ,with one row of drawers mounted |  |  |
|  | on the teliscopic drawer slider of Ebco make and shuttered shelf under |  |  |
|  | the drawer, all the external exposed surfaces of the cabinet covered |  |  |
|  | with 1.0 mm thk laminate of good quality branded ISI ISO certified brand, |  |  |
|  | all the internal/hidden surfaces finished with one coat of primer and three |  |  |
|  | coats of synthetic enamel paint of Berger, Nerolac, Asian paints make |  |  |
|  | all the edges/corners of plywood protected. With 6 mm thk plain teak |  |  |
|  | wood bead.and all the exposed teak wood surfaces must be finished |  |  |
|  | with three coats of melamine polish, including fixing lock and handle |  |  |
|  | on shutters and drawers complete work. | NOS | 11 |
|  | 2'0" (W) X 2'3" (H) X 11NOS |  |  |
| 12 | CONFERENCE TABLE |  |  |
|  | Providing 6 seater table of stainless steel/ |  |  |
|  | wrought iron with 12mm glass top complete with |  |  |
|  | paint/polish & hardware etc. With stainless steel chairs |  |  |
|  | Size -5'6"(L) X 2'9"(W) X 2'6" (H) | nos | 1 |
| 13 | M S STORAGE RACKS |  |  |
|  | Providing & fixing MS angled storage racks of Godrej or equivalent |  |  |
|  | with adjustable ms sheet shelves complete with fixing, powder |  |  |
|  | coating etc. |  |  |
|  | Size - 3'0" x 7'0" x 3nos |  |  |
|  |  | nos | 3 |
| 14 | CURTAIN |  |  |
|  | P/f Window Curtain made of good quality fabric |  |  |
|  | with powder coated curtain rod & brackets and |  |  |
|  | finished with Ornamental ring complete |  |  |
|  | with hardware etc.complete. | SQFT | 77 |
|  | LAB |  |  |
|  | size - 11'0" X 7'0" |  |  |
| 15 | ROLLER BLINDS |  |  |
|  | Providing & fixing cord roller vertical roller blinds of Vista , Marvel or |  |  |
|  | equivalent Complete with channel,roller,cord and good quality fabric | SQFT | 415 |
| 16 | Tripod Stool |  |  |
|  | Supplying and fitting fixing of Patient checkup tripod stool of stainless |  |  |
|  | steel of qulity tc including all cost of labour, materials, installation |  |  |
|  | transportation, T & P etc complete as per the direction of |  |  |
|  | Engineer-in-charge | nos | 22 |
| 17 | Low Back office Chair |  |  |
|  | Supplying and fitting fixing of revolving and tilting low back chairs of make |  |  |
|  | (Godrej/methodex/Featherlite/EEZY) of approved colour (seat & back) fabrics |  |  |
|  | including all cost of labour, materials, installation, transportation, |  |  |
|  | T & P etc complete as per the direction of Engineer-in-charge. | nos | 25 |
| 18 | Medium Back Office Chairs |  |  |
|  | Supplying and fitting fixing of revolving and tilting high back chairs |  |  |
|  | of make (Godrej/methodex/Featherlite/EEZY) of approved colour (seat & back) |  |  |
|  | fabrics including all cost of labour, materials, installation, transportation |  |  |
|  | T & P etc complete as per the direction of Engineer-in-charge. | nos | 6 |
| 19 | Phlebotomy Chair |  |  |
|  | Supplying and fitting fixing of Phlebotomy chairs |  |  |
|  | of make (any reputed brand) of approved colour (seat & back) |  |  |
|  | fabrics including all cost of labour, materials, installation, transportation, |  |  |
|  | taxes, T & P etc complete as per the direction of Engineer-in-charge. | nos | 4 |

Place								Signature of the Bidder