TENDER   DOCUMENT FOR CIVIL, ELECTRICAL & INTERIOR WORKS AT LIFECARE CENTRE,

REGIONAL INSTITUTE OF OPHTHALMOLOGY (RIO), TRIVANDRUM

Tender No. HLL/CHO/HCS/PROJ/2014-15/002 dated 05-08-2014

HLL LIFECARE Ltd

CORPORATE AND REGD. OFFICE

HLL BHAVAN, POOJAPPURA

THIRUVANANTHAPURAM

0471-2354949, Ext. 331,325,326,289.

CONTENTS

| SL No. | ITEM | Page No. |
| --- | --- | --- |
| 1 | Notice Inviting Tender | 3 |
| 2 | Terms and Conditions | 4 - 6 |
| 3 | Annexure A - Schedule | 7 - 18 |

NOTICE INVITING TENDER

Sub: Civil, Electrical & Interior work for Lifecare Centre at Regional Institute of ophthalmology (RIO), Trivandrum

Tender No. - HLL/CHO/HCS/PROJ/2014-15/002 dated 11-06-2013

HLL Lifecare Ltd, a Government of India Undertaking, invites sealed and super scribed quotation from interested contractors having minimum two years of experience in the relevant field for the work at Regional Institute of Ophthalmology, Trivandrum for our Lifecare Centre.

Publishing date of bid: 05thAugust 2014.

Last date of submission (at the office of the Group Head (HCS), Corporate and Regd. Office, HLL Bhavan, Poojappura, Thiruvananthapuram): 20th August 2014, 15.00 Hours.

Opening of bid: 20th August 2014, 16.00 Hours at HLL Lifecare Ltd, Poojappura Thiruvananthapuram)

Interested applicants can participate in the bid.

HLL reserves the right to accept or reject any application without assigning any reason or incurring any liability whatsoever. Prospective bidders are advised to regularly scan through HLL web site as corrigendum/amendments etc., if any, will be notified on the HLL web site and separate advertisement will not be made for this regard

For HLL Lifecare Ltd.

Associate Vice President (HCS)i/c

GENERAL TERMS & CONDITIONS

- Eligibility criteria:

  - Bidders should have executed satisfactorily at least one job preferably of similar kind and contract should be valuing not less than ten lakhs during the last one year in satisfactory manner.

  - Bidders should have minimum two years’ experience in the relevant field.

Documental proof for the above eligibility criteria should be submitted along with offer.

- The schedule of quantities (ANNEXURE-A) is appended. Bidders are advised to go through the entire general terms and conditions of this contract carefully.

- Earnest money of Rs. 50,000/-(Rupees Fifty thousand only) in the form of a Demand Draft of a scheduled bank issued in favor of HLL Lifecare LTD, HLL Bhavan, Poojappura, Thiruvananthapuram, which should be placed in a separate sealed cover marked “Earnest Money” shall be submitted along with the tender.EMD of the unsuccessful bidders will be returned with in 15days from the date of issue of work order. The EMD of the successful bidder will be converted into security deposit. Security deposit will be released along with the final payment.

- The filled & duly signed tender (signature on all pages of the tender) should be submitted to Associate Vice President (HCS)i/c, Healthcare Service Division, HLL Lifecare Limited, HLL Bhavan, Poojappura, Thiruvananthapuram, PH- 0471-2354949 Ext. 331, 325, 326, 289 on or before 20-08-2013 by 15.00 Hrs. The application shall be clearly marked “Tender for Civil, Electrical & Interior work at Lifecare Centre at Regional Institute of Ophthalmology, Trivandrum”. The tender will be opened at 16.00Hrs on the same day itself in the presence of interested bidders or their authorized representatives who choose to attend at the time of opening of tender.

- The work shall be completed within 30 days from the date of receipt of Work Order or LOI

- The tender for the work shall remain open for acceptance for a period of 90days from the date of opening of tender.

- The contractor shall visit the site before quoting the rates and clarifications if any required can be had from the address mentioned above.

- Rates quoted should be inclusive of all cost of materials, Tools/Equipments labor charges, conveyance to site, handling charges, loading and unloading charges, hiring charges, clearing of debris, statutory payments etc.

- The Management reserves the right to accept or reject the Tender either partially or fully without assigning any reason what so ever.

- The work should be carried out without causing any inconvenience to the public and shall ensure that no damages are caused to the existing site premises.

- During the execution of work the contractor or authorized representative should be present at site.

- All Materials, Equipments/ Tools required for the work should be arranged by the contractor and brought to site for the timely completion of the work.

- The materials used shall be as per specification and of good quality.

- The Contractor has to arrange necessary insurance coverage for the machine, workmen etc. deployed by him. He shall arrange all safety measures to protect his workmen and also the properties of HLL.

- Retention Money and Defect liability period: Retention money will be recovered @ 5% on each bills submitted by the contractor. The retention money will be released after the defect liability period of one year from the date of issue of Work Completion Certificate. The work site safety of all employees, their ESI, PF etc will have to be borne by the contractor.

- Taxes & Duties: The amounts quoted in the tender  shall be inclusive of all taxes and duties

- Penalty for delay: Penalty shall be imposed at 0.5% per week to a maximum of 7.5% of the contract value for any delay committed by the contractor in completing the work as per the order.

- Measurement & Payment terms: The method of measurement of completed work shall be in accordance with the standard measurement. Payment will be made on satisfactorily completion of work as per the order. Interim or running account bills shall be submitted by the contractor for the work executed on the basis of such recorded measurements in the format of HLL. All such interim payments shall be regarded as payment by way of advances against final payment only, and shall not preclude the requiring of bad, unsound and imperfect or unskilled work to be rejected, removed, taken away and reconstructed or re erected. Any certificate given by the officer  relating to the work done or materials delivered forming part of such payment, may be modified, or corrected by any subsequent such certificate(s) or by the final certificate and shall not by itself be conclusive evident that any work or material so which it relates is /are in accordance with the contract and certificate. Any such interim payment,/any part  there of shall not in any respect conclude, determine or affect in any way powers of the engineer in charge under the contract or any of such payment s be treated as final settlement and adjustment of accounts or in any way vary or affect the contract.

- Payment Schedule: The contractor can submit two running account bills during the work period. 80% of the bill value will be paid on each running account bill submitted by the contractor.  The remaining 20% will be paid along with the final bill only after issue of Work Completion certificate by Engineer In Charge/Officer In Charge of HLL Lifecare Ltd  .The running account bills is to be submitted detailing the work description ,quantity and rate as per the Work Order. Payments will be made after effecting the statutory deductions of TDS, Work Contract Tax etc .Retention money @ 5% will be deducted on each bill as per clause 15.

- The Quantity shown in the schedule is an approximate estimated quantity. No rate revision will be entertained if the quantity exceeds due to the site condition while executing the work.

- Final payment shall be paid only after clearing the site as per direction of Engineer-in-charge/ Officer in charge.

- Settlement of Dispute: Arbitration shall not be the means of settlement of dispute or claims or   anything on account of this Contract. If any disputes and difference arising out of the contract are to be settled by a civil court at Thiruvananthapuram.

For HLL Lifecare Ltd.

Associate Vice President (HCS)i/c

ANNEXURE-A

| Item No | Details of items | Quantity | Unit | Rate | Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | Dismantling old plaster or skirting raking out joints and cleaning the surface for plaster including disposal of rubbish to the dumping ground within 50 metres lead. | 45.00 | m2 |  |  |
| 2 | Demolishing cement concrete manually/ by mechanical means including disposal of material within 50 metres lead as per direction of Engineer - in - charge : Nominal concrete 1:3:6 or richer mix (i/c equivalent design mix) | 6.50 | m3 |  |  |
| 3 | Dismantling steel work in built up/tubular sections in angles, tees, flats and channels including all gusset plates, bolts, nuts, cutting rivets, welding etc. including dismembering and stacking within 50metres lead | 650.00 | kg |  |  |
| 4 | Dismantling roofing including ridges, hips, valleys and gutters etc., and stacking the material within 50 metres lead of: G.S. Sheet | 45.00 | m2 |  |  |
| 5 | Disposal of building rubbish / malba / similar unserviceable, dismantled or waste materials by mechanical means including loading, transporting, unloading to approved municipal dumping ground or as approved by Engineer-in-charge, beyond 50 m initial lead, for all loads including all lifts involved | 6.50 | m3 |  |  |
| 6 | Earth work in excavation by mechanical means (Hydraulic excavator) / manual means in foundation trenches or drains (not exceeding 1.5 m in width or 10 sqm on plan) including dressing of sides and ramming of bottoms, lift upto 1.5 m, including getting out the excavated soil and disposal of surplus excavated soil as directed, within a lead of 50 m. All kinds of soil. | 30.00 | m3 |  |  |
| 7 | Providing and laying in position cement concrete of specified grade excluding the cost of centering and shuttering - All work up to plinth level : 1:4:8 (1 Cement : 4 coarse sand : 8 graded stone aggregate 40 mm nominal size). | 5.50 | m3 |  |  |
| 8 | Providing and laying in position cement concrete of specified grade excluding the cost of centring and shuttering - All work up to plinth level : 1:3:6 (1 Cement : 3 coarse sand : 6 graded stone aggregate 20 mm nominal size). For flooring concrete | 4.70 | m3 |  |  |
| 9 | Random rubble masonry with hard stone in foundation and plinth including levelling up with cement concrete 1:6:12 (1 cement : 6 coarse sand : 12 graded stone aggregate 20mm nominal size) upto plinth level with : Cement mortar 1:6 (1 cement : 6 coarse sand). | 29.00 | m3 |  |  |
| 10 | Brick work with common burnt clay F.P.S. (non modular) bricks of class designation 7.5 in superstructure above plinth level up to floor V level in all shapes and sizes in : Cement mortar 1:6 (1 cement : 6 coarse sand) | 26.00 | m3 |  |  |
| 11 | Reinforced cement concrete work in walls (any thickness), including attached pilasters, buttresses, plinth and string courses, fillets, columns, lintels, beams , pillars, piers, abutments, posts and struts etc. up to floor five level excluding cost of centering, shuttering, finishing and reinforcement: 1:1.5:3 (1 cement : 1.5 coarse sand : 3 graded stone aggregate 20 mm nominal size) | 1.70 | m3 |  |  |
| 12 | Centering and shuttering including strutting, propping etc. and removal of form for : Lintels, beams, plinth beams, girders, bressumers and cantilevers. | 20.00 | m2 |  |  |
| 13 | Steel reinforcement for R.C.C. work including straightening, cutting, bending, placing in position and binding all complete above plinth level. Thermo-Mechanically Treated bars. | 170.00 | kg |  |  |
| 14 | Providing and fixing Ist quality ceramic glazed wall tiles 300 x 450mm size conforming to IS : 15622 (thickness to be specified by the manufacturer ) of approved make in all colours, shades except burgundy, bottle green, black of any size as approved by Engineer-in-Charge, in skirting, risers of steps and dados over 12 mm thick bed of cement Mortar 1:3 (1 cement : 3 coarse sand) and jointing with grey cement slurry @ 3.3kg per sqm including pointing in white cement mixed with pigment of matching shade complete. | 26.00 | m2 |  |  |
| 15 | Providing and laying Ceramic glazed floor tiles 300x300 mm (thickness to be specified by the manufacturer) of 1st quality conforming to IS : 15622 of approved make in colours such as White, Ivory, Grey, Fume Red Brown, laid on 20 mm thick Cement Mortar 1:4 (1 Cement : 4 Coarse sand) including pointing the joints with white cement and matching pigment etc., complete. | 4.00 | m2 |  |  |
| 16 | Providing and laying vitrified floor tiles in different sizes (thickness to be specified by the manufacturer) with water absorption's less than 0.08% and conforming to IS : 15622 of approved make in all colours and shades, laid on 20mm thick cement mortar 1:4 (1 cement : 4 coarse sand) including grouting the joints with white cement and matching pigments etc., complete. Size of Tile 600x600 mm | 75.00 | m2 |  |  |
| 17 | 12 mm cement plaster of mix : 1:4 (1 cement: 4 fine sand)for inside walls | 240.00 | m2 |  |  |
| 18 | Providing and fixing 12 mm thick frameless toughened glass door shutter of approved brand and manufacture, including providing and fixing top & bottom pivot & spring type fixing arrangement and making necessary holes etc. for fixing required door fittings, all complete as per direction of Engineer-in-charge (Door handle, lock and stopper etc.to be paid separately). | 9.50 | m2 |  |  |
| 19 | Supplying and fixing of BTS 75 V floor spring for Aluminium door (Dorma make) as per specification and as directed by the Engineer in charge. | 1 | Each |  |  |
| 20 | Providing and fixing aluminium tubular handle bar 32 mm outer dia, 3.0 mm thick & 2100 mm long with SS screws etc .complete as per direction of Engineer-in-Charge. Powder coated minimum thickness 50 micron aluminium tubular handle bar. | 2 | Each |  |  |
| 21 | Providing wood work in frames of doors, windows, clerestory windows and other frames, wrought framed and fixed in position with hold fast lugs or with dash fasteners of required dia & length ( hold fast lugs or dash fastener shall be paid for separately). Kiln seasoned and chemically treated hollock wood. | 0.40 | m3 |  |  |
| 22 | Providing and fixing panelled or panelled and glazed shutters for doors, windows and clerestory windows including ISI marked M.S. pressed butt hinges bright finished of required size with necessary screws excluding, panelling which will be paid for separately, all complete as per direction of Engineer-in-charge. Kiln seasoned and chemically treated hollock wood: 30 mm thick shutters | 4.50 | m2 |  |  |
| 23 | Providing and fixing panelling or panelling and glazing in panelled or panelled and glazed shutters for doors, windows and clerestory windows (Area of opening for panel inserts excluding portion inside grooves or rebates to be measured). Panelling for panelled or panelled and glazed shutters 25 mm to 40 mm thick : |  |  |  |  |
| a | Kiln seasoned and chemically treated hollock wood. | 4.50 | m2 |  |  |
| b | 5.5 mm thick glass panes | 1.60 | m2 |  |  |
| 24 | Providing and fixing factory made UPVC door frame made of UPVC extruded sections having an overall dimension as below (tolerance ±1mm), with wall thickness 2.0 mm (± 0.2mm), corners of the door frame to be Jointed with galvanized brackets and stainless steel screws, joints mitred and Plastic welded. The hinge side vertical of the frames reinforced by galvanized M.S. tube of size 19 X 19mm and 1mm (± 0.1 mm) wall thickness and 3 nos. stainless steel hinges fixed to the frame complete as per manufacturers specification and direction of Engineer-in-charge Extruded section profile size 48x40 mm | 11.50 | m |  |  |
| 25 | Providing and fixing to existing door frames. 24 mm thick factory made PVC door shutters made of styles and rails of a UPVC hollow section of size 59x24 mm and wall thickness 2 mm (± 0.2 mm) with inbuilt edging on both sides. The styles and rails mitred and joint at the corners by means of M.S. galvanised/plastic brackets of size 75x220 mm having wall thickness 1.0 mm and stainless steel screws. The styles of the shutter reinforced by inserting galvanised M.S. tube of size 20x20 mm and 1 mm (± 0.1 mm) wall thickness | 3.20 | m2 |  |  |
| 26 | Providing and fixing M.S. grills of required pattern in frames of windows etc. with M.S. flats, square or round bars etc. including priming coat with approved steel primer all complete. Fixed to openings /wooden frames with rawl plugs screws etc. | 1200.00 | kg |  |  |
| 27 | Providing and fixing hard drawn steel wire fabric 75x25 mm mesh of weight not less than 7.75 Kg per sqm to window frames etc. including 62x19 mm beading of second class teak wood and priming coat with approved steel primer all complete | 13.00 | m2 |  |  |
| 28 | Supplying and fixing rolling shutters of approved make, made of required size M.S. laths, interlocked together through their entire length and jointed together at the end by end locks, mounted on specially designed pipe shaft with brackets, side guides and arrangements for inside and outside locking with push and pull operation complete, including the cost of providing and fixing necessary 27.5 cm long wire springs manufactured from high tensile steel wire of adequate strength conforming to IS: 4454 – part 1 and M.S. top cover of required thickness for rolling shutters. 80x1.20 mm M.S. laths with 1.20 mm thick top cover | 16.00 | m2 |  |  |
| 29 | Steel work in built up tubular (round, square or rectangular hollow tubes etc.) trusses etc. including cutting, hoisting, fixing in position and applying a priming coat of approved steel primer, including welding and bolted with special shaped washers etc. complete. Hot finished welded type tubes | 1150.00 | kg |  |  |
| 30 | Providing and fixing precoated galvanised iron profile sheets (size, shape and pitch of corrugation as approved by Engineer-in-charge) 0.50 mm + 0.05 %, total coated thickness with zinc coating 120 gsm as per IS: 277 in 240 mpa steel grade, 5-7 microns epoxy primer on both side of the sheet and polyester top coat 15-18 microns. Sheet should have protective guard film of 25 microns minimum to avoid scratches while transportation and should be supplied in single length upto 12 metre or as desired by Engineer-in-charge. |  |  |  |  |
|  | The sheet shall be fixed using self drilling /self tapping screws of size (5.5x 55mm) with EPDM seal, complete upto any pitch in horizontal/ vertical or curved surfaces excluding the cost of purlins, rafters and trusses and including cutting to size and shape wherever required. | 112.00 | m2 |  |  |
| 31 | Providing and fixing precoated galvanised steel sheet roofing accessories 0.50 mm + 0.05 % total coated thickness, Zinc coating 120gsm as per IS: 277 in 240 mpa steel grade, 5-7 microns epoxy primer on both side of the sheet and polyester top coat 15-18 microns using self drilling/ self tapping screws complete : Ridges plain (500 - 600mm). | 14.00 | m |  |  |
| 32 | Providing and applying plaster of paris putty of 2 mm thickness over plastered surface to prepare the surface even and smooth complete | 155.00 | m2 |  |  |
| 33 | Applying one coat of water thinnable cement primer of approved brand and manufacture on wall surface : Water thinnable cement primer | 155.00 | m2 |  |  |
| 34 | Wall painting with acrylic emulsion paint of approved brand and manufacture to give an even shade:Two or more coats on new work- Inside walls -Two or more coats on new work. | 155.00 | m2 |  |  |
| 35 | Finishing walls with Acrylic Smooth exterior paint of required shade : New work (Two or more coat applied @ 1.67 ltr/10 sqm over and including priming coat of exterior primer applied @ 2.20 kg/ 10 sqm).-external walls | 85.00 | m2 |  |  |
| 36 | Painting with synthetic enamel paint of approved brand and manufacture to give an even shade : Two or more coats on new work. For steel works | 125.00 | m2 |  |  |
| 37 | Painting wood work with Deluxe Multi Surface Paint of required shade. Two or more coat applied @ 0.90 ltr/10 sqm over an under coat of primer applied @0.75 ltr/ 10 sqm of approved brand or manufacture | 20.00 | m2 |  |  |
| 38 | Providing and fixing false ceiling at all height including providing and fixing of frame work made of special sections, power pressed from M.S. sheets and galvanized with zinc coating of 120 gms/sqm (both side inclusive) as per IS : 277 and consisting of angle cleats of size 25 mm wide x 1.6 mm thick with flanges of 27 mm and 37mm, at 1200 mm centre to centre, one flange fixed to the ceiling with dash fastener 12.5 mm dia x 50mm long with 6mm dia bolts other flange of cleat fixed to the angle hangers of 25x10x0.50 mm of required length with nuts & bolts of required size and other end of angle hanger fixed with intermediate G.I. channels 45x15x0.9 mm running at the rate of 1200 mm centre to centre to which the ceiling section 0.5 mm thick bottom wedge of 80 mm with tapered flanges of 26 mm each having lips of 10.5 mm, at 450 mm centre to centre, shall be fixed in a direction perpendicular to G.I. intermediate channel with connecting clips made out of 2.64 mm dia x 230 mm long G.I. wire at every junction, including fixing perimeter channels 0.5 mm thick 27 mm high having flanges of 20 mm and 30 mm long, the perimeter of ceiling fixed to wall/partition with the help of rawl plugs at 450 mm centre, with 25mm long dry wall screws @ 230 mm interval, including fixing of gypsum board to ceiling section and perimeter channel with the help of dry wall screws of size 3.5 x 25 mm at 230 mm c/c, including jointing and finishing to a flush finish of tapered and square edges of the board with recommended |  |  |  |  |
|  | jointing compound , jointing tapes , finishing with jointing compound in 3 layers covering upto 150 mm or both sides of joint and two coats of primer suitable for board, all as per manufacturer’s specification and also including the cost of making openings for light fittings, grills, diffusers, cutouts made with frame of perimeter channels suitably fixed, all complete as per drawings, specification and direction of the Engineer in Charge but excluding the cost of painting with : 12.5 mm thick tapered edge gypsum fire resistant board conforming to IS: 2095- Part I. | 40.00 | m2 |  |  |
| 39 | Providing and fixing Pre-laminated (both side) flat pressed 3 layer (medium density) particle board or graded wood particle board IS : 3087 marked, Type II exterior grade IS : 12823 marked, in shelves with screws and fittings wherever required, including railing for the easy movement of drawers (fittings to be paid separately). 18 mm thick | 110.00 | m2 |  |  |
| 40 | Supplying and fixing 6mm thick glass panes for disply racks including necessary supports to the glass panes etc. complete | 21.00 | m2 |  |  |
| 41 | Supplying and fixing Mirror to the shutters 60x150cm side including fitting charges etc. complete. | 5.00 | Each |  |  |
| 42 | Providing and fixing bright finished brass handles with screws etc. complete : 125mm | 46.00 | Each |  |  |
| 43 | Providing and fixing water closet squatting pan (Indian type W.C. pan ) with 100mm sand cast Iron P or S trap, 10 litre low level white P.V.C. flushing cistern, including flush pipe, with manually controlled device (handle lever) conforming to IS : 7231, with all fittings and fixtures complete including cutting and making good the walls and floors wherever required : White Vitreous china Orissa pattern W.C. pan of size 580x440mm with integral type foot rests. | 2 | Each |  |  |
| 44 | Providing and fixing wash basin with CI / MS brackets, 15mm CP brass Pillar taps, 32mm CP brass waste of standard pattern, including painting of fittings and brackets, cutting and making good the walls wherever required.: White Vitreous China Wash basin size 630 x 450mm with a single 15mm C.P brass piller tap. Make:Hindware,Cera,Jaquar etc.., | 1 | Each |  |  |
| 45 | Providing and fixing C.P. brass bib cock of approved quality conforming to IS:8931. Make:Hindware,Cera,Jaquar etc.., 15 mm nominal bore. | 4 | Each |  |  |
| 46 | Providing and fixing C.P. brass stop cock (concealed) of standard design and of approved make conforming to IS:8931: 15 mm nominal bore. | 2 | Nos. |  |  |
| 47 | Providing and fixing C.P. brass angle valve for basin mixer and geyser points of approved quality conforming to IS:8931 a) 15 mm nominal bore: 15mm nominal bore | 1 | Nos. |  |  |
| 48 | Bottle trap 38mm single piece moulded with height of 270mm, effective length of tail pipe 260mm from the centre of the waste coupling 77mm breadth with 25mm minimum water seal, weighing not less than 263gms. | 1 | Nos. |  |  |
| 49 | Providing and fixing CP Waste coupling for wash basin and sink of approved quality, colour and make. Waste coupling 31mm of 79mm length and 62mm breadth, weighing not less than 45 gms. Make:Hindware,Jaquar etc.., | 1 | Nos. |  |  |
| 50 | Providing and fixing PVC Waste pipe for sink or wash basin including PVC waste fittings complete.:Semi rigid pipe 32mm dia | 1 | Nos |  |  |
| 51 | Providing and fixing SS soap Dish Holder having length of 138mm, breadth 102mm, height of 75mm with concealed fitting arrangements. Weighing not less than 106 gms. | 3 | Nos. |  |  |
| 52 | Providing and fixing Toilet paper holder . Make:Hindware,Cera,Jaquar etc..,:C.P Brass | 2 | Nos. |  |  |
| 53 | Providing and fixing 600 x 450mm bevelled edge mirror of superior glass (of approved quality) complete with 6mm thick hard board ground fixed to wooden cleats with C.P. brass screws and washers complete. | 1 | Nos. |  |  |
| 54 | Providing and fixing SS towel rail complete with brackets fixed to wooden cleats with CP brass screws with concealed fitting arrangement of approved quality and colour. 600mm long towel rail with total length of 645mm, width 78mm and effective height of 88mm, weighing not less than 190gms. | 3 | Nos. |  |  |
| 55 | Providing and fixing C.P. brass shower rose with 15 or 20 mm inlet : 150 mm diameter | 2 | Nos. |  |  |
| 56 | Providing and fixing PVC pipes for water supply pipe line including earthwork excavation cost of pipes, specials and labour charges etc. complete. |  |  |  |  |
|  | External works |  |  |  |  |
| a) | 20mm dia. PVC pipes 12 kg/cm2 | 50.00 | Mtr. |  |  |
| b) | 25mm dia. PVC pipes 10 kg/cm3 | 10.00 | Mtr. |  |  |
| 57 | Providing and fixing PVC pipes for water supply pipe line including cost of pipes, specials and labour charges. |  |  |  |  |
|  | Concealed work including cutting chases and making good the wall etc. |  |  |  |  |
| a) | 20mm dia. PVC pipes 12 kg/cm2 | 20.00 | Mtr. |  |  |
| b) | 25mm dia. PVC pipes 10 kg/cm3 | 15.00 | Mtr. |  |  |
| 58 | Providing and fixing Ball Valve (brass) of approved quality, High or low pressure, with plastic floats complete. Make :Lehry, Supreme, Finolex, Prince etc..,: 20mm dia nominal bore | 3 | Nos. |  |  |
| 59 | Providing and fixing Gun metal gate valve with C.I. wheel of approved quality (screwed end). Make: Lehry,Supreme,Finolex,Prince etc.., 25 mm dia nominal bore | 1 | Nos. |  |  |
| 60 | Cutting holes upto 15 x 15 cm in R.C.C. floors and roofs for passing drain pipe etc. and repairing the hole after insertion of drain pipe etc. with cement concrete 1:2:4 (1 cement : 2 coarse sand : 4 graded stone aggregate 20mm nominal size) including finishing complete. | 3 | each |  |  |
| 61 | Providing and fixing on wall face unplasticised Rigid PVC rain water pipes conforming to IS : 13592 Type A including jointing with seal ring conforming to IS : 5382 leaving 10 mm gap for thermal expansion.(i) Single socketed pipes. 110 mm diameter | 5 | mtr. |  |  |
| 62 | Providing and fixing on wall face unplasticised - PVC moulded fittings/ accessories for unplasticised Rigid PVC rain water pipes conforming to IS : 13592 Type A including jointing with seal ring conforming to IS : 5382 leaving 10 mm gap for thermal expansion. |  |  |  |  |
| a) | 110 mm Coupler | 2 | each |  |  |
| c) | 110 mm bend | 2 | each |  |  |
| d) | 110 mm Shoe | 1 | each |  |  |
| 63 | Providing and fixing unplasticised -PVC pipe clips of approved design to unplasticised - PVC rain water pipes by means of 50x50x50mm hard wood plugs, screwed with M.S. screws of required length including cutting brick work and fixing in cement mortar 1:4 (1 cement : 4 coarse sand) and making good the wall etc. complete: 110mm | 5 | each |  |  |
| 64 | Providing and fixing PVC soil, waste and vent pipe to IS : 13592 of SWR quality 6kg / sq.cm pressure rated up to 110mm dia and 6 kg / sq.cm pressure rated agricultural series above 110mm, including all fittings, like EG : Bends, Tees, Elbows, Collars, junction, inspection doors, cowls, offsets, access pipes, jointing with rubber ring / solvent cement by application of leak proof adhesive like FRP paste including necessary earthwork excavation, cutting holes in walls and floors and making good. Make:Supreme,Finolex,Prince etc.., |  |  |  |  |
| a) | 50mm dia (6kg / sq.cm) | 10.00 | Mtr. |  |  |
| b) | 110mm dia (6 kg / sq.cm) | 30.00 | Mtr. |  |  |
| 65 | S.C.I. S&S, Slotted Cowl (Terminal Guard) 100 mm | 1 | Nos |  |  |
| 66 | Providing and fixing to the Inlet mouth of rain water pipe S.S grating 110mm diameter and weighing not less than 440 grams. | 1 | Nos |  |  |
| 67 | Providing and fixing PVC floor trap self cleansing design with or without vent arm complete including cutting and making good the walls and floors. Make:Supreme,Finolex,Prince etc..,110mm dia inlet 110mm dia outlet | 2 | Nos |  |  |
| 68 | Providing and fixing CP grating of 6mm thick with rim including setting in floor with white cement mortar and making good the floor (Chilli make) :125mm dia | 2 | Nos. |  |  |
| 69 | Constructing brick masonry manhole in cement mortar 1:4 ( 1 cement : 4 coarse sand ) with R.C.C. top slab with 1:2:4 mix (1 cement : 2 coarse sand : 4 graded stone aggregate 20 mm nominal size), foundation concrete 1:4:8 mix (1 cement : 4 coarse sand : 8 graded stone aggregate 40mm nominal size), inside plastering 12mm thick with cement mortar 1:3 (1 cement : 3 coarse sand) finished with floating coat of neat cement and making channels in cement concrete 1:2:4 (1 cement : 2 coarse sand : 4 graded stone aggregate 20mm nominal size) finished with a floating coat of neat cement complete as per standard design : |  |  |  |  |
| 70a | Inside size 90x80 cm and 45 cm deep including C.I. cover with frame (light duty) 455x610 mm internal dimensions, total weight of cover and frame to be not less than 38 kg (weight of cover 23 kg and weight of frame 15 kg) : With common burnt clay F.P.S. (non modular) bricks of class designation 7.5 | 4 | Nos. |  |  |
| 70b | Inside size 120x90 cm and 90 cm deep including C.I. cover with frame (medium duty) 500 mm internal diameter, total weight of cover and frame to be not less than 116 kg (weight of cover 58 kg and weight of frame 58 kg) : With common burnt clay F.P.S. (non modular) bricks of class designation 7.5 | 1 | Nos. |  |  |
| 71 | Extra for depth for manholes |  |  |  |  |
| a) | Size 90x80 cm: With common burnt clay F.P.S. (non modular) bricks of class designation 7.5 | 1 | m |  |  |
| b) | Size 120x90 cm: with common burnt clay F.P.S. (non modular) bricks of class designation 7.5 | 1 | m |  |  |
| 72 | Providing and fixing square – mouth PVC. Gully trap grade “A” complete with CI grating brick masonry chamber with water tight tight CI cover with frame of 300 x 300mm size (inside) the weight of cover to be not less than 4.50 kg and frame to be not less than 2.70 kg as per standard design : 150 x 100mm size P type with common burnt clay F.P.S. (non modular) bricks of class designation 7.5 | 3 | each |  |  |
| 73 | Providing and placing on terrace polyethylene water storage tank ISI : 12701 marked with cover and suitable locking arrangement and making necessary holes for inlet, outlet and overflow pipes but without fittings and the base support for tank | 500 | Ltrs |  |  |
|  | TOTAL AMOUNT (in figures) |  |  |  |  |
|  | TOTAL AMOUNT (in words) |  |  |  |  |

Place                                                 Signature of the Bidder

Date                                                  Name of the Bidder