HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

TENDER NO.HL:BG:PS:SS REACTOR:2013-14     DT.25.11.2013

USER REQUIREMENT SPECIFICATION

ANNEXURE 1

All the Specifications given below shall be completed in the Supplier Specifications Column with accurate information i.e., in case of all the Parameters are complied as per the given Specification Parameters, the same Specification shall be mentioned in the respective column, and in the case of deviation from the given Specification Parameters, appropriate deviations from the required Specifications shall clearly be stated. In the absence of above and in case the columns are filled with “Yes and No” Parameters such Tenders will be summarily rejected. Qualification Criteria in Annexure 2 is must for qualification.

| SL. NO. | #### SCOPE OF SUPPLY |
| --- | --- |
| 1 | Supply Installation and Commission SS Reactor with Condenser and Receiver as per Detailed Specification given in the document. |
| 2 | Validation Documents IQ / OQ / DQ |
| 3 | Installation & Commissioning of supplied System and imparting Training to Employees |
| 4 | Validation of System |
| 5 | Flameproof Certificate from Certifying Body wherever applicable shall be provided. |

We have read and understood the above Specifications and agree to abide by the same.

Place:							Signature of the Bidder

Date:							Name, Seal and Address of the Bidder

Page NO.1 of 8

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

TENDER NO.HL:BG:PS:SS REACTOR:2013-14     DT.25.11.2013

USER REQUIREMENT SPECIFICATION

ANNEXURE 1

| SUPPLIERS NAME: |  |  |
| --- | --- | --- |
| CLIENT’S NAME: HLL LIFECARE LIMITED, KANAGALA |  |  |
| Sub: Technical Bid for Supply, Installation and Commissioning of REACTOR SS WITH CONDENSER AND RECEIVER Applicable Design Code: ASME Sec VIII Div. I (Latest addenda) |  |  |
| Sl No | HLL Specification | Supplier Specifications |
| I | TECHNICAL SPECIFICATION |  |
| 01 | Equipment Name: SS Reactor |  |
| 02 | Type: Impeller |  |
| 03 | Capacity: 2000 liters |  |
| 04 | Design Pressure (Shell): 6 Kg/cm2/full vacuum |  |
| 05 | Design Pressure (Jacket): 6 Kg/cm2/ full vacuum |  |
| 06 | Design Temperature: -10 to 180oC max |  |
| 07 | Operating Pressure (Shell): 4.5 Kg/cm2 / full vacuum |  |
| 08 | Operating Pressure (Jacket): 4 Kg/cm2 / full vacuum (for steam application) |  |
| 09 | Operating Temperature: -5 0C TO 145 oC |  |
| 10 | Test Pressure (Shell): 7.8 Kg/cm2 |  |
| 11 | Test Pressure (Jacket): 7.8 Kg/cm2 |  |
| 12 | Shell inside dia: 1300 mm |  |
| 13 | Height: 1500 mm |  |
| 14 | Jacket inside dia: 1400 mm |  |
| 15 | Shell Thickness: As per Design Parameters |  |
| 16 | Dished end Thickness: As per Design Parameters |  |
| 17 | Jacket Thickness: As per Design Parameters |  |
| 18 | Dish end Jacket Thickness: As per Design Parameters |  |
| II | MATERIAL OF CONSTRUCTION |  |
| 01 | Shell, Heads: SS316L |  |
| 02 | Jacket: CS |  |
| 03 | Internals: SS316L |  |
| III | Support: The Reactor is provided with Four LUG Support. |  |
| 01 | Reflux line size pipe = 40 NB |  |
| 02 | With U bend and valve pipe MOC: S.S 316L |  |
| 03 | With lantern Assembly, Stuffing Box, Bearing Housing, With Bearing Gland Pusher, With Gun metal Bush for cooling arrangement. (To be considered in Agitator Scope) |  |

We have read and understood the above Specifications and agree to abide by the same.

Place:							Signature of the Bidder

Date:							Name, Seal and Address of the Bidder

Page NO.2 of 8

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

TENDER NO.HL:BG:PS:SS REACTOR:2013-14     DT.25.11.2013

USER REQUIREMENT SPECIFICATION

ANNEXURE 1

| SUPPLIERS NAME: |  |  |
| --- | --- | --- |
| CLIENT’S NAME: HLL LIFECARE LIMITED, KANAGALA |  |  |
| Sub: Technical Bid for Supply, Installation and Commissioning of REACTOR SS WITH CONDENSER AND RECEIVER Applicable Design Code: ASME Sec VIII Div. I (Latest addenda) |  |  |
| Sl No | HLL Specification | Supplier Specifications |
| 04 | Temperature Sensor line is to be made 1 inch size on the topside Reactor. |  |
| 05 | Lubricants Detail: VG / Servo Mesh 320 for Gear Box. |  |
| 06 | Insulation of Jacket: 100 mm (50 mm + 50 mm) thick Rock Wool / PUF Insulation with 120 kg / cm3 density with SS304 Cladding (12 Gauges with screwed construction). |  |
| 07 | Provide Solenoid Valve to Steam line. With FLP PID Controller (Radix) |  |
| 08 | Temperature setting Indicator. With high & low Alarms |  |
| 09 | Reactor Volume Indicator. |  |
| IV | INSTRUMENTATION |  |
| 01 | Compound Gauge (Reactor Shell Side): 1 NO. |  |
| 02 | Temperature Gauge (Reactor Shell Side): 1 NO. |  |
| 03 | Pressure Gauge (Reactor Jacket Side): 1 NO. |  |
| 04 | Safety Valve (Reactor Shell Side): 1 NO. |  |
| 05 | Safety Valve (Reactor Jacket Side): 1 NO. |  |
| 06 | Compound Gauge (Receiver Shell side): 1 NO. |  |
| 07 | Light Glass (Reactor Shell side): 1 NO. |  |
| 08 | Side Glass (Reactor Shell side): 1 NO. |  |
| 09 | Manhole (Reactor Shell side): 1 NO. |  |
| 10 | Sampling Nozzle (Reactor Shell side): 1 NO. |  |
| 11 | Thermowell Nozzle (Reactor Shell side): 1 NO. |  |
| 12 | Bottom Outlet Nozzle with Ball Valve & Inline view glass (Reactor Shell side): 1 NO. |  |
| V | AGITATOR DATA SHEET |  |
| 01 | Type: Flat Blade Turbine |  |
| 02 | Speed: 150 rpm |  |
| 03 | Shaft dia: 80 mm |  |
| 04 | Number of Impellers: 4 NOS. |  |
| 05 | No of Blades: 6 NOS. |  |
| 06 | Impeller Sweep dia.: 420 mm |  |
| 07 | Distance between Impellers: 600 mm (As per Design) |  |

We have read and understood the above Specifications and agree to abide by the same.

Place:							Signature of the Bidder

Date:							Name, Seal and Address of the Bidder

Page NO.3 of 8

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

TENDER NO.HL:BG:PS:SS REACTOR:2013-14     DT.25.11.2013

USER REQUIREMENT SPECIFICATION

ANNEXURE 1

| SUPPLIERS NAME: |  |  |
| --- | --- | --- |
| CLIENT’S NAME: HLL LIFECARE LIMITED, KANAGALA |  |  |
| Sub: Technical Bid for Supply, Installation and Commissioning of REACTOR SS WITH CONDENSER AND RECEIVER Applicable Design Code: ASME Sec VIII Div. I (Latest addenda) |  |  |
| Sl No | HLL Specification | Supplier Specifications |
| 08 | MOC of Shaft: SS 316L |  |
| 09 | MOC of Impeller: SS 316L |  |
| 10 | Stool Top Bearing: 32212 |  |
| 11 | Sto1l Bottom Bearing: 32213 |  |
| 12 | % Solid in mixing (40%) |  |
| VI | GEAR BOX |  |
| 01 | Make: Greaves Gears |  |
| 02 | Type: V 500 |  |
| 03 | Reduction: 10:1 Worm Reduction Gear Type – RV |  |
| 04 | High Speed Coupling / Size: Direct Coupled |  |
| 05 | Low Speed Coupling Bolt: Flexible Pin & Bush Coupling BC4 |  |
| 06 | Stuffing Box: Mechanical Seal with Cooling |  |
| 07 | Electrical Panel Board: Control Panel Board Machine with Flameproof, Starter, On / Off |  |
| VII | MECHANICAL SEAL |  |
| 01 | Materials Specification: Complete Mechanical Seal with Sleeve Housing |  |
| 02 | Type: Double Mechanical Seal |  |
| 03 | Size: 3.125” – Approx. |  |
| 04 | Make: Leakproof / Colossus / AES |  |
| VIII | FLAMEPROOF MOTOR DATA SHEET |  |
| 01 | Frame: 132 |  |
| 02 | KW: 7.5 |  |
| 03 | HP: 10 |  |
| 04 | Amps: 14.5 |  |
| 05 | RPM: 1450 |  |
| 06 | Volt: 415 + 10% |  |
| 07 | Hz: 50 + 5% |  |
| 08 | Phase: 3 Phase |  |
| 09 | Type of Duty: S1 |  |
| 10 | Ins. Class: F |  |
| 11 | Rating: CONT |  |

We have read and understood the above Specifications and agree to abide by the same.

Place:							Signature of the Bidder

Date:							Name, Seal and Address of the Bidder

Page NO.4 of 8

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

TENDER NO.HL:BG:PS:SS REACTOR:2013-14     DT.25.11.2013

USER REQUIREMENT SPECIFICATION

ANNEXURE 1

| SUPPLIERS NAME: |  |  |
| --- | --- | --- |
| CLIENT’S NAME: HLL LIFECARE LIMITED, KANAGALA |  |  |
| Sub: Technical Bid for Supply, Installation and Commissioning of REACTOR SS WITH CONDENSER AND RECEIVER Applicable Design Code: ASME Sec VIII Div. I (Latest addenda) |  |  |
| Sl No | HLL Specification | Supplier Specifications |
| 12 | Motor Body Earthing: Double Earthing to Motor body with Copper wire of suitable size as per Standard. |  |
| 13 | Discharge of Static Electricity: To ensure Discharge Electricity and entire Reactor and drums used. |  |
| 14 | Specification of Static Electric Discharge Device: Model: Bond-Rite Clamp with 5 m (16Ft) cable BRC05 (Self testing clamps with and built in LED Indicators for Safety in critical operations that are prone to corrosion) |  |
| 15 | Flameproof Control Panel includes: Digital Temperature Indicator, Star Delta Starter, R-Y-B Lamp |  |
|  | Party should give successful trials for the above-mentioned functionality. |  |
| IX | CONDENSER DATA SHEET |  |
| 01 | Name of the Equipment: Condenser |  |
| 02 | Type: Shell and Tube type |  |
| 03 | Heat Transfer Area: 6.0 m2 |  |
| 04 | Tube Details: 1” x 2020 mm L x 56 Nos x Triangular Pitched to 31.75 mm |  |
| 05 | Operating Temperature: Tube side = 90 o C max., Shell side = 40 o C max. |  |
| 06 | Operating Pressure: Tube side = 6.0 kg / cm2 / full vacuum, Shell side - 3.0 kg / cm2 |  |
| 07 | Test Pressure: Tube side - 9.5 kg / cm2 Shell side – 5.2 kg / cm2 |  |
| 08 | Vapour Inlet: Tube side (SS 316L) |  |
| 09 | Cooling Medium: Shell side (SS 316L) with Insulation |  |
| X | CONDENSER INSTRUMENTATION |  |
| 01 | Compound Gauge: Condenser Vapor Inlet - 1 NO. |  |
| 02 | Temperature Gauge: Condenser Vapor Inlet and Outlet - 1 NO. each |  |
| 03 | Compound Gauge: Condenser Vapor Outlet - 1 NO. |  |
| 04 | Pressure Gauge: Condenser Vapor Outlet - 1 NO. |  |
| 05 | Vent Valve |  |

We have read and understood the above Specifications and agree to abide by the same.

Place:							Signature of the Bidder

Date:							Name, Seal and Address of the Bidder

Page NO.5 of 8

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

TENDER NO.HL:BG:PS:SS REACTOR:2013-14     DT.25.11.2013

USER REQUIREMENT SPECIFICATION

ANNEXURE 1

| SUPPLIERS NAME: |  |  |
| --- | --- | --- |
| CLIENT’S NAME: HLL LIFECARE LIMITED, KANAGALA |  |  |
| Sub: Technical Bid for Supply, Installation and Commissioning of REACTOR SS WITH CONDENSER AND RECEIVER Applicable Design Code: ASME Sec VIII Div. I (Latest addenda) |  |  |
| Sl No | HLL Specification | Supplier Specifications |
| XI | RECEIVER DATA SHEET |  |
| 01 | Name of the Equipment: Receiver |  |
| 02 | Type: Shell and Jacket type |  |
| 03 | Capacity: 300 L |  |
| 04 | Design Pressure (Shell): 2 kg/ cm2 |  |
| 05 | Design Pressure (Jacket): 5 kg/ cm2 |  |
| 06 | Design Temperature: 180 o C max |  |
| 07 | Operating Pressure (Shell): Atmosphere |  |
| 08 | Operating Pressure (Jacket): 3.0 kg / cm2 |  |
| 09 | Operating Temperature: 140 o C max |  |
| 10 | Test Pressure (Shell) [Kg/ cm2 ]: 2.6 kg/ cm2 |  |
| 11 | Test Pressure (Jacket) [Kg/ cm2 ]: 6.5 kg/ cm2 |  |
| 12 | Shell inside dia (mm): 700 mm |  |
| 13 | Height (mm): 800 mm |  |
| 14 | Jacket inside dia (mm): 800 mm |  |
| 15 | Shell Thickness (mm): As per Design Parameters |  |
| 16 | Dished end Thickness (mm): As per Design Parameters |  |
| 17 | Jacket Thickness (mm): As per Design Parameters |  |
| 18 | Dish end Jackets Thickness (mm): As per Design Parameters |  |
| 19 | Light and Sight Glass (mm): 100 mm |  |
| 20 | Insulation of Jacket: 100 mm (50 mm + 50 mm) thick Rock Wool / PUF Insulation with 120 kg / cm3 density with SS304 Cladding (12 Gauges with screwed construction). |  |
| XII | RECEIVER INSTRUMENTATION |  |
| 01 | Compound Gauge: Receiver Shell Side - 1 NO. |  |
| 02 | Temperature Gauge: Receiver Shell Side - 1 NO. |  |
| 03 | Pressure Gauge: Receiver Jacket Side - 1 NO. |  |
| 04 | Light Glass: Receiver Shell Side - 1 NO. |  |
| 05 | Side Glass: Receiver Shell Side - 1 NO. |  |
| 06 | Bottom Outlet Nozzle with Ball Valve: Receiver Shell Side - 1 NO. |  |
| 07 | Level Indicator |  |

We have read and understood the above Specifications and agree to abide by the same.

Place:							Signature of the Bidder

Date:							Name, Seal and Address of the Bidder

Page NO.6 of 8

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

TENDER NO.HL:BG:PS:SS REACTOR:2013-14     DT.25.11.2013

USER REQUIREMENT SPECIFICATION

ANNEXURE 1

SUPPLIERS NAME:

CLIENT’S NAME: HLL LIFECARE LIMITED, KANAGALA

Sub: Technical Bid for Supply, Installation and Commissioning of REACTOR SS WITH CONDENSER AND RECEIVER Applicable Design Code: ASME Sec VIII Div. I (Latest addenda)

NOTE:

- All the Instruments / Digital Indicators / Motor Control Panel etc. should Comply with the Flameproof (Gas Zone 2A, 2B) Requirements & Certifications.

- Nozzles for Reactor, Receiver & Condenser will be NB based pipes with flanged end connection. Flanges will be as per ASME B 16.5 Standard. Nut bolts & Counter Flange along with Gasket will be in Supplier’s scope of supply.

- Interconnecting piping & valves, between Reactor, Receiver & Condenser will be in Supplier’s scope of supply.

- All Utility piping & related Valves & Instruments will be in M/s. HLL scope.

We have read and understood the above Specifications and agree to abide by the same.

Place:							Signature of the Bidder

Date:							Name, Seal and Address of the Bidder

Page NO.7 of 8

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

TENDER NO.HL:BG:PS:SS REACTOR:2013-14     DT.25.11.2013

QUALIFICATION CRITERIA

ANNEXURE 2

| SUPPLIERS NAME: |  |  |
| --- | --- | --- |
| CLIENT’S NAME: HLL LIFECARE LIMITED, KANAGALA |  |  |
| Sub: Technical Bid for Supply, Installation and Commissioning of REACTOR SS WITH CONDENSER AND RECEIVER Applicable Design Code: ASME Sec VIII Div. I (Latest addenda) |  |  |
| Sl No | Eligibility Criteria | Supplier Response |
| 01 | Party should have supplied at least 3 NOS. of similar SS Reactors (same or higher capacity) Supply Order copies shall be enclosed as proof for verification. Supported with Customer Satisfaction Certificate. |  |
| 02 | Party should have supplied these Reactors to at least 1 API / Bulk Drug Manufacturers Supply Order copies shall be enclosed as proof for verification of the Vendors. |  |
| 03 | Party also should provide the references of Customers to whom we can contact about the Performance of the Equipment and also if required Demonstration at the Party’s premises will have to be arranged by Bidder. |  |

List of Enclosures to Technical Bid:

- _____________________________

- _____________________________

- _____________________________

- _____________________________

- _____________________________

SPECIAL NOTE:

- The Bid should include specific answers strictly with respect to our Tender Specifications and Specification deviation if any, should be highlighted with clarifications. Otherwise, the Bid will not be considered. Wherever space is insufficient, please enclose separate sheets.

- The Bid along with its enclosures should be furnished with Original only. Bid received by FAX / E-MAIL will not be considered.

Place:							Signature of the Bidder

Date:							Name, Seal and Address of the Bidder

Page NO.8 of 8