HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone: +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                 16/04/2014

## SECTION VIII

## ABSTRACT PRICE SCHEDULE

Sub: Electrical work of Hospital Projects works at Himachal Pradesh  :   KANGRA  District,:-

| Sl. No. | Details of Designated Site (Hospital Location) | Project reference |  |
| --- | --- | --- | --- |
| 1 | Public Health Centre, Haripur, KANGRA, Himachal Pradesh | Annexure 1 | a) ELECTRICAL WORK OF NBCC |
| 2 | Community Health Centre, Indora KANGRA, Himachal Pradesh | Annexure 2 | a) ELECTRICAL WORK OF NBCC |
| 3 | Community Health Centre, Rehan KANGRA, Himachal Pradesh | Annexure 3 | a) ELECTRICAL WORK OF NBCC |
| 4 | Community Health Centre, Jawali, KANGRA, Himachal Pradesh | Annexure 4 | a) ELECTRICAL WORK OF NBCC |
| 5 | Zonal Hospital, Dharmasala KANGRA, Himachal Pradesh | Annexure 5 | a) ELECTRICAL WORK OF NBCC |
|  |  |  | b) ELECTRICAL WORK OF NBCC at OT |
|  |  |  | c) ELECTRICAL WORK OF SNCU |
| 6 | Community Health Centre, Chadihar KANGRA, Himachal Pradesh | Annexure 6 | a) ELECTRICAL WORK OF NBCC |
|  |  |  | b) ELECTRICAL WORK OF NBSU |
| 7 | Community Health Centre, Jaisingpur KANGRA, Himachal Pradesh | Annexure 7 | a) ELECTRICAL WORK OF NBCC |
|  |  |  | b) ELECTRICAL WORK OF NBSU |
|  |  |  | c) ELECTRICAL WORK OF BSC |
| 8 | Community Health Centre, Nagrota Bhagawan, KANGRA, Himachal Pradesh | Annexure 8 | a) ELECTRICAL WORK OF NBCC |
|  |  |  | b) ELECTRICAL WORK OF BSC |
| 9 | Civil Hospital, Kangra KANGRA, Himachal Pradesh | Annexure 9 | a) ELECTRICAL WORK OF NBCC |
|  |  |  | b) ELECTRICAL WORK OF NBCC at OT |
|  |  |  | c) ELECTRICAL WORK OF NBSU |
|  |  |  | d) ELECTRICAL WORK OF BSC |
| 10 | Community Health Centre, Shahpur KANGRA, Himachal Pradesh | Annexure 10 | a) ELECTRICAL WORK OF NBCC |
|  |  |  | b) ELECTRICAL WORK OF NBSU |
| 11 | Community Health Centre, Gangath KANGRA, Himachal Pradesh | Annexure 11 | a) ELECTRICAL WORK OF NBCC |
|  |  |  | b) ELECTRICAL WORK OF NBSU |
| 12 | Civil Hospital, Nurpur KANGRA, Himachal Pradesh | Annexure 12 | a) ELECTRICAL WORK OF NBCC |
|  |  |  | b) ELECTRICAL WORK OF NBCC at OT |
|  |  |  | c) ELECTRICAL WORK OF SNCU |
|  |  |  | d) ELECTRICAL WORK OF BSC |
| 13 | Civil Hospital, Palampur KANGRA, Himachal Pradesh | Annexure 13 | a) ELECTRICAL WORK OF NBCC |
|  |  |  | b) ELECTRICAL WORK OF NBCC at OT |
|  |  |  | c) ELECTRICAL WORK OF SNCU |
| 14 | Civil Hospital, Baijnath KANGRA, Himachal Pradesh | Annexure 14 | a) ELECTRICAL WORK OF NBCC |
|  |  |  | b) ELECTRICAL WORK OF NBSU |
|  |  |  | c) ELECTRICAL WORK OF BSC |
| 15 | Civil Hospital, Thurnal KANGRA, Himachal Pradesh | Annexure 15 | a) ELECTRICAL WORK OF NBCC |
|  |  |  | b) ELECTRICAL WORK OF NBSU |
| 16 | Community Health Centre, Nagrota Surain, KANGRA, Himachal Pradesh | Annexure 16 | a) ELECTRICAL WORK OF NBCC |
|  |  |  | b) ELECTRICAL WORK OF BSC |
| 17 | Civil Hospital, Dehra KANGRA, Himachal Pradesh | Annexure 17 | a) ELECTRICAL WORK OF NBCC |
|  |  |  | b) ELECTRICAL WORK OF NBCC at OT |
|  |  |  | c) ELECTRICAL WORK OF NBSU |
|  |  |  | d) ELECTRICAL WORK OF BSC |
| 18 | Civil Hospital Jwalamukhi KANGRA, Himachal Pradesh | Annexure 18 | a) ELECTRICAL WORK OF NBCC |
|  |  |  | b) ELECTRICAL WORK OF NBSU |
|  |  |  | c) ELECTRICAL WORK OF BSC |

DRAWING ATTACHED

### The Tenderer has to submit their quote for all the schedules of the respective Tender.  If a tenderer is not quoting for all the schedules of the respective Tender, the tender will be  summarily rejected. HLL will place Work Order to the tenderer, whose total rate for all the hospital of the respective tender comes to lowest

### v

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                      16/04/2014

## HLL LIFECARE LIMITED

Akkulam Factory

THIRUVANANTHAPURAM-695017

HIMACHAL PRADESH  :  KANGRA DISTRICT

ELECTRICAL WORK

# ABSTRACT PRICE BID

| Sl. No. | Details of Designated Site (Hospital Location) |  | Amount (Rs:) |
| --- | --- | --- | --- |
| 1 | Public Health Centre, Haripur, KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
| 2 | Community Health Centre, Indora KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
| 3 | Community Health Centre, Rehan KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
| 4 | Community Health Centre, Jawali, KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
| 5 | Zonal Hospital, Dharmasala KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
|  |  | b) ELECTRICAL WORK OF NBCC at OT |  |
|  |  | c) ELECTRICAL WORK OF SNCU |  |
| 6 | Community Health Centre, Chadihar KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
|  |  | b) ELECTRICAL WORK OF NBSU |  |
| 7 | Community Health Centre, Jaisingpur KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
|  |  | b) ELECTRICAL WORK OF NBSU |  |
|  |  | c) ELECTRICAL WORK OF BSC |  |
| 8 | Community Health Centre, Nagrota Bhagawan, KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
|  |  | b) ELECTRICAL WORK OF BSC |  |
| 9 | Civil Hospital, Kangra KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
|  |  | b) ELECTRICAL WORK OF NBCC at OT |  |
|  |  | c) ELECTRICAL WORK OF NBSU |  |
|  |  | d) ELECTRICAL WORK OF BSC |  |
| 10 | Community Health Centre, Shahpur KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
|  |  | b) ELECTRICAL WORK OF NBSU |  |
| 11 | Community Health Centre, Gangath KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
|  |  | b) ELECTRICAL WORK OF NBSU |  |
| 12 | Civil Hospital, Nurpur KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
|  |  | b) ELECTRICAL WORK OF NBCC at OT |  |
|  |  | c) ELECTRICAL WORK OF SNCU |  |
|  |  | d) ELECTRICAL WORK OF BSC |  |
| 13 | Civil Hospital, Palampur KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
|  |  | b) ELECTRICAL WORK OF NBCC at OT |  |
|  |  | c) ELECTRICAL WORK OF SNCU |  |
| 14 | Civil Hospital, Baijnath KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
|  |  | b) ELECTRICAL WORK OF NBSU |  |
|  |  | c) ELECTRICAL WORK OF BSC |  |
| 15 | Civil Hospital, Thurnal KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
|  |  | b) ELECTRICAL WORK OF NBSU |  |
| 16 | Community Health Centre, Nagrota Surain, KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
|  |  | b) ELECTRICAL WORK OF BSC |  |
| 17 | Civil Hospital, Dehra KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
|  |  | b) ELECTRICAL WORK OF NBCC at OT |  |
|  |  | c) ELECTRICAL WORK OF NBSU |  |
|  |  | d) ELECTRICAL WORK OF BSC |  |
| 18 | Civil Hospital, Jwalamukhi KANGRA, Himachal Pradesh | a) ELECTRICAL WORK OF NBCC |  |
|  |  | b) ELECTRICAL WORK OF NBSU |  |
|  |  | c) ELECTRICAL WORK OF BSC |  |
| Total Rs: |  |  |  |

The Tenderer has to submit their quote for all the schedules of the respective Tender.  If a tenderer is not quoting for all the schedules of the respective Tender, the tender will be  summarily rejected. HLL will place Work Order to the tenderer, whose total rate for all the hospital of the respective tender comes to lowest

SEAL OF THE APPLICANT

Date:                                                                           SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone: +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Public Health Centre, Haripur)

[Annexure 1 (a)]

| NBCC : ELECTRICAL WORKS FOR Public Health Centre, Haripur |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |
|  | 3 module | Each | 1 |  |  |
|  | 6mod | Each | 1 |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL Rs: |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Indora)

[Annexure 2 (a)]

| NBCC : ELECTRICAL WORKS FOR Community Health Centre, Indora |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Rehan)

[Annexure 3 (a)]

| NBCC : ELECTRICAL WORKS FOR Community Health Centre, Rehan |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Jawali)

[Annexure 4 (a)]

| NBCC : ELECTRICAL WORKS FOR Community Health Centre, Jawali |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Zonal Hospital, Dharmasala)

[Annexure 5 (a)]

| NBCC : ELECTRICAL WORKS FOR Zonal Hospital, Dharmasala |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Zonal Hospital, Dharmasala)

[Annexure 5 (b)]

| ELECTRICAL WORKS FOR NBCC-OT at Zonal Hospital, Dharmasala |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ESTIMATE |  |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required..(Make: Legrand/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand/Crabtree) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Zonal Hospital, Dharmasala)

[Annexure 5 (c)]

| ELECTRICAL WORKS FOR SCNU AT ZH DHARMASALA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 15 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |
| a | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 150 |  |  |
| b | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 185 |  |  |
| c | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 100 |  |  |
| 3 | A/C point |  |  |  |  |
|  | S/F 20A SPN MCB industrial socket outlet: Supplying and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top alongwith 20 amps "C" curve, SP, MCB, in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket out let and complete with connections, testing and commissioning etc. as required.(Make: Legrand) | Point | 1 |  |  |
| 4 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand) | Each | 15 |  |  |
| 5 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
| a | 3 module | Each | 5 |  |  |
| b | 6 module | Each | 19 |  |  |
| c | 8 module | Each | 1 |  |  |
| 6 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
| a | 5/6 Amps switch | Each | 25 |  |  |
| b | 3 pin 5/6 Amp socket outlet | Each | 25 |  |  |
| c | 15/16 Amps switch | Each | 15 |  |  |
| d | 6pin 15/16 Amps (universal) socket outlet | Each | 15 |  |  |
| e | Telephone socket outlet | Each | 1 |  |  |
| 7 | Telephone wiring in existing conduit: |  |  |  |  |
|  | Supplying and drawing following pair 0.5 sq mm FR PVC insulated annealed copper conductor, unarmored telephone cable in the existing surface/ recessed steel/ PVC conduit as required. |  |  |  |  |
| a | 2 Pair | Metre | 50 |  |  |
| 8 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
| a | 20 mm | Metre | 45 |  |  |
| 9 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 10 |  |  |
| 10 | CABLE LAYING |  |  |  |  |
|  | Laying and fixing of one number PVC insulated and PVC sheathed / XLPE power cable of 1.1kV grade of following size clamped on wall surface with 1mm thick saddles as required complete. |  |  |  |  |
| a | From VDB to LDB |  |  |  |  |
|  | 4X10 sq.mm | Metre | 10 |  |  |
| b | From VDB to PDB |  |  |  |  |
|  | 4X16 sq.mm | Metre | 20 |  |  |
| c | From Panel board to VDB |  |  |  |  |
|  | 3.5X25 sq.mm | Metre | 150 |  |  |
| 11 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of PVC insulated and PVC sheathed/XLPE aluminium conductor cable fo 1.1KV grade as required. |  |  |  |  |
| a | 4X10 sq.mm | Each | 2 |  |  |
| b | 4X16 sq.mm | Each | 2 |  |  |
| c | 3.5X25 sq.mm | Each | 2 |  |  |
| 13 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N) | Each | 1 |  |  |
| 14 | Supply and installation of 2x36W recess mounted, decorative Soft lite luminaire, powder coated CRCA housing complete with all electrical accessories like electronic ballast, lamp holder, 36W CFL lamp etc complete with all mounting accessories like chain, anchor fastner etc as required. (Make Bajaj, ordering product 530909 luminarie type BCLA 236 CFL LHD, lamp type ‘L’ type CFL 2 x 36W or any approved make/PHILIPS/GE/HAVELLS ). | Each | 6 |  |  |
| 15 | Supply & installation of 1x18 watts recess mounted down light fixture, with aluminium reflector, CRCA sheet housing, complete with all accessories like ballast, 18W CFL lamp etc complete as required. (Make: Havells Lamp type:upto 2x18W PL-C, ordering code LHOC33218399 optics general diffuse) | Each | 3 |  |  |
| 16 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way, three pole and neutral, sheet steel, Vertical MCB distribution board, 415 volts, on surface/recess, complete with tinned copper busbar, neutral busbar, earth bar, din bar, detachable gland plate, interconnections, phosphatised and powder painted including earthing etc. as required (with provision for isolator as incomer) (Make: Legrand) |  |  |  |  |
|  | 4 way, double door | Each | 1.00 |  |  |
| 16.1 | S/F of FP Isolators |  |  |  |  |
|  | Supplying and fixing following rating, four pole, (three phase and neutral) 415 volts isolators in the existing Vertical DB as incomer complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 63/100 A, 4P isolator | Each | 1.00 |  |  |
| 16.2 | S/F of TP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 63 amps rating, 415 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 32A TP MCB | Each | 1.00 |  |  |
|  | 63A TP MCB | Each | 1.00 |  |  |
| 17 | S/F of TPN MCB PDB |  |  |  |  |
|  | Supplying and fixing following way, three pole and neutral, sheet steel, MCB distribution board, 415 volts, on surface/recess, complete with tinned copper bus bar, neutral busbar, earth bar, din bar, detachable gland plate, interconnections, phosphatised and powder painted including earthing etc. as required (But without MCB/ RCBO/isolator) (Make: Legrand) |  |  |  |  |
|  | 8 way Double door, horizontal type. | Each | 1.00 |  |  |
| 17.1 | S/F of TPN RCBO |  |  |  |  |
|  | Supplying and fixing following rating, four pole, (three phase and neutral) 415 volts RCBO, having a sensitivity current upto 100/300 milliamperes in the existing MCB PDB complete with connections, testing and commissioning etc. as required.(Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 63 A 4P RCBO 100mA | Each | 1.00 |  |  |
| 17.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for power loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 15A SP MCB | Each | 24.00 |  |  |
| 18 | TPN MCB LDB |  |  |  |  |
|  | Supplying and fixing following way, three pole and neutral, sheet steel, MCB distribution board, 415 volts, on surface/recess, complete with tinned copper bus bar, neutral busbar, earth bar, din bar, detachable gland plate, interconnections, phosphatised and powder painted including earthing etc as required(But without MCB/RCBO /isolator) (Make:Legrand) |  |  |  |  |
|  | 4 way (4+12), double door | Each | 1.00 |  |  |
| 18.1 | S/F of TPN RCBO |  |  |  |  |
|  | Supplying and fixing following rating, four pole, (three phase and neutral) 415 volts RCBO, having a sensitivity current upto 30 milliamperes in the existing MCB LDB complete with connections, testing and commissioning etc. as required.(Make: Legrand) |  |  |  |  |
|  | 32 A 4P RCBO 30mA | Each | 1.00 |  |  |
| 18.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 10 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/10A SP MCB | Each | 12 |  |  |
| 19 | CABLE SUPPLY |  |  |  |  |
|  | Supplying the following PVC insulated and PVC sheathed/XLPE power cable of 1.1 KV grade as per CPWD standards as required. (Make:Havells/ Glostar/Nicco) |  |  |  |  |
| a | 4X10 sq.mm | Metre | 10 |  |  |
| b | 4X16 sq.mm | Metre | 20 |  |  |
| c | 3.5X35 sq.mm | Metre | 125 |  |  |
| 20 | Pipe earthing with G.I. earth pipe 2.5 metre, long, 40 mm dia including accessories, and providing masonry enclosure with cover plate having locking arrangement and watering pipe etc. with charcoal/ coke and salt ) as required complete. | Set | 4 |  |  |
| 21 | Supplying and laying 6 SWG Copper wire on, surface or in recess for loop earthing along with existing surface/recessed conduit/submain wiring/ cable as required. | Metre | 40 |  |  |
| 22 | Supplying and laying 10 SWG Copper wire on, surface or in recess for loop earthing along with existing surface/recessed conduit/submain wiring/ cable as required. | Metre | 80 |  |  |
| 23 | WORK COMPLETION |  |  |  |  |
| 23.1 | Leading, Positioning, Installation, testing and commissioning of 20-25 kVA DG set with all its accessories (including test certificate for CT & Energy meter)as req. complete | LS | 1 |  |  |
| 23.2 | Preparation of all drawings submitting and getting approval from Electrical Inspectorate, submitting of Completion certificates and arranging inspection, getting sanction and energisation certificates from Electrical Inspectorate & HPEB, obtaining power allocation & power connection etc. for commissioning of the whole installation in the premises. all statutory payments will be reimbursed on production of the original reciepts. (Finding the exact Feeder to tap power is under the scope of Contractor) | LS | 1 |  |  |
| 24 | Miscellaneous Items |  |  |  |  |
| 24.1 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
| 25 | Supply and pointing exhaust fan shutter type white colour-9"/10" | Each | 1 |  |  |
| 26 | Supply and Fixing of Calling bell with its all accessories | Each | 2 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Chadihar)

[Annexure 6 (a)]

| NBCC : ELECTRICAL WORKS FOR Community Health Centre, Chadihar |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Chadihar)

[Annexure 6 (b)]

| ELECTRICAL WORKS FOR NBSU AT CHC CHADIHAR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 7 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make Havells) |  |  |  |  |
|  | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 75 |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 90 |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 90 |  |  |
| 3 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
|  | 3 module | Each | 6 |  |  |
|  | 6 module | Each | 13 |  |  |
| 4 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
|  | 5/6 Amps switch | Each | 18 |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 18 |  |  |
|  | 15/16 Amps switch | Each | 9 |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 9 |  |  |
| 5 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
|  | 20/25 mm | Metre | 35 |  |  |
| 6 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 6 |  |  |
| 7 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
|  | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 65 |  |  |
|  | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
|  | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
| 8 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
|  | 2X6 sq.mm | Each | 8 |  |  |
| 9 | Supply and installation of 2x36W wall mounted, decorative Soft lite luminaire, powder coated CRCA housing complete with all electrical accessories like electronic ballast, lamp holder, 36W CFL lamp etc complete with all mounting accessories like chain, anchor fastner etc as required. (Philips / Wipro / GE/HAVELLS/BAJAJ ). | Each | 4 |  |  |
| 10 | Supply & installation of 1x18 watts recess mounted down light fixture, with aluminium reflector, CRCA sheet housing, complete with all accessories like ballast, 18W CFL lamp etc complete as required. (Wipro / Philips FBH145 M 1XPL-C/4P18W HFM 240V / GE | Each | 2 |  |  |
| 11 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 11.a | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 11.b | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 12 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T, Legrand) |  |  |  |  |
|  | 20/32 A COS | Each | 1 |  |  |
| 13 | WORK COMPLETION |  |  |  |  |
|  | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 14 | Miscellaneous Items |  |  |  |  |
|  | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Jaisingpur)

[Annexure 7 (a)]

| NBCC : ELECTRICAL WORKS FOR Community Health Centre, Jaisingpur |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Jaisingpur)

[Annexure 7 (b)]

| ELECTRICAL WORKS FOR NBSU AT CHC JAISINGPUR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 5 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make Havells) |  |  |  |  |
|  | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 70 |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 85 |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 80 |  |  |
| 3 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
|  | 3 module | Each | 6 |  |  |
|  | 6 module | Each | 13 |  |  |
| 4 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
|  | 5/6 Amps switch | Each | 18 |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 18 |  |  |
|  | 15/16 Amps switch | Each | 9 |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 9 |  |  |
| 5 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
|  | 20/25 mm | Metre | 35 |  |  |
| 6 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 5 |  |  |
| 7 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
|  | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 65 |  |  |
|  | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
|  | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
| 8 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
|  | 2X6 sq.mm | Each | 8 |  |  |
| 9 | Supply and installation of 2x36W wall mounted, decorative Soft lite luminaire, powder coated CRCA housing complete with all electrical accessories like electronic ballast, lamp holder, 36W CFL lamp etc complete with all mounting accessories like chain, anchor fastner etc as required. (Philips / Wipro / GE/HAVELLS/BAJAJ ). | Each | 2 |  |  |
| 10 | Supply & installation of 1x18 watts recess mounted down light fixture, with aluminium reflector, CRCA sheet housing, complete with all accessories like ballast, 18W CFL lamp etc complete as required. (Wipro / Philips FBH145 M 1XPL-C/4P18W HFM 240V / GE | Each | 2 |  |  |
| 11 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 11.a | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 11.b | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 12 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T, Legrand) |  |  |  |  |
|  | 20/32 A COS | Each | 1 |  |  |
| 13 | WORK COMPLETION |  |  |  |  |
|  | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 14 | Miscellaneous Items |  |  |  |  |
|  | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Jaisingpur)

[Annexure 7 (c)]

| ELECTRICAL WORKS FOR BSC AT CHC JAISINGPUR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 3 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |
| a | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 25 |  |  |
| b | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 75 |  |  |
| c | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 130 |  |  |
| 3 | A/C point |  |  |  |  |
|  | S/F 20A SPN MCB industrial socket outlet: Supplying and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top alongwith 20 amps "C" curve, SP, MCB, in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket out let and complete with connections, testing and commissioning etc. as required. | Point | 1 |  |  |
| 4 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand) | Each | 8 |  |  |
| 5 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
| a | 3 module | Each | 7 |  |  |
| b | 6 module | Each | 1 |  |  |
| 6 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
| a | 5/6 Amps switch | Each | 4 |  |  |
| b | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |
| c | 15/16 Amps switch | Each | 6 |  |  |
| d | 6pin 15/16 Amps (universal) socket outlet | Each | 6 |  |  |
| 7 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
| a | 20 mm | Metre | 30 |  |  |
| 8 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 2 |  |  |
| 9 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
| a | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 45 |  |  |
| b | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 60 |  |  |
| c | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 30 |  |  |
| 10 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
| a | 2 X 6 sq. mm | Each | 6 |  |  |
| 11 | Supply and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top along with 2 pole and earth, metal enclosed plug top along with 20 amps "C" curve, SP, MCB in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket outlet and complete with connections, testing and commissioning etc as required. | Each | 2 |  |  |
| 12 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 2 |  |  |
| 13 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 13.1 | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 13.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 14 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T,Legrand) |  |  |  |  |
|  | 16/25A COS | Each | 1 |  |  |
| 15 | WORK COMPLETION |  |  |  |  |
| 15.1 | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 15.2 | Obtaining power allocation & power connection etc. for commissioning of the whole installation in the premises. all statutory payments will be reimbursed on production of the original reciepts. | LS | 1 |  |  |
|  | Miscellaneous Items |  |  |  |  |
| 16 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Nagrota Bhagawan)

[Annexure 8 (a)]

| NBCC : ELECTRICAL WORKS FOR Community Health Centre, Nagrota Bhagawan |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Nagrota Bhagawan)

[Annexure 8 (b)]

| ELECTRICAL WORKS FOR BSC AT CHC NARGOTA BAGWAN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 3 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |
| a | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 20 |  |  |
| b | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 75 |  |  |
| c | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 135 |  |  |
| 3 | A/C point |  |  |  |  |
|  | S/F 20A SPN MCB industrial socket outlet: Supplying and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top alongwith 20 amps "C" curve, SP, MCB, in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket out let and complete with connections, testing and commissioning etc. as required.(Make: Legrand) | Point | 1 |  |  |
| 4 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand) | Each | 8 |  |  |
| 5 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
| a | 3 module | Each | 7 |  |  |
| b | 6 module | Each | 1 |  |  |
| 6 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
| a | 5/6 Amps switch | Each | 4 |  |  |
| b | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |
| c | 15/16 Amps switch | Each | 6 |  |  |
| d | 6pin 15/16 Amps (universal) socket outlet | Each | 6 |  |  |
| 7 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
| a | 20 mm | Metre | 30 |  |  |
| 8 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 2 |  |  |
| 9 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
| a | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 40 |  |  |
| b | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 70 |  |  |
| c | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 40 |  |  |
| 10 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
| a | 2 X 6 sq. mm | Each | 6 |  |  |
| 11 | Supply and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top along with 2 pole and earth, metal enclosed plug top along with 20 amps "C" curve, SP, MCB in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket outlet and complete with connections, testing and commissioning etc as required. | Each | 2 |  |  |
| 12 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 2 |  |  |
| 13 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 13.1 | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 13.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 14 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T, Legrand) |  |  |  |  |
|  | 16/25A COS | Each | 1 |  |  |
| 15 | WORK COMPLETION |  |  |  |  |
| 15.1 | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 15.2 | Obtaining power allocation & power connection etc. for commissioning of the whole installation in the premises. all statutory payments will be reimbursed on production of the original reciepts. | LS | 1 |  |  |
|  | Miscellaneous Items |  |  |  |  |
| 16 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Knagra)

[Annexure 9 (a)]

| NBCC : ELECTRICAL WORKS FOR Civil Hospital, Kangra |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Kangra)

[Annexure 9 (b)]

| ELECTRICAL WORKS FOR NBCC-OT at Civil Hospital, Kangra |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ESTIMATE |  |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required..(Make: Legrand/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand/Crabtree) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Kangra)

[Annexure 9 (c)]

| ELECTRICAL WORKS FOR NBSU AT CH KANGRA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 7 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make Havells) |  |  |  |  |
|  | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 75 |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 150 |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 100 |  |  |
| 3 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
|  | 3 module | Each | 6 |  |  |
|  | 6 module | Each | 22 |  |  |
| 4 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
|  | 5/6 Amps switch | Each | 23 |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 23 |  |  |
|  | 15/16 Amps switch | Each | 10 |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 10 |  |  |
| 5 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
|  | 20 mm | Metre | 40 |  |  |
| 6 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 6 |  |  |
| 7 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
|  | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 65 |  |  |
|  | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 30 |  |  |
|  | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
| 8 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
|  | 2X6 sq.mm | Each | 8 |  |  |
| 9 | Supply and installation of 2x36W wall mounted, decorative Soft lite luminaire, powder coated CRCA housing complete with all electrical accessories like electronic ballast, lamp holder, 36W CFL lamp etc complete with all mounting accessories like chain, anchor fastner etc as required. (Philips / Wipro / GE/HAVELLS/BAJAJ ). | Each | 4 |  |  |
| 10 | Supply & installation of 1x18 watts recess mounted down light fixture, with aluminium reflector, CRCA sheet housing, complete with all accessories like ballast, 18W CFL lamp etc complete as required. (Wipro / Philips FBH145 M 1XPL-C/4P18W HFM 240V / GE | Each | 2 |  |  |
| 11 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 11.a | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 11.b | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 12 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T) |  |  |  |  |
|  | 20/32 A COS | Each | 1 |  |  |
| 13 | WORK COMPLETION |  |  |  |  |
|  | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 14 | Miscellaneous Items |  |  |  |  |
|  | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Kangra)

[Annexure 9 (d)]

| ELECTRICAL WORKS FOR BSC AT CH KANGRA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for Havells point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 3 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |
| a | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 20 |  |  |
| b | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 75 |  |  |
| c | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 130 |  |  |
| 3 | A/C point |  |  |  |  |
|  | S/F 20A SPN MCB industrial socket outlet: Supplying and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top alongwith 20 amps "C" curve, SP, MCB, in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket out let and complete with connections, testing and commissioning etc. as required.(Make: Legrand) | Point | 1 |  |  |
| 4 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand) | Each | 8 |  |  |
| 5 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
| a | 3 module | Each | 7 |  |  |
| b | 6 module | Each | 1 |  |  |
| 6 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
| a | 5/6 Amps switch | Each | 4 |  |  |
| b | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |
| c | 15/16 Amps switch | Each | 6 |  |  |
| d | 6pin 15/16 Amps (universal) socket outlet | Each | 6 |  |  |
| 7 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
| a | 20 mm | Metre | 30 |  |  |
| 8 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 2 |  |  |
| 9 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
| a | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 45 |  |  |
| b | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 60 |  |  |
| c | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 25 |  |  |
| 10 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
| a | 2 X 6 sq. mm | Each | 6 |  |  |
| 11 | Supply and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top along with 2 pole and earth, metal enclosed plug top along with 20 amps "C" curve, SP, MCB in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket outlet and complete with connections, testing and commissioning etc as required. | Each | 2 |  |  |
| 12 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 2 |  |  |
| 13 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 13.1 | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 13.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for Havellsing and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 14 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T, Legrand) |  |  |  |  |
|  | 16/25A COS | Each | 1 |  |  |
| 15 | WORK COMPLETION |  |  |  |  |
| 15.1 | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 15.2 | Obtaining power allocation & power connection etc. for commissioning of the whole installation in the premises. all statutory payments will be reimbursed on production of the original reciepts. | LS | 1 |  |  |
|  | Miscellaneous Items |  |  |  |  |
| 16 | Miscellaneous electrical work, removing of existing conduits and interconnections of Havells,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Shahpur)

[Annexure 10 (a)]

| NBCC : ELECTRICAL WORKS FOR Community Health Centre, Shahpur |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Shahpur)

[Annexure 10 (b)]

| ELECTRICAL WORKS FOR NBSU AT CHC SHAPUR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 7 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make Havells) |  |  |  |  |
|  | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 75 |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 125 |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 100 |  |  |
| 3 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
|  | 3 module | Each | 6 |  |  |
|  | 6 module | Each | 22 |  |  |
| 4 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
|  | 5/6 Amps switch | Each | 22 |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 22 |  |  |
|  | 15/16 Amps switch | Each | 10 |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 10 |  |  |
| 5 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
|  | 20 mm | Metre | 40 |  |  |
| 6 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 6 |  |  |
| 7 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
|  | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 65 |  |  |
|  | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 30 |  |  |
|  | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 55 |  |  |
| 8 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
|  | 2X6 sq.mm | Each | 8 |  |  |
| 9 | Supply and installation of 2x36W wall mounted, decorative Soft lite luminaire, powder coated CRCA housing complete with all electrical accessories like electronic ballast, lamp holder, 36W CFL lamp etc complete with all mounting accessories like chain, anchor fastner etc as required. (Philips / Wipro / GE/HAVELLS/BAJAJ ). | Each | 4 |  |  |
| 10 | Supply & installation of 1x18 watts recess mounted down light fixture, with aluminium reflector, CRCA sheet housing, complete with all accessories like ballast, 18W CFL lamp etc complete as required. (Wipro / Philips FBH145 M 1XPL-C/4P18W HFM 240V / GE | Each | 2 |  |  |
| 11 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 11.a | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 11.b | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 12 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T, Legrand) |  |  |  |  |
|  | 20/32 A COS | Each | 1 |  |  |
| 13 | WORK COMPLETION |  |  |  |  |
|  | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 14 | Miscellaneous Items |  |  |  |  |
|  | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Gangath)

[Annexure 11 (a)]

| NBCC : ELECTRICAL WORKS FOR Community Health Centre, Gangath |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Gangath)

[Annexure 11 (b)]

| ELECTRICAL WORKS FOR NBSU AT CHC GANGATH |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 7 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make Havells) |  |  |  |  |
|  | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 70 |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 95 |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 90 |  |  |
| 3 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
|  | 3 module | Each | 6 |  |  |
|  | 6 module | Each | 13 |  |  |
| 4 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
|  | 5/6 Amps switch | Each | 18 |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 18 |  |  |
|  | 15/16 Amps switch | Each | 9 |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 9 |  |  |
| 5 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
|  | 20/25 mm | Metre | 35 |  |  |
| 6 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 5 |  |  |
| 7 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
|  | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 65 |  |  |
|  | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
|  | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
| 8 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
|  | 2X6 sq.mm | Each | 8 |  |  |
| 9 | Supply and installation of 2x36W wall mounted, decorative Soft lite luminaire, powder coated CRCA housing complete with all electrical accessories like electronic ballast, lamp holder, 36W CFL lamp etc complete with all mounting accessories like chain, anchor fastner etc as required. (Philips / Wipro / GE/HAVELLS/BAJAJ ). | Each | 4 |  |  |
| 10 | Supply & installation of 1x18 watts recess mounted down light fixture, with aluminium reflector, CRCA sheet housing, complete with all accessories like ballast, 18W CFL lamp etc complete as required. (Wipro / Philips FBH145 M 1XPL-C/4P18W HFM 240V / GE | Each | 2 |  |  |
| 11 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 11.a | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 11.b | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 12 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T, Legrand) |  |  |  |  |
|  | 20/32 A COS | Each | 1 |  |  |
| 13 | WORK COMPLETION |  |  |  |  |
|  | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 14 | Miscellaneous Items |  |  |  |  |
|  | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Nurpur)

[Annexure 12 (a)]

| NBCC : ELECTRICAL WORKS FOR Civil Hospital, Nurpur |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Nurpur)

[Annexure 12 (b)]

| ELECTRICAL WORKS FOR NBCC-OT at Civil Hospital, Nurpur |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ESTIMATE |  |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required..(Make: Legrand/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand/Crabtree) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Nurpur)

[Annexure 12 (c)]

| ELECTRICAL WORKS FOR SCNU AT CH NURPUR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 15 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |
| a | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 150 |  |  |
| b | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 150 |  |  |
| c | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 140 |  |  |
| 3 | A/C point |  |  |  |  |
|  | S/F 20A SPN MCB industrial socket outlet: Supplying and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top alongwith 20 amps "C" curve, SP, MCB, in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket out let and complete with connections, testing and commissioning etc. as required.(Make: Legrand) | Point | 1 |  |  |
| 4 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand) | Each | 15 |  |  |
| 5 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
| a | 3 module | Each | 5 |  |  |
| b | 6 module | Each | 25 |  |  |
| c | 8 module | Each | 1 |  |  |
| 6 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
| a | 5/6 Amps switch | Each | 34 |  |  |
| b | 3 pin 5/6 Amp socket outlet | Each | 34 |  |  |
| c | 15/16 Amps switch | Each | 20 |  |  |
| d | 6pin 15/16 Amps (universal) socket outlet | Each | 20 |  |  |
| e | Telephone socket outlet | Each | 1 |  |  |
| 7 | Telephone wiring in existing conduit: |  |  |  |  |
|  | Supplying and drawing following pair 0.5 sq mm FR PVC insulated annealed copper conductor, unarmored telephone cable in the existing surface/ recessed steel/ PVC conduit as required. |  |  |  |  |
| a | 2 Pair | Metre | 50 |  |  |
| 8 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
| a | 20 mm | Metre | 45 |  |  |
| 9 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 12 |  |  |
| 10 | CABLE LAYING |  |  |  |  |
|  | Laying and fixing of one number PVC insulated and PVC sheathed / XLPE power cable of 1.1kV grade of following size clamped on wall surface with 1mm thick saddles as required complete. |  |  |  |  |
| a | From VDB to LDB |  |  |  |  |
|  | 4X10 sq.mm | Metre | 10 |  |  |
| b | From VDB to PDB |  |  |  |  |
|  | 4X16 sq.mm | Metre | 20 |  |  |
| c | From Panel board to VDB |  |  |  |  |
|  | 3.5X25 sq.mm | Metre | 150 |  |  |
| 11 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of PVC insulated and PVC sheathed/XLPE aluminium conductor cable fo 1.1KV grade as required. |  |  |  |  |
| a | 4X10 sq.mm | Each | 2 |  |  |
| b | 4X16 sq.mm | Each | 2 |  |  |
| c | 3.5X25 sq.mm | Each | 2 |  |  |
| 12 | Supplying and fixing PVC angle batten/batten holder with 18W CFL(Make:Havells/Philips) including connection as required. | Each | 2 |  |  |
| 13 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/) | Each | 2 |  |  |
| 14 | Supply and installation of 2x36W recess mounted, decorative Soft lite luminaire, powder coated CRCA housing complete with all electrical accessories like electronic ballast, lamp holder, 36W CFL lamp etc complete with all mounting accessories like chain, anchor fastner etc as required. (Make Bajaj, ordering product 530909 luminarie type BCLA 236 CFL LHD, lamp type ‘L’ type CFL 2 x 36W or any approved make/PHILIPS/GE/HAVELLS ). | Each | 6 |  |  |
| 15 | Supply & installation of 1x18 watts recess mounted down light fixture, with aluminium reflector, CRCA sheet housing, complete with all accessories like ballast, 18W CFL lamp etc complete as required. (Make: Havells Lamp type:upto 2x18W PL-C, ordering code LHOC33218399 optics general diffuse) | Each | 3 |  |  |
| 16 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way, three pole and neutral, sheet steel, Vertical MCB distribution board, 415 volts, on surface/recess, complete with tinned copper busbar, neutral busbar, earth bar, din bar, detachable gland plate, interconnections, phosphatised and powder painted including earthing etc. as required (with provision for isolator as incomer) (Make: Legrand) |  |  |  |  |
|  | 4 way, double door | Each | 1.00 |  |  |
| 16.1 | S/F of FP Isolators |  |  |  |  |
|  | Supplying and fixing following rating, four pole, (three phase and neutral) 415 volts isolators in the existing Vertical DB as incomer complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 63/100 A, 4P isolator | Each | 1.00 |  |  |
| 16.2 | S/F of TP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 63 amps rating, 415 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 32A TP MCB | Each | 1.00 |  |  |
|  | 63A TP MCB | Each | 1.00 |  |  |
| 17 | S/F of TPN MCB PDB |  |  |  |  |
|  | Supplying and fixing following way, three pole and neutral, sheet steel, MCB distribution board, 415 volts, on surface/recess, complete with tinned copper bus bar, neutral busbar, earth bar, din bar, detachable gland plate, interconnections, phosphatised and powder painted including earthing etc. as required (But without MCB/ RCBO/isolator) (Make: Legrand) |  |  |  |  |
|  | 8 way Double door, horizontal type. | Each | 1.00 |  |  |
| 17.1 | S/F of TPN RCBO |  |  |  |  |
|  | Supplying and fixing following rating, four pole, (three phase and neutral) 415 volts RCBO, having a sensitivity current upto 100/300 milliamperes in the existing MCB PDB complete with connections, testing and commissioning etc. as required.(Make: Legrand) |  |  |  |  |
|  | 63 A 4P RCBO 100mA | Each | 1.00 |  |  |
| 17.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for power loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 15A SP MCB | Each | 24.00 |  |  |
| 18 | TPN MCB LDB |  |  |  |  |
|  | Supplying and fixing following way, three pole and neutral, sheet steel, MCB distribution board, 415 volts, on surface/recess, complete with tinned copper bus bar, neutral busbar, earth bar, din bar, detachable gland plate, interconnections, phosphatised and powder painted including earthing etc as required(But without MCB/RCBO /isolator) (Make:Legrand) |  |  |  |  |
|  | 4 way (4+12), double door | Each | 1.00 |  |  |
| 18.1 | S/F of TPN RCBO |  |  |  |  |
|  | Supplying and fixing following rating, four pole, (three phase and neutral) 415 volts RCBO, having a sensitivity current upto 30 milliamperes in the existing MCB LDB complete with connections, testing and commissioning etc. as required.(Make: Legrand) |  |  |  |  |
|  | 32 A 4P RCBO 30mA | Each | 1.00 |  |  |
| 18.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 10 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/10A SP MCB | Each | 12 |  |  |
| 19 | CABLE SUPPLY |  |  |  |  |
|  | Supplying the following PVC insulated and PVC sheathed/XLPE power cable of 1.1 KV grade as per CPWD standards as required. (Make:Havells/ Glostar/Nicco) |  |  |  |  |
| a | 4X10 sq.mm | Metre | 10 |  |  |
| b | 4X16 sq.mm | Metre | 20 |  |  |
| c | 3.5X25 sq.mm | Metre | 125 |  |  |
| 20 | Pipe earthing with G.I. earth pipe 2.5 metre, long, 40 mm dia including accessories, and providing masonry enclosure with cover plate having locking arrangement and watering pipe etc. with charcoal/ coke and salt ) as required complete. | Set | 4 |  |  |
| 21 | Supplying and laying 6 SWG Copper wire on, surface or in recess for loop earthing along with existing surface/recessed conduit/submain wiring/ cable as required. | Metre | 40 |  |  |
| 22 | Supplying and laying 10 SWG Copper wire on, surface or in recess for loop earthing along with existing surface/recessed conduit/submain wiring/ cable as required. | Metre | 80 |  |  |
| 23 | WORK COMPLETION |  |  |  |  |
| 23.1 | Leading, Positioning, Installation, testing and commissioning of 20-25 kVA DG set with all its accessories (including test certificate for CT & Energy meter)as req. complete | LS | 1 |  |  |
| 23.2 | Preparation of all drawings submitting and getting approval from Electrical Inspectorate, submitting of Completion certificates and arranging inspection, getting sanction and energisation certificates from Electrical Inspectorate & HPEB, obtaining power allocation & power connection etc. for commissioning of the whole installation in the premises. all statutory payments will be reimbursed on production of the original reciepts. (Finding the exact Feeder to tap power is under the scope of Contractor) | LS | 1 |  |  |
| 24 | Miscellaneous Items |  |  |  |  |
| 24.1 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
| 25 | Supply and pointing exhaust fan shutter type white colour-9"/10" | Each | 1 |  |  |
| 26 | Supply and Fixing of Calling bell with its all accessories | Each | 2 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Nurpur)

[Annexure 12 (d)]

| ELECTRICAL WORKS FOR BSC AT CH NURPUR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for Havells point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 3 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |
| a | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 25 |  |  |
| b | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 75 |  |  |
| c | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 125 |  |  |
| 3 | A/C point |  |  |  |  |
|  | S/F 20A SPN MCB industrial socket outlet: Supplying and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top alongwith 20 amps "C" curve, SP, MCB, in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket out let and complete with connections, testing and commissioning etc. as required.(Make: Legrand) | Point | 1 |  |  |
| 4 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand) | Each | 8 |  |  |
| 5 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
| a | 3 module | Each | 7 |  |  |
| b | 6 module | Each | 1 |  |  |
| 6 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
| a | 5/6 Amps switch | Each | 4 |  |  |
| b | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |
| c | 15/16 Amps switch | Each | 6 |  |  |
| d | 6pin 15/16 Amps (universal) socket outlet | Each | 6 |  |  |
| 7 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
| a | 20 mm | Metre | 30 |  |  |
| 8 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 2 |  |  |
| 9 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
| a | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
| b | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 60 |  |  |
| c | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 25 |  |  |
| 10 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
| a | 2 X 6 sq. mm | Each | 6 |  |  |
| 11 | Supply and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top along with 2 pole and earth, metal enclosed plug top along with 20 amps "C" curve, SP, MCB in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket outlet and complete with connections, testing and commissioning etc as required. | Each | 2 |  |  |
| 12 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 2 |  |  |
| 13 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 13.1 | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 13.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for Havellsing and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 14 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T, legrand) |  |  |  |  |
|  | 16/25A COS | Each | 1 |  |  |
| 15 | WORK COMPLETION |  |  |  |  |
| 15.1 | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 15.2 | Obtaining power allocation & power connection etc. for commissioning of the whole installation in the premises. all statutory payments will be reimbursed on production of the original reciepts. | LS | 1 |  |  |
|  | Miscellaneous Items |  |  |  |  |
| 16 | Miscellaneous electrical work, removing of existing conduits and interconnections of Havells,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Palampur)

[Annexure 13 (a)]

| NBCC : ELECTRICAL WORKS FOR Civil Hospital, Palampur |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Palampur)

[Annexure 13 (b)]

| ELECTRICAL WORKS FOR NBCC-OT at civil Hospital, Palampur |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ESTIMATE |  |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required..(Make: Legrand/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand/Crabtree) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Palampur)

[Annexure 13 (c)]

| ELECTRICAL WORKS FOR SCNU ATCH PALAMPUR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 17 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make Havells) |  |  |  |  |
| a | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 150 |  |  |
| b | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 175 |  |  |
| c | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 150 |  |  |
| 3 | A/C point |  |  |  |  |
|  | S/F 20A SPN MCB industrial socket outlet: Supplying and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top alongwith 20 amps "C" curve, SP, MCB, in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket out let and complete with connections, testing and commissioning etc. as required.(Make Legrand/Havells)(Make: LEgrand) | Point | 1 |  |  |
| 4 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand) | Each | 15 |  |  |
| 5 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
| a | 3 module | Each | 5 |  |  |
| b | 6 module | Each | 22 |  |  |
| c | 8 module | Each | 1 |  |  |
| 6 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
| a | 5/6 Amps switch | Each | 28 |  |  |
| b | 3 pin 5/6 Amp socket outlet | Each | 28 |  |  |
| c | 15/16 Amps switch | Each | 15 |  |  |
| d | 6pin 15/16 Amps (universal) socket outlet | Each | 15 |  |  |
| e | Telephone socket outlet | Each | 1 |  |  |
| 7 | Telephone wiring in existing conduit: |  |  |  |  |
|  | Supplying and drawing following pair 0.5 sq mm FR PVC insulated annealed copper conductor, unarmored telephone cable in the existing surface/ recessed steel/ PVC conduit as required. |  |  |  |  |
| a | 2 Pair | Metre | 50 |  |  |
| 8 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
| a | 20 mm | Metre | 45 |  |  |
| 9 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 12 |  |  |
| 10 | CABLE LAYING |  |  |  |  |
|  | Laying and fixing of one number PVC insulated and PVC sheathed / XLPE power cable of 1.1kV grade of following size clamped on wall surface with 1mm thick saddles as required complete. |  |  |  |  |
| a | From VDB to LDB |  |  |  |  |
|  | 4X10 sq.mm | Metre | 10 |  |  |
| b | From VDB to PDB |  |  |  |  |
|  | 4X16 sq.mm | Metre | 20 |  |  |
| c | From Panel board to VDB |  |  |  |  |
|  | 3.5X25 sq.mm | Metre | 150 |  |  |
| 11 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of PVC insulated and PVC sheathed/XLPE aluminium conductor cable fo 1.1KV grade as required. |  |  |  |  |
| a | 4X10 sq.mm | Each | 2 |  |  |
| b | 4X16 sq.mm | Each | 2 |  |  |
| c | 3.5X25 sq.mm | Each | 2 |  |  |
| 12 | Supply and installation of 2x36W recess mounted, decorative Soft lite luminaire, powder coated CRCA housing complete with all electrical accessories like electronic ballast, lamp holder, 36W CFL lamp etc complete with all mounting accessories like chain, anchor fastner etc as required. (Make Bajaj, ordering product 530909 luminarie type BCLA 236 CFL LHD, lamp type ‘L’ type CFL 2 x 36W or any approved make/PHILIPS/GE/HAVELLS ). | Each | 7 |  |  |
| 13 | Supply & installation of 1x18 watts recess mounted down light fixture, with aluminium reflector, CRCA sheet housing, complete with all accessories like ballast, 18W CFL lamp etc complete as required. (Make: Havells Lamp type:upto 2x18W PL-C, ordering code LHOC33218399 optics general diffuse) | Each | 3 |  |  |
| 14 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way, three pole and neutral, sheet steel, Vertical MCB distribution board, 415 volts, on surface/recess, complete with tinned copper busbar, neutral busbar, earth bar, din bar, detachable gland plate, interconnections, phosphatised and powder painted including earthing etc. as required (with provision for isolator as incomer) (Make: Legrand) |  |  |  |  |
|  | 4 way, double door | Each | 1.00 |  |  |
| 14.1 | S/F of FP Isolators |  |  |  |  |
|  | Supplying and fixing following rating, four pole, (three phase and neutral) 415 volts isolators in the existing Vertical DB as incomer complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 63/100 A, 4P isolator | Each | 1.00 |  |  |
| 14.2 | S/F of TP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 63 amps rating, 415 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 32A TP MCB | Each | 1.00 |  |  |
|  | 63A TP MCB | Each | 1.00 |  |  |
| 15 | S/F of TPN MCB PDB |  |  |  |  |
|  | Supplying and fixing following way, three pole and neutral, sheet steel, MCB distribution board, 415 volts, on surface/recess, complete with tinned copper bus bar, neutral busbar, earth bar, din bar, detachable gland plate, interconnections, phosphatised and powder painted including earthing etc. as required (But without MCB/ RCBO/isolator) (Make: Legrand) |  |  |  |  |
|  | 8 way Double door, horizontal type. | Each | 1.00 |  |  |
| 15.1 | S/F of TPN RCBO |  |  |  |  |
|  | Supplying and fixing following rating, four pole, (three phase and neutral) 415 volts RCBO, having a sensitivity current upto 100/300 milliamperes in the existing MCB PDB complete with connections, testing and commissioning etc. as required.(Make: Legrand) |  |  |  |  |
|  | 63 A 4P RCBO 100mA | Each | 1.00 |  |  |
| 15.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for power loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 15A SP MCB | Each | 24.00 |  |  |
| 16 | TPN MCB LDB |  |  |  |  |
|  | Supplying and fixing following way, three pole and neutral, sheet steel, MCB distribution board, 415 volts, on surface/recess, complete with tinned copper bus bar, neutral busbar, earth bar, din bar, detachable gland plate, interconnections, phosphatised and powder painted including earthing etc as required(But without MCB/RCBO /isolator) (Make:Legrand) |  |  |  |  |
|  | 4 way (4+12), double door | Each | 1.00 |  |  |
| 16.1 | S/F of TPN RCBO |  |  |  |  |
|  | Supplying and fixing following rating, four pole, (three phase and neutral) 415 volts RCBO, having a sensitivity current upto 30 milliamperes in the existing MCB LDB complete with connections, testing and commissioning etc. as required.(Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 32 A 4P RCBO 30mA | Each | 1.00 |  |  |
| 16.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 10 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/10A SP MCB | Each | 12 |  |  |
| 17 | CABLE SUPPLY |  |  |  |  |
|  | Supplying the following PVC insulated and PVC sheathed/XLPE power cable of 1.1 KV grade as per CPWD standards as required. (Make:Havells/ Glostar/Nicco) |  |  |  |  |
| a | 4X10 sq.mm | Metre | 10 |  |  |
| b | 4X16 sq.mm | Metre | 20 |  |  |
| c | 3.5X35 sq.mm | Metre | 125 |  |  |
| 18 | Pipe earthing with G.I. earth pipe 2.5 metre, long, 40 mm dia including accessories, and providing masonry enclosure with cover plate having locking arrangement and watering pipe etc. with charcoal/ coke and salt ) as required complete. | Set | 4 |  |  |
| 19 | Supplying and laying 6 SWG Copper wire on, surface or in recess for loop earthing along with existing surface/recessed conduit/submain wiring/ cable as required. | Metre | 40 |  |  |
| 20 | Supplying and laying 10 SWG Copper wire on, surface or in recess for loop earthing along with existing surface/recessed conduit/submain wiring/ cable as required. | Metre | 80 |  |  |
| 21 | WORK COMPLETION |  |  |  |  |
| 21.1 | Leading, Positioning, Installation, testing and commissioning of 20-25 kVA DG set with all its accessories (including test certificate for CT & Energy meter)as req. complete | LS | 1 |  |  |
| 21.2 | Preparation of all drawings submitting and getting approval from Electrical Inspectorate, submitting of Completion certificates and arranging inspection, getting sanction and energisation certificates from Electrical Inspectorate & HPEB, obtaining power allocation & power connection etc. for commissioning of the whole installation in the premises. all statutory payments will be reimbursed on production of the original reciepts. (Finding the exact Feeder to tap power is under the scope of Contractor) | LS | 1 |  |  |
| 22 | Miscellaneous Items |  |  |  |  |
| 22.1 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
| 23 | Supply and pointing exhaust fan shutter type white colour-9"/10" | Each | 1 |  |  |
| 24 | Supply and Fixing of Calling bell with its all accessories | Each | 2 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Baijnath)

[Annexure 14 (a)]

| NBCC : ELECTRICAL WORKS FOR Civil Hospital, Baijnath |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Baijnath)

[Annexure 14 (b)]

| ELECTRICAL WORKS FOR NBSU AT CH BAIJNATH |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 7 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make Havells) |  |  |  |  |
|  | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 75 |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 175 |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 110 |  |  |
| 3 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
|  | 3 module | Each | 6 |  |  |
|  | 6 module | Each | 22 |  |  |
| 4 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
|  | 5/6 Amps switch | Each | 22 |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 22 |  |  |
|  | 15/16 Amps switch | Each | 10 |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 10 |  |  |
| 5 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
|  | 20 mm | Metre | 40 |  |  |
| 6 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 6 |  |  |
| 7 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
|  | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 65 |  |  |
|  | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 30 |  |  |
|  | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
| 8 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
|  | 2X6 sq.mm | Each | 8 |  |  |
| 9 | Supply and installation of 2x36W wall mounted, decorative Soft lite luminaire, powder coated CRCA housing complete with all electrical accessories like electronic ballast, lamp holder, 36W CFL lamp etc complete with all mounting accessories like chain, anchor fastner etc as required. (Philips / Wipro / GE/HAVELLS/BAJAJ ). | Each | 4 |  |  |
| 10 | Supply & installation of 1x18 watts recess mounted down light fixture, with aluminium reflector, CRCA sheet housing, complete with all accessories like ballast, 18W CFL lamp etc complete as required. (Wipro / Philips FBH145 M 1XPL-C/4P18W HFM 240V / GE ) | Each | 2 |  |  |
| 11 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 11.a | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 11.b | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 12 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T) |  |  |  |  |
|  | 20/32 A COS | Each | 1 |  |  |
| 13 | WORK COMPLETION |  |  |  |  |
|  | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 14 | Miscellaneous Items |  |  |  |  |
|  | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Baijnath)

[Annexure 14 (c)]

| ELECTRICAL WORKS FOR BSC AT CH BAIJNATH |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 3 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make Havells) |  |  |  |  |
| a | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 25 |  |  |
| b | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 75 |  |  |
| c | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 125 |  |  |
| 3 | A/C point |  |  |  |  |
|  | S/F 20A SPN MCB industrial socket outlet: Supplying and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top alongwith 20 amps "C" curve, SP, MCB, in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket out let and complete with connections, testing and commissioning etc. as required.(Make: Legrand) | Point | 1 |  |  |
| 4 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Havells) | Each | 8 |  |  |
| 5 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
| a | 3 module | Each | 7 |  |  |
| b | 6 module | Each | 1 |  |  |
| 6 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
| a | 5/6 Amps switch | Each | 4 |  |  |
| b | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |
| c | 15/16 Amps switch | Each | 6 |  |  |
| d | 6pin 15/16 Amps (universal) socket outlet | Each | 6 |  |  |
| 7 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
| a | 20 mm | Metre | 30 |  |  |
| 8 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 2 |  |  |
| 9 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
| a | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 45 |  |  |
| b | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 60 |  |  |
| c | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 25 |  |  |
| 10 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
| a | 2 X 6 sq. mm | Each | 6 |  |  |
| 11 | Supply and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top along with 2 pole and earth, metal enclosed plug top along with 20 amps "C" curve, SP, MCB in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket outlet and complete with connections, testing and commissioning etc as required. | Each | 2 |  |  |
| 12 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 2 |  |  |
| 13 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 13.1 | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP ISOLATOR | Each | 1.00 |  |  |
| 13.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 14 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T, Legrand) |  |  |  |  |
|  | 16/25A COS | Each | 1 |  |  |
| 15 | WORK COMPLETION |  |  |  |  |
| 15.1 | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 15.2 | Obtaining power allocation & power connection etc. for commissioning of the whole installation in the premises. all statutory payments will be reimbursed on production of the original reciepts. | LS | 1 |  |  |
|  | Miscellaneous Items |  |  |  |  |
| 16 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Thurnal)

[Annexure 15 (a)]

| NBCC : ELECTRICAL WORKS FOR Civil Hospital, Thurnal |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Thurnal)

[Annexure 15 (b)]

| ELECTRICAL WORKS FOR NBSU AT CH THURNAL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 6 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make Havells) |  |  |  |  |
|  | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 70 |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 90 |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 85 |  |  |
| 3 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
|  | 3 module | Each | 6 |  |  |
|  | 6 module | Each | 12 |  |  |
| 4 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
|  | 5/6 Amps switch | Each | 18 |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 18 |  |  |
|  | 15/16 Amps switch | Each | 9 |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 9 |  |  |
| 5 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
|  | 20/25 mm | Metre | 35 |  |  |
| 6 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 5 |  |  |
| 7 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
|  | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 65 |  |  |
|  | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
|  | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
| 8 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
|  | 2X6 sq.mm | Each | 8 |  |  |
| 9 | Supply and installation of 2x36W wall mounted, decorative Soft lite luminaire, powder coated CRCA housing complete with all electrical accessories like electronic ballast, lamp holder, 36W CFL lamp etc complete with all mounting accessories like chain, anchor fastner etc as required. (Philips / Wipro / GE/HAVELLS/BAJAJ ). | Each | 3 |  |  |
| 10 | Supply & installation of 1x18 watts recess mounted down light fixture, with aluminium reflector, CRCA sheet housing, complete with all accessories like ballast, 18W CFL lamp etc complete as required. (Wipro / Philips FBH145 M 1XPL-C/4P18W HFM 240V / GE | Each | 2 |  |  |
| 11 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 11.a | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 11.b | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 12 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T, legrand) |  |  |  |  |
|  | 20/32 A COS | Each | 1 |  |  |
| 13 | WORK COMPLETION |  |  |  |  |
|  | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 14 | Miscellaneous Items |  |  |  |  |
|  | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Nagrota Surain)

[Annexure 16 (a)]

| NBCC : ELECTRICAL WORKS FOR Community Health Centre, Nagrota Surain |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Nagrota Surain)

[Annexure 16 (b)]

| ELECTRICAL WORKS FOR BSC AT CHC NARGOTA SURIAN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 3 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |
| a | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 25 |  |  |
| b | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 75 |  |  |
| c | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 135 |  |  |
| 3 | A/C point |  |  |  |  |
|  | S/F 20A SPN MCB industrial socket outlet: Supplying and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top alongwith 20 amps "C" curve, SP, MCB, in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket out let and complete with connections, testing and commissioning etc. as required.(Make: Legrand) | Point | 1 |  |  |
| 4 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand) | Each | 8 |  |  |
| 5 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
| a | 3 module | Each | 7 |  |  |
| b | 6 module | Each | 1 |  |  |
| 6 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
| a | 5/6 Amps switch | Each | 4 |  |  |
| b | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |
| c | 15/16 Amps switch | Each | 6 |  |  |
| d | 6pin 15/16 Amps (universal) socket outlet | Each | 6 |  |  |
| 7 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
| a | 20 mm | Metre | 30 |  |  |
| 8 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 2 |  |  |
| 9 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
| a | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
| b | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 60 |  |  |
| c | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 35 |  |  |
| 10 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
| a | 2 X 6 sq. mm | Each | 6 |  |  |
| 11 | Supply and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top along with 2 pole and earth, metal enclosed plug top along with 20 amps "C" curve, SP, MCB in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket outlet and complete with connections, testing and commissioning etc as required. | Each | 2 |  |  |
| 12 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 2 |  |  |
| 13 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 13.1 | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 13.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 14 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T, Legrand) |  |  |  |  |
|  | 16/25A COS | Each | 1 |  |  |
| 15 | WORK COMPLETION |  |  |  |  |
| 15.1 | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 15.2 | Obtaining power allocation & power connection etc. for commissioning of the whole installation in the premises. all statutory payments will be reimbursed on production of the original reciepts. | LS | 1 |  |  |
|  | Miscellaneous Items |  |  |  |  |
| 16 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Dehara)

[Annexure 17 (a)]

| NBCC : ELECTRICAL WORKS FOR Civil Hospital, Dehara |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Dehara)

[Annexure 17 (b)]

| ELECTRICAL WORKS FOR NBCC-OT at Civil Hospital, Dehara |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ESTIMATE |  |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required..(Make: Legrand/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand/Crabtree) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Dehara)

[Annexure 17 (c)]

| ELECTRICAL WORKS FOR NBSU AT CH DEHRA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 6 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make Havells) |  |  |  |  |
|  | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 70 |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 90 |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 85 |  |  |
| 3 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
|  | 3 module | Each | 6 |  |  |
|  | 6 module | Each | 24 |  |  |
| 4 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
|  | 5/6 Amps switch | Each | 18 |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 18 |  |  |
|  | 15/16 Amps switch | Each | 9 |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 9 |  |  |
| 5 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
|  | 20/25 mm | Metre | 35 |  |  |
| 6 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 5 |  |  |
| 7 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
|  | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 65 |  |  |
|  | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
|  | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
| 8 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
|  | 2X6 sq.mm | Each | 8 |  |  |
| 9 | Supply and installation of 2x36W wall mounted, decorative Soft lite luminaire, powder coated CRCA housing complete with all electrical accessories like electronic ballast, lamp holder, 36W CFL lamp etc complete with all mounting accessories like chain, anchor fastner etc as required. (Philips / Wipro / GE/HAVELLS/BAJAJ ). | Each | 3 |  |  |
| 10 | Supply & installation of 1x18 watts recess mounted down light fixture, with aluminium reflector, CRCA sheet housing, complete with all accessories like ballast, 18W CFL lamp etc complete as required. (Wipro / Philips FBH145 M 1XPL-C/4P18W HFM 240V / GE | Each | 2 |  |  |
| 11 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 11.a | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 11.b | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 12 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T) |  |  |  |  |
|  | 20/32 A COS | Each | 1 |  |  |
| 13 | WORK COMPLETION |  |  |  |  |
|  | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 14 | Miscellaneous Items |  |  |  |  |
|  | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | Total Rs: |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Civil Hospital, Dehara)

[Annexure 17 (d)]

| ELECTRICAL WORKS FOR BSC AT CH DEHRA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 3 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |
| a | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 25 |  |  |
| b | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 75 |  |  |
| c | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 130 |  |  |
| 3 | A/C point |  |  |  |  |
|  | S/F 20A SPN MCB industrial socket outlet: Supplying and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top alongwith 20 amps "C" curve, SP, MCB, in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket out let and complete with connections, testing and commissioning etc. as required.(Make: Legrand) | Point | 1 |  |  |
| 4 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand) | Each | 8 |  |  |
| 5 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
| a | 3 module | Each | 7 |  |  |
| b | 6 module | Each | 1 |  |  |
| 6 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
| a | 5/6 Amps switch | Each | 4 |  |  |
| b | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |
| c | 15/16 Amps switch | Each | 6 |  |  |
| d | 6pin 15/16 Amps (universal) socket outlet | Each | 6 |  |  |
| 7 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
| a | 20 mm | Metre | 30 |  |  |
| 8 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 2 |  |  |
| 9 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
| a | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
| b | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 60 |  |  |
| c | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 30 |  |  |
| 10 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
| a | 2 X 6 sq. mm | Each | 6 |  |  |
| 11 | Supply and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top along with 2 pole and earth, metal enclosed plug top along with 20 amps "C" curve, SP, MCB in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket outlet and complete with connections, testing and commissioning etc as required. | Each | 2 |  |  |
| 12 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 2 |  |  |
| 13 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 13.1 | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 13.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 14 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T, Legrand) |  |  |  |  |
|  | 16/25A COS | Each | 1 |  |  |
| 15 | WORK COMPLETION |  |  |  |  |
| 15.1 | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 15.2 | Obtaining power allocation & power connection etc. for commissioning of the whole installation in the premises. all statutory payments will be reimbursed on production of the original reciepts. | LS | 1 |  |  |
|  | Miscellaneous Items |  |  |  |  |
| 16 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Jawalamukhi)

[Annexure 18 (a)]

| NBCC : ELECTRICAL WORKS FOR Community Health Centre, Jawalamukhi |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |  |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |  |
|  | Group A | Point | 1.00 |  |  |  |
| 2 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 1.00 |  |  |  |
| 3 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 1.00 |  |  |  |
| 4 | Supplying and fixing DP sheet steel enclosure on surface/ recess along with 25/32amps 240 volts “C” curve DP MCB complete with connections, testing and commissioning etc. as required.. (Make: Legrand/Havells/HPL) | Each | 2.00 |  |  |  |
| 5 | Wiring through PVC conduit |  |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 20 |  |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 20 |  |  |  |
| 6 | Modular type blanking plate: |  |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand/Crabtree) | Each | 3 |  |  |  |
| 7 | Modular boxes, bases & cover plate: |  |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required. (Make: Legrand) |  |  |  |  |  |
|  | 3 module | Each | 1 |  |  |  |
|  | 6mod | Each | 1 |  |  |  |
| 8 | Modular type switch/socket: |  |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |  |
|  | 5/6 Amps switch | Each | 4 |  |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |  |
|  | 15/16 Amps switch | Each | 1 |  |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 1 |  |  |  |
| 9 | S/F PVC conduit: |  |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |  |
|  | 20 mm | Metre | 20 |  |  |  |
| 10 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |  |
|  | TOTAL Rs: |  |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Jawalamukhi)

[Annexure 18 (b)]

| ELECTRICAL WORKS FOR NBSU AT CHC JWALMUKHI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 7 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make Havells) |  |  |  |  |
|  | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 75 |  |  |
|  | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 175 |  |  |
|  | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 100 |  |  |
| 3 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
|  | 3 module | Each | 6 |  |  |
|  | 6 module | Each | 23 |  |  |
| 4 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
|  | 5/6 Amps switch | Each | 22 |  |  |
|  | 3 pin 5/6 Amp socket outlet | Each | 22 |  |  |
|  | 15/16 Amps switch | Each | 10 |  |  |
|  | 6pin 15/16 Amps (universal) socket outlet | Each | 10 |  |  |
| 5 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
|  | 20 mm | Metre | 45 |  |  |
| 6 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 6 |  |  |
| 7 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
|  | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 65 |  |  |
|  | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 30 |  |  |
|  | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 50 |  |  |
| 8 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
|  | 2X6 sq.mm | Each | 8 |  |  |
| 9 | Supply and installation of 2x36W wall mounted, decorative Soft lite luminaire, powder coated CRCA housing complete with all electrical accessories like electronic ballast, lamp holder, 36W CFL lamp etc complete with all mounting accessories like chain, anchor fastner etc as required. (Philips / Wipro / GE/HAVELLS/BAJAJ ). | Each | 4 |  |  |
| 10 | Supply & installation of 1x18 watts recess mounted down light fixture, with aluminium reflector, CRCA sheet housing, complete with all accessories like ballast, 18W CFL lamp etc complete as required. (Wipro / Philips FBH145 M 1XPL-C/4P18W HFM 240V / GE | Each | 2 |  |  |
| 11 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 11.a | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 11.b | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 12 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T, Legrand) |  |  |  |  |
|  | 20/32 A COS | Each | 1 |  |  |
| 13 | WORK COMPLETION |  |  |  |  |
|  | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 14 | Miscellaneous Items |  |  |  |  |
|  | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT

HLL LIFECARE LIMITED

(A GOVT. OF INDIA ENTERPRISE)

AKKULAM  FACTORY, SREEKARIAM  P.O.

THIRUVANANTHAPURAM – 17

Phone:  +91 471 244 5930, Fax: +91 471 244 5935

Email: tradingpaft@lifecarehll.com

IFB NO: HLL/AFT/TDG/HP-II/ELECTRICAL/2014-15/II-015                                                          16/04/2014

## PRICE SCHEDULE

## (Community Health Centre, Jawalamukhi)

[Annexure 18 (c)]

| ELECTRICAL WORKS FOR BSC AT CHC JWALAMUKHI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCHEDULE |  |  |  |  |  |
| Sl.No | DESCRIPTION | UNIT | QTY | RATE | AMOUNT |
| 1 | Point wiring in PVC conduit with modular type switch. |  |  |  |  |
|  | Wiring for light point/fan point/exhaust fan point /call bell point with 1.5 sq mm FR PVC insulated copper conductor single core cable in surface/recessed medium class PVC conduit, with modular switch, modular plate, suitable PVC box and earthing the point with 1.5 sq.mm FR PVC insulated copper conductor single core cable etc as required. |  |  |  |  |
|  | Group C | Point | 3 |  |  |
| 2 | Wiring through PVC conduit |  |  |  |  |
|  | Wiring for circuit/submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/recessed medium class PVC conduit as required.(Make: Havells) |  |  |  |  |
| a | 2X1.5 sq.mm + 1X1.5 sq.mm earth wire | Metre | 25 |  |  |
| b | 2X2.5 sq.mm + 1 X2.5 sq mm earth wire | Metre | 80 |  |  |
| c | 2X4 sq.mm + 1X4 sq.mm earth wire | Metre | 135 |  |  |
| 3 | A/C point |  |  |  |  |
|  | S/F 20A SPN MCB industrial socket outlet: Supplying and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top alongwith 20 amps "C" curve, SP, MCB, in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket out let and complete with connections, testing and commissioning etc. as required.(Make: Legrand) | Point | 1 |  |  |
| 4 | Modular type blanking plate: |  |  |  |  |
|  | Supplying and fixing modular blanking plate on the existing modular plate & switch box excluding modular plate as required. (Make: Legrand) | Each | 8 |  |  |
| 5 | Modular boxes, bases & cover plate: |  |  |  |  |
|  | Supplying and fixing following size/modules, PVC box along with modular base & cover plate for modular switches in recess etc as required.(Make: Legrand) |  |  |  |  |
| a | 3 module | Each | 7 |  |  |
| b | 6 module | Each | 1 |  |  |
| 6 | Modular type switch/socket: |  |  |  |  |
|  | Supplying and fixing following modular switch/socket on the existing modular plate & switch box including connections but excluding modular plate etc as required.(Make: Legrand) |  |  |  |  |
| a | 5/6 Amps switch | Each | 4 |  |  |
| b | 3 pin 5/6 Amp socket outlet | Each | 4 |  |  |
| c | 15/16 Amps switch | Each | 6 |  |  |
| d | 6pin 15/16 Amps (universal) socket outlet | Each | 6 |  |  |
| 7 | S/F PVC conduit: |  |  |  |  |
|  | Supplying and fixing of following sizes of medium class PVC conduit along with accessories in surface/recess including cutting the wall and making good the same in case of recessed conduit as required. |  |  |  |  |
| a | 20 mm | Metre | 30 |  |  |
| 8 | ITC Fluorescent fitting directly on surface: Installation, testing and commissioning of pre-wired, fluorescent fitting / compact fluorescent fitting of all types, complete with all accessories and tube etc. directly on ceiling/ wall, including connection with 1.5 sq. mm FR PVC insulated, copper conductor, single core cable and earthing etc. as required. | Each | 2 |  |  |
| 9 | CABLE LAYING |  |  |  |  |
|  | Wiring for circuit/ submain wiring along with earth wire with the following sizes of FR PVC insulated copper conductor, single core cable in surface/ recessed medium class PVC conduit as required. |  |  |  |  |
| a | From EB to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 45 |  |  |
| b | From DG to COS |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 75 |  |  |
| c | From COS to DB |  |  |  |  |
|  | 2 X 6 sq. mm + 1 X 6 sq. mm earth wire | Metre | 25 |  |  |
| 10 | END TERMINATION |  |  |  |  |
|  | Supply and making end termination with brass compression gland, earth clip and aluminium lugs for following size of cables as required. |  |  |  |  |
| a | 2 X 6 sq. mm | Each | 6 |  |  |
| 11 | Supply and fixing 20 amps, 240 volts, SPN industrial type, socket outlet, with 2 pole and earth, metal enclosed plug top along with 2 pole and earth, metal enclosed plug top along with 20 amps "C" curve, SP, MCB in sheet steel enclosure, on surface or in recess, with chained metal cover for the socket outlet and complete with connections, testing and commissioning etc as required. | Each | 2 |  |  |
| 12 | Supply of 1x28W flourescent tube fitting sleek with decorative end cap, powder coated CRCA steel channel complete with all electrical accessories like electronic ballast, lamp holder, 28W T5 lamp etc complete as required. (Philips TMS 540 HF / Wipro WRF 80128 SG / GE DMS 28 N N/Havells) | Each | 2 |  |  |
| 13 | S/F Vertical Type TP MCB DB: |  |  |  |  |
|  | Supplying and fixing following way prewired SP&N MCB, distribution board of steel sheet for 240 volts on surface/recess complete with loose wire box, terminal connectors for all incoming and outgoing circuits, duly prewired with suitable size FR PVC insulated copper conductor up to terminal blocks, tinned copper bus bar, neutral link, earth bar, detachable gland plate, interconnections, powder painted including earthing etc.as required(But withour MCB/RCCB/Isolator) (Make: Legrand) |  |  |  |  |
|  | 12 way SPN, double door | Each | 1.00 |  |  |
| 13.1 | S/F of DP ISOLATOR |  |  |  |  |
|  | Supplying and fixing following rating, double pole, (single phase and neutral), 240 volts, ISOLATOR having a sensitivity current upto 100 to 300 milliamperes in the existing MCB DB complete with connections, testing and commissioning etc, as required.(Make: Legrand) |  |  |  |  |
|  | 16/25 A, DP isolator | Each | 1.00 |  |  |
| 13.2 | S/F of SP MCB |  |  |  |  |
|  | Supplying and fixing 6 amps to 32 amps rating, 240 Volts, "C" series, miniature cirucuit breaker suitable for lighting and other loads of following poles in the existing MCB DB complete with connections, testing and commissioning etc as required.(Make: Legrand/Havells/HPL) |  |  |  |  |
|  | 6/32 A SP MCB | Each | 12 |  |  |
| 14 | Supplying and fixing the following ratings of load change over switch inside a cubile board for changing the load from EB to DG with its all accessories (Make ABB, L&T, Legrand) |  |  |  |  |
|  | 16/25A COS | Each | 1 |  |  |
| 15 | WORK COMPLETION |  |  |  |  |
| 15.1 | Leading, Positioning, Installation, testing and commissioning of 5 kVA DG set with all its accessories | LS | 1 |  |  |
| 15.2 | Obtaining power allocation & power connection etc. for commissioning of the whole installation in the premises. all statutory payments will be reimbursed on production of the original reciepts. | LS | 1 |  |  |
|  | Miscellaneous Items |  |  |  |  |
| 16 | Miscellaneous electrical work, removing of existing conduits and interconnections of light,6A socket/16A socket/A/C point from exisitng DB | LS | 1 |  |  |
|  | TOTAL |  |  |  |  |

SEAL OF THE APPLICANT

SIGNATURE

NAME AND ADDRESS OF APPLICANT