Ref:  Tender No. HLL/CHO/HCS/PROJ/2014-15/006 dated 23-02-20145

The following amendments have been incorporated to the bid document for the above tender - TENDER   DOCUMENT FOR CIVIL, ELECTRICAL & INTERIOR WORKS AT LIFECARE CENTRE, GOVERNMENT MEDICAL COLLEGE, THRISSUR, KERALA.

FOR

- General Terms & Condition Point No: 5

The works shall be completed within 30 days from the date of receipt of work order or LOI.

- Annexure A	Point no. 5

Supplying and providing partition with Aluminium frames, partially glazed partially prelaminate partical board etc with door accessories.

- Annexure A point No.11

Providing and fixing to existing door frames. 24 mm thick factory made PVC door shutters made of styles and rails of a UPVC hollow section of size 59x24 mm and wall thickness 2 mm (± 0.2 mm) with inbuilt edging on both sides. The styles and rails mitred and joint at the corners by means of M.S. galvanised/plastic brackets of size 75x220 mm having wall thickness 1.0 mm and stainless steel screws. The styles of the shutter reinforced by inserting galvanised M.S. tube of size 20x20 mm and 1 mm (± 0.1 mm) wall thickness.

- Annexure A point No 12

Supplying and fixing aluminium windows/ ventilators using 1.2mm thick section including all cost of material, labour etc complete as per the instruction of EIC

MAY BE READ AS

- General Terms & Condition Point No: 5

The works shall be completed within 45 days from the date of receipt of work order or LOI.

- Annexure A	Point no. 5

Supplying and providing partition with Aluminium frames powder coated 20guage (mat finish) frame work size 10x5 cm, partially glazed partially prelaminate partical board etc with door accessories.

- Annexure A point No.11

Providing and fixing door frames. 24 mm thick factory made PVC door shutters made of styles and rails of a UPVC hollow section of size 59x24 mm and wall thickness 2 mm (± 0.2 mm) with inbuilt edging on both sides. The styles and rails mitred and joint at the corners by means of M.S. galvanised/plastic brackets of size 75x220 mm having wall thickness 1.0 mm and stainless steel screws. The styles of the shutter reinforced by inserting galvanised M.S. tube of size 20x20 mm and 1 mm (± 0.1 mm) wall thickness.

- Annexure A point No 12

Supplying and fixing aluminium windows powder coated 20guage (mat finish) frame work size 5x2.50cm/ ventilators using 1.2mm thick section including all cost of material, labour etc complete as per the instruction of EIC

All other terms and conditions remain same as per the original tender no HLL/CHO/HCS/PROJ/2014-15/006 dated 23-02-2015